3GPP TSG-SA5 Meeting #164	S5-255152

Dallas, USA, 17 - 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Potential Requirements and Solution on Time Issue of External Management Data

Document for:	Approval

Agenda item:	6.20.8

Spec:	3GPP TR 28.887

Version:	0.2.0

Work Item:	FS_MADCOL_Ph3

Comments

Reason for change: Potential solutions for Use Case#1: Time Issue of External Management Data is missing, this pCR introduces potential requirements and one potential solution.

Proposed Changes

* * * First Change * * * *

### 4.1.1	Use Case#1: Time Issue of External Management Data

#### 4.1.1.1      Description

External management data are modelled in TS 28.622[X] by ExternalDataType IOC (clause 4.3.73). This IOC defines attributes to indicate the type of external management data and related meta data.

Specified meta data are

- mediaLocation: “address from which the described external management data can be retrieved. “

- externalDataTypeSchema: “URI where the MnS consumer can get the schema to parse the external management data.”

- externalDataScope: “concrete scope (e.g., geographical areas) which the external management data is applicable.”

#### 4.1.1.2      Problem Statement

The attribute mediaLocation of IOC ExternalDataType specifies the endpoint where to retrieve the external management data. However, there is no indication on the validity of this data on a time scale. E.g. for which period of time applies the provided external data is applicable. Is this a record of the past, or a prediction for the future. The information of time is of importance in case of a request for historical data as well as for present data or for future data.

The explicit information of time regarding the requested or reported external management data is missing.

#### 4.1.1.3      Potential Requirements

REQ- 1: 	Management system shall be able to provide information about generation time of external management data.

REQ- 2: 	Management system shall be able to provide information about the time window of input data used to generate external management data.

REQ- 3: 	Management system shall be able to provide information about validity time of external management data.

#### 4.1.1.4      Potential Solution

There are several aspects of time to be considered:

-	generation time: It provides information on the point of time when the external management data has been generated. 
For example, if the considered external management data is a weather forecast. It makes a difference whether the weather forecast is generated for the next day or for one or several days more in the future.

-	time window of input data: It provides information on the time window which has been considered to generate external management data.

-	validity time: It provides information on the point of time or duration of time where the external management data may apply. The validity time is based on the use case. 
Examples:

- 	In case of external management data of type “Event Schedule”, it comprises the time when the described event(s) will take place or took place.

-	In case of external management data of type “Camera Photo” it defines the point of time or time window for which the photo is valid e.g. a photo of a platform may be valid from the point of time taking the photo until the point of time the next photo is taken.

Note in some cases not all aspects need to be considered e.g. in case of describing a camera photo the generation time and the time window of input data may be the same.

The changes that are required to reflect the time aspects of external management data specified by ExternalDataType IOC (clause 4.3.73) in TS 28.622[X] include the following:

- Introducing an attribute indicating the time of generation of the described external management data.

- Introducing an attribute indicating the time window considered to generate external management data.

- Introducing an attribute indicating the validity time of the described external management data.

* * * End of Changes * * * *