3GPP TSG-SA5 Meeting #164	S5-255176

Dallas, USA, 17 - 21 November 2025

# Source:	Huawei

# Title:	Rel-20 pCR 32.872 Introduce the solution#1 on LBO Inter CHFs scenario Key issue #3

# Document for:	Approval

Agenda Item:	7.5.2

1	Decision/action requested

This is a pCR to TR 32.872 Introduce a solution on Local Breakout Inter CHFs scenario key issue3.

2	References

[1]	3GPP TR 32.872: "Study on 5GA roaming charging reliability enhancement".

3	Rationale

This pCR proposes to introduce a new solution on Local Breakout Inter CHFs scenario key issue3.

4	Detailed proposal

Propose to incorporate the following change into the TR 32.872 [1]

First change

5.1.4.d	Solution #3.x: Align the Unit Count Inactivity Timer between two charging sessions

A possible solution for key issue #3 covering requirements REQ-3GPPCH-LBIC-3, handling the association of two charging sessions.

In order to avoid one of the inter-CHF charging sessions is terminated while another one remains active, the unit count inactivity timer can be disabled or aligned between two charging sessions. For the latter, if the V-CHF received a new Unit Count Inactivity Timer from the H-CHF, the V-CHF responds with the Unit Count Inactivity Timer received from the H-CHF for use in the V-SMF(CTF). if the V-CHF doesn’t receive a new Unit Count Inactivity Timer from the H-CHF, the V-CHF decides the Unit Count Inactivity Timer for use in the V-SMF(CTF).

End of change