3GPP TSG-SA5 Meeting #165	S5-260060

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.572 | CR | 0004 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.572 Plan management corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PlanM |  |  |  |  |  | Date: |  |  | 2026-01-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | There are inconstencies in the within the stage-2 and between the stage-2 and stage-3 of the solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections and updating YAML to match existing stage-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies, ambigious definitions, interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 3.3, 5, 6.1.2, 6.1.3, 6.3.1, 6.3.4, 6.4.2, 6.5.1, 6.5.2, 6.5.3, 6.11, 6.13, 6.14, 7.1.2, 7.1.3, 7.1.3.1, 7.2, 7.2.1, 7.2.2, 7.2.3, 7.2.3.1, 7.3, 7.3.1, 7.3.2, 7.4, 7.4.1, 7.4.2, 7.4.3, 7.4.3.1, 7.5, 7.5.1, 7.5.2, 7.5.3.5, 7.5.3.7, 7.6, 7.6.1, 7.6.2, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/2014 at commit b26999e0cbc919fe53d367c016a82318c2472ca4 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] apply.

## 3.2	Symbols

None

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply.

==============Next change==============

# 5	Requirements

Requirements related to planned configurations

Req-PCG-1: The 3GPP management system shall support creating, reading and deleting planned configurations for managed systems.

Req-PCG-2: The 3GPP management system should support updating planned configurations for managed systems.

Req-PCG-3: The 3GPP management system shall support creating, reading, updating, and deleting meta data for planned configurations.

Requirements related to control parameters for validation and activation of planned configurations

Req-PCC-1: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer if validation and activation shall be processed in best effort or atomic mode.

Requirements related to planned configuration groups

Req-PGG-1: The 3GPP management system should allow MnS consumers to group planned configurations into planned configuration groups.

Req-PGG-2: The 3GPP management system should allow MnS consumers to group planned configuration groups into planned configuration groups.

Req-PGG-3: The 3GPP management system shall support creating, reading and deleting planned configuration groups.

Req-PGG-4: The 3GPP management system should support updating planned configuration groups.

Req-PGG-5: The 3GPP management system shall support creating, reading, updating, and deleting meta data for planned configuration groups.

Requirements related to control parameters for validation and activation of planned configuration groups

Req-PGC-1: The 3GPP management system shall support creating, reading, updating, and deleting control parameters for planned configuration groups.

General plan validation requirements

Req-PVG-1: The 3GPP management system shall support a capability allowing a MnS consumer to request a MnS producer to validate a planned configuration.

Req-PVG-2: The 3GPP management system should support a capability allowing a MnS consumer to retrieve the status of a plan validation.

Req-PVG-3: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to cancel an ongoing plan validation.

Req-PVG-4: The 3GPP management system shall support a capability allowing a MnS consumer to retrieve the result of a plan validation.

Req-PVG-5: The 3GPP management system should allow for multiple sequential validation of the same plan.

Req-PAG-6: The 3GPP management system shall support a capability allowing a MnS consumer to find out if a plan has been previously validated.

Requirements related to the validation of multiple plans

Req-PVM-1: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to validate multiple planned configurations with a single request.

Req-PVM-2: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the type of relationship between the plans, for the case where multiple plans are requested to be validated with a single request.

General plan activation requirements

Req-PAG-1: The 3GPP management system shall support a capability allowing a MnS consumer to request a MnS producer to activate a planned configuration.

Req-PAG-2: The 3GPP management system should support a capability allowing a MnS consumer to retrieve the status of a plan activation.

Req-PAG-3: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to cancel an ongoing plan activation.

Req-PAG-4: The 3GPP management system shall support a capability allowing a MnS consumer to retrieve the result of a plan activation.

Req-PAG-5: The 3GPP management system should allow for multiple sequential activations of the same plan.

Req-PAG-6: The 3GPP management system shall support a capability allowing a MnS consumer to find out if a plan has been previously activated.

Req-PAG-7: The 3GPP management system shall validate a planned configuration automatically when activation of the planned configuration is requested without prior validation.

Requirements related to control parameters for activation

Req-PAC-1: The 3GPP management system should support a capability allowing a MnS consumer to indicate to a MnS producer if activation should be done with the least possible service impact or within the shortest possible time.

Req-PAC-2: The 3GPP management system should support a capability allowing a MnS consumer to indicate to a MnS producer if fallback shall be enabled.

Requirements related to the activation of multiple plans

Req-PAM-1: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to activate multiple planned configurations with a single request.

Req-PAM-2: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the type of relationship between the plans, for the case where multiple plans are requested to be activated with a single request.

Req-PAM-3: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the behavior if plan conflicts are detected, for the case where multiple plans are requested to be activated with a single request.

Requirements related to conditional activation of plans

Req-CAP-1: The 3GPP management system should allow MnS consumers to create conditions on MnS producers that specify when planned configurations are activated.

==============Next change==============

### 6.1.2	Format of planned configurations

A planned configuration is represented by a set of operations to be applied to the current configuration. Each operation is described by three parameters: a modify operator, a target, and a value. An operation may be annotated with a description. The order of operations in the operation set is irrelevant.

The target parameter identifies a data node in the current configuration that is to be manipulated by the modify operator. This data node is referred to as target data node. The modify operarator specifies the modification to be applied to the target data node. The value parameter specifies the value that is used by the modify operator on the target data node.

The modify operator indicates, if a new data node is to be created in the current configuration, an existing data node is to be updated, or an existing data node is to be deleted. The value parameter is absent for data node deletions, in all other cases it needs to be present.

This specification defines a general format for planned configurations for which the following applies.

-	The data node identified by the target parameter may be any kind of data node (managed objects, attributes, attribute fields, attribute elements).

-	The following modify operations are defined:

-	create: creates the target data node in the current configuration with the representation specified by the value parameter, if the target data node does not exist in the current configuration. If the target data node already exists, an error is raised.

-	merge: merges the (partial) representation of the target data node specified in the value parameter into the current representation of the target data node, if the target data node exists in the current configuration. If the target data node does not exist, an error is raised. Deletion of data nodes is possible.

-	merge-create: merges the (partial) representation of the target data node specified in the value parameter into the current representation of the target data node, if the target data node exists in the current configuration. If the target data node does not exist, it is created with its representation set to the representation specified in the value parameter. Deletion of data nodes is possible.

-	delete: deletes the target data node (and all its contained data nodes), if the target data node exists in the current configuration. If the target data node does not exist, the MnS producer shall ignore the operation and shall not raise an error.

-	In a valid planned configuration, the managed objects containg the target managed object shall exist either in the current configuration or as part of the planned configuration. The MnS producer is not required to implicitly create managed objects instances that are used as path components but which do not exist in the plan nor in the current configuration.

For the use with current configurations that are based on the managed object concept a special profile called Managed Object Plan is provided. For a Managed Object Plan the following applies:

-	The data node identified by the target parameter shall be a managed object instance. No other data node types are allowed as target data nodes. The MnS producer shall reject a planned configuration if it contains target data data nodes that are not managed objects.

-	All updates for an existing managed object should be grouped into a single "merge" or "merge-create" operation. The MnS producr may reject planned configurations where more than one operation targets the same managed object instance.

The producer shall indicate whether general plans or only Managed Object Plans are supported.

The MnS consumer may specify an identifier for each operation in the creation request for a planned configuration descriptor. The MnS producer may discard this identifier upon reception of the request and use the positional index of the operation in the operation set instead whenever a reference to an operation in the operation set is required. The positional indx of the first operation in the operation set is "0".

### 6.1.3	Meta data for planned configurations

Each planned configuration is described by meta data. This includes the name, the version, and a human readable textual description. It is also possible to specify additional MnS consumer defined properties (key value pairs) to further describe and qualify the planned configuration, for example to specify who created the planned configuration. These annotations are for usage by a (human) MnS consumer only. The are not processed by the MnS producer.

Furthermore, it includes the content type of the planned configuration and

it is specified if the operations of the operations set are an atomic set, where all operations need to be activated successfully, or if some operations are allowed to fail (non-atomic or best effort set). In other words, for atomic sets, all operations of the set must be processed successfully or no operation at all. Operations, that are already applied when an error occurs, must be rolled back. For best effort sets, the operations that can be processed successfully are processed, and those that for whatever reason cannot be processed successfully are not processed. In this mode the activation process continues on the occurrence of an error. In a third mode the activation process stops on the occurrence of an error.

For example, subscriptions to alarm notifications on multiple Network Functions may have value also in case the subscription on some Network Functions fails. When analytics are computed based on measurements collected at some well selected Network Functions, then all measurement collection jobs must be created. If one job cannot be created it does not make sense to collect the remaining measurements.

The date and time at which information in the planned configuration was modified the last time is provided as well.

==============Next change==============

### 6.3.1	Definition

Fallback is the process of returning to the configuration that was used by the managed system before the activation of a planned configuration or planned configuration group. A fallback configuration is the configuration describing this configuration.

Fallback configurations are generated by the system. Fallback may be applied when the activation of a planned configuration or a planned configuration group does not yield the desired results. Use cases include:

- A wrong planned configuration was activated. The MnS consumer requests to return to the old configuration.

- The new configuration does not perform as expected. The MnS consumer requests to return to the old configuration.

- The new configuration was deployed on existing Network Functions in anticipation of some new Network Functions being rolled out. The roll out does not happen. The existing Network Functions are rolled back to the configuration state at the time the activation was triggered.

There are two types of fallback configurations:

-	"undo": This type only reverses the changes resulting from the activation of a planned configuration or planned configuration group. It has no effect on configuration data nodes that are not affected by the activation of the planned configuration or planned configuration group. Therefore, changes to other data nodes, that occur after activation and before fallback, are not undone. The format of this type of fallback configuration is similar to a planned configuration.

-	"complete restore": This type reverts all configuration changes done after the activation of a planned configuration or planned configuration group, i.e. those resulting from the activation of a planned configuration or planned configuration group and those done via some other means. This method restores the complete old configuration. Therefore, changes to data nodes, that occur after activation and before fallback, are undone as well. Note that the complete data node tree is restored in this mode. Therefore, this mode might not be supported in deployment scenarios with large data node trees. The format of this type of fallback configuration is implementation specific.

==============Next change==============

### 6.3.4	Fallback configuration descriptor

The fallback configuration and its meta data are specified in a fallback configuration descriptor. Each descriptor has a unique identifier.

A fallback configuration has the following properties:

-	activation mode is always atomic (plan descriptor)

-	must include the id of the activation job

-	"configChangesContentType" is set by the MnS producer and is read-only (plan descriptor)

-	cannot be a member of a planned configuration group (group descriptor)

-	cannot be activated by trigger conditions (trigger conditions)

-	fallback configurations (of the "complete restore" type) are not allowed to be validated (as they are always valid)

-	no fallback is possible (activation)

-	no cancelling is possible (activation)

-	a pointer to the fallback configuration descriptor shall be included in the related activation job (activation)

==============Next change==============

### 6.4.2	Usage

Trigger conditions are provided for conditional activation of planned configurations and planned configuration groups. The planned configuration (group) is activated automatically when the provided conditions are met.

For example, additional performance metric collection jobs or assurance functions shall be created when metrics that are always collected cross certain thresholds.

==============Next change==============

### 6.5.1	Definition

Planned configurations may have problems making it impossible to activate them. Validation is the process to discover potential problems with planned configurations before their activation.

The validation function has two input arguments: the planned configuration and a reference configuration which typically is a snapshot of the current configuration at a certain point of time. The planned configuration is applied to reference configuration. The resulting configuration data node tree must comply with the (stage 2) NRM definitions and (stage 3) NRM schema definitions.

### 6.5.2	Potential problems

Validation discovers problems of planned configurations. These problems may be classified as follows:

-	Information model problems

-	Problems of the planned configuration itself

-	Problems of the planned configuration in relationship to the current configuration

-	Application layer problems

Examples of problems with the planned configuration itself:

-	Update of an attribute with a value that does not match the data type defined for the value of the attribute

-	Creation of an attribute name value pair that is not defined

-	Update of an existing attribute that is defined as read-only

-	Creation of an object with an object class that is not defined

-	Creation of an object with a representation that does not match the object class defined for the object

-	Creation of an object under an object whose class does not support this containment

Examples of problems of the planned configuration in relationship to the current configuration:

-	Creation of objects under parents that do not exist

-

-	Violation of the cardinality property of an object

-	Violation of the multiplicity property of an attribute

Examples of application layer problems

-	An attribute value in the planned configuration is in the value range allowed by the attribute properties but the value is not allowed in the current state of the application.

### 6.5.3	Validation process

The purpose of validation is to detect problems of the planned configuration. As described in the previous clause, problems can be categorized into three groups.

Problems of the operations themselves are detected by analyzing the operations together with the NRM schema. The current configuration is not involved in this process. Problematic operations are always invalid. If an atomic operation set contains at least one invalid operation, the complete planned configuration is invalid. If a non-atomic operation set contains a bad operation, only that operation is invalid. The operation set with the other operations may still be valid, depending on the validation result for the other operations.

Problems of the planned configuration in relationship to the current configuration are detected by applying the operations of the operation set to a snapshot of the current configuration. This may result in intermediate states that do not comply to the constraints defined by the NRM schema. For example, assume the schema defines a constraint that an object has to contain exactly a certain number of child objects. When a plan contains an operation to remove one child object, and an operation to add a child object, there might be intermediate states at certain times during the execution of the validation with the constraint violated, as there can be too few or too many child objects. However, after validating all operations, the constraint is fulfilled again. The validation process needs to take this into account: Problems of the planned configuration in relationship to the current configuration may be temporary only and be healed later when more operations are processed. A final assessment on the validity of operations can be made only after processing all operations.

Problems of the planned configuration at application layer are detected by evaluating the updates specified by the operations in the context of the configuration that is produced by applying the planned configuration against the current configuration. As for problems of the planned configuration in relationship to the current configuration, a final assessment on the validity of operations can be made only after processing all operations.

All constraint violations shall be reported.

The result of a validation is valid only for the given reference configuration, i.e. in many use cases the conceptual snapshot of the current configuration at a certain point in time. The same validation could have a different result later for many reasons e.g. current configuration changes or network environment changes.

A successful validation does not guarantee a successful activation.

==============Next change==============

## 6.11	Managing fallback configurations

Fallback configuration descriptors are not directly managed by MnS consumers. They are created automatically by MnS producers when the "enableFallback" attribute in an activation job is set to true. Only the "name", "version", and "description" can be set by MnS consumers once the object is created. Fallback configurations cannot be modified by MnS consumers. Each fallback configuration is identified by an identifier. The identifier is assigned by the MnS producer.

When many fallbacks are enabled, the situation can become soon complex and the risk for misconfigurations increases. MnS producers may therefore apply implementation specific strategies to reduce the risk for misconfigurations. For example, they may allow only for one fallback configuration to exist; if another fallback is enabled, the existing fallback configuration is overwritten. Or they may allow for multiple fallback configurations only if they affect different parts of the network.

Fallback configurations can be deleted by MnS consumers, or by the MnS producer based on implementation specific policies.

No fallback on the activation of a fallback configuration is possible.

==============Next change==============

## 6.13	Managing validations

Validation of a planned configuration is requested by MnS consumers and performed by MnS producers. To request validation the MnS consumer needs to create a validation job on the MnS producer. Each job is identified by an identifier. The identifier is assigned by the MnS producer.

The MnS consumer may annotate the job with a "name", a textual human-readable "description" and his own identifier "mnsConsumerId".

The planned configuration to be validated is specified by including the identifier of the corresponding planned configuration descriptor in the job creation request. Alternatively, for validating a planned configuration group, a planned configuration group descriptor can be specified.

In case no planned configuration descriptor or no planned configuration group descriptor has been created for the planned configurations to be validated, a descriptor may be specified directly in the validation job creation request.

The validation mode meta data specifies whether the validation stops on the first error found or runs till the full planned configuration(group) is validated, potentially listing multiple errors.

The MnS producer adds the following information elements to the job: "jobState", "jobDetails", "currentConfigTime", "startedAt", "stoppedAt", "validationState", and "validationDetails".

The information element "error" is a structured data type with the following properties:

-	The "type" property that provides high level error information.

-	The "title" that provides a short, human-readable summary of the problem type. It shall not change from occurrence to occurrence of the problem.

-	The "reason" property" that provides more details on the error conditions than the "type" property.

-	The "detail" property that provides a human readable explanation specific to this occurrence of the problem.

-	The "errorInfo" property that provides any additional error information.

-	The "badDataNode" property that specifies the data node to which the operation could not be applied.

The "type" property is an enumeration of string values. A MnS producer should use the following values. Other values may be used as well if deemed more appropriate for specific errors.

-	SCHEMA_VALIDATION_ERROR: Content in the operation request does not validate or the data node tree of the current configuration after applying the operation becomes invalid. Therefore, the operation cannot be processed.

-	DATA_NODE_TREE_ERROR: The assumptions on the state of the data node tree of the current configuration as expressed in "path" do not match the real state (for example, a data node referred to in "path" that must exist does not exist). Therefore, the operation cannot be processed. The operation request is valid, though.

-	MODIFICATION_NOT_ALLOWED: The operation is well formed and understood by the MnS producer but not allowed.

-	ACCESS_CONTROL_CONFLICT: The MnS consumer is not allowed to perform the requested operation, because it does not have the required access rights.

-	APPLICATION_LAYER_ERROR: The operation is well formed and understood by the MnS producer, but the MnS producer cannot satisfy the request due to application layer issues.

-	SERVER_ERROR: The operation is well formed and understood by the MnS producer, but the MnS producer cannot process the operation due to an error in the server.

For each "type" value several reasons are provided:

SCHEMA_VALIDATION_ERROR

-	NEW_DATA_NODE_NAME_INVALID: The data node cannot be created as requested, because its name, as specified as last segment of "path", is invalid at the target location.

-	NEW_DATA_NODE_VALUE_INVALID: The data node specified in the "path" property cannot be created or cannot be updated as requested, because the value specified in "value" is invalid.

-	NEW_DATA_NODE_CONTAINMENT_INVALID: The data node cannot be created under the specified parent data node as requested, because this containment is not allowed.

-	FINAL_DATA_NODE_VALUE_INVALID: The current value of the target data node cannot be modified as requested, because this would result in an invalid value of the target data node. The value in “value” itself is valid.

-	FINAL_DATA_NODE_UNIQUENESS_INVALID: The data node cannot be updated or created as requested, because this would result in violating uniqueness constraints. The value in “value” itself is valid. This error reason applies to multi-valued data nods only.

-	FINAL_DATA_NODE_MULTIPLICITY_INVALID: The data node cannot be created or deleted as requested, because this would result in violating multiplicity constraints. The value in “value” itself is valid. This error reason applies to multi-valued data nods only.

-	FINAL_DATA_NODE_CARDINALITY_INVALID: The data node cannot be created or deleted as requested, because this would result in violating cardinality constraints. The value in “value” itself is valid.

DATA_NODE_TREE_ERROR

-	TARGET_DATA_NODE_NOT_FOUND: The target data node specified by “path” does not exist in the current configuration. Therefore, the “merge” operation cannot be applied as requested.

-	TARGET_DATA_NODE_PARENT_NOT_FOUND: The parent of the target data node specified by “path” does not exist in the current configuration. Therefore, the target data node cannot be created as requested by the "create" operation.

-	TARGET_DATA_NODE_FOUND: The target data node specified by “path” exists in the current configuration. Therefore, the target data node cannot be created as requested by the "create" operation. The value of the target data node in the planned configuration and the value of that node in the current configuration may or may not be identical.

MODIFICATION_NOT_ALLOWED

-	TARGET_DATA_NODE_NOT_WRITABLE: The data node cannot be modified as requested, because the data node is not writable by MnS consumers.

-	TARGET_DATA_NODE_INVARIANT: The data node cannot be modified as requested, because the data node is invariant.

-	TARGET_DATA_NODE_CREATION_NOT_ALLOWED: The data node cannot be created by MnS consumers.

-	TARGET_DATA_NODE_DELETION_NOT_ALLOWED: The data node cannot be deleted by MnS consumers.

ACCESS_CONTROL_CONFLICT

-	ACCESS_DENIED: Access to the target data node is denied as a result of access control.

APPLICATION_LAYER_ERROR

SERVER_ERROR

Multiple validation jobs can run in parallel, even if the planned configurations target the same portions of the current configuration.

To provide a consistent result, a planned configuration or planned configuration group (incl. its members) cannot be modified during validation, or more precisely when the "jobState" of the validation job has either of the following values: "RUNNING", "CANCELLING". Modification requests shall be rejected.

## 6.14	Managing activations

Activation of a planned configuration is requested by MnS consumers and performed by MnS producers. To request activation the MnS consumer needs to create an activation job on the MnS producer. Each job is identified by an identifier. The identifier is assigned by the MnS producer.

The planned configuration to be activated is specified by including the identifier of the corresponding planned configuration descriptor in the job creation request. Alternatively, for activating a planned configuration group, a planned configuration group descriptor can be specified.

In case no planned configuration descriptor or no planned configuration group descriptor has been created for the planned configurations to be activated, a descriptor may be specified also directly in the activation request.

For falling back to an old configuration, the identifier of a fallback configuration descriptor needs to be specified.

Activation shall start immediately upon creation of the activation job, unless the "isImmediateActivation" parameter is set to false. In this case the activation is triggered by a trigger condition that points to the activation job. Note that the activation job does not know the related conditions.

For immediate activation an activation job can activate a planned configuration only once. If the planned configuration is to be activated a second time a new job needs to be created. For conditional activation, in contrast, an activation job can activate the same planned configuration multiple times. For example, if a planned configuration is for weekdays and another planned configuration is for weekends, two activation jobs can be used for activating both planned configurations. One job is triggered by a trigger condition triggering the activation when the weekend starts, the other job is triggered by a trigger condition when the weekdays start.

Furthermore, the following control parameters need to be specified in the job creation request: "isEnableFallback", and "serviceImpact", and "isImmediateActivation".

The MnS producer adds the following information elements to the job: "state", "startedAt", "stoppedAt", "result", and "errors".

Only planned configurations, that are successfully validated, should be activated. Therefor a MnS consumer should always create a validation job first. Only after successful validation of a planned configuration, a job for activating the planned configuration should be created. In case the planned configuration was not validated the MnS producer shall create a validation job for the planned configuration. The activation is started only if the planned configuration is valid. If the planned configuration is invalid the activation is not started and the result attribute of the activation indicates that the activation failed.

A MnS consumer can request a MnS producer to cancel a running activation process. The behavior depends on the application mode:

- For "ATOMIC" the configuration is rolled back.

- For "BEST_EFFORT" the activation is stopped.

- For "STOP_ON_ERROR" the activation is stopped

As for validation, a planned configuration or planned configuration group (incl. its members) cannot be modified or deleted during its activation, or more precisely when the "jobState" of the activation job has either of the following values: "RUNNING", or "CANCELLING".

Multiple activation jobs can run simultaneously if the planned configurations do not affect the same portions of the current configuration. However, if they do affect the same portion, they cannot run in parallel and shall be queued.

If a new configuration is provided together with the activation request, the MnS producer shall create automatically a planned configuration descriptor or planned configuration group descriptor and replace the inline specification of the planned configuration or planned configuration group with a reference to the newly created descriptor.

==============Next change==============

### 7.1.2	Information Elements

The following table specifies the information elements of a planned configuration descriptor.

# Table 7.1.2-1: Information elements of the "PlannedConfigurationDescriptor"

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the planned configuration. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the planned configuration. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| version | M | The version of the planned configuration. Its format is implementation specific. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description | M | The textual human-readable description of the planned configuration. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| customProperties | M | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |
| configChangesContentType | M | The format of the planned configuration in "planConfig". The format depends on the solution set of the current configuration. allowedValues: - OPENAPI_BASED - YANG_BASED | type: ENUM multiplicity: 1 isInvariant: True isWritable: True |
| configChanges | M | The operation set specifying the planned configurationThe format of "configChanges" is specified in "configChangesContentType" | type: ConfigChange multiplicity: 1..* isInvariant: False isWritable: True |
| activationMode | M | Specifies if the operations in the operation set are treated for activation as atomic, best effort, or if processing shall stop on the occurrence of the first error. "ATOMIC": Either all or none of the operations are executed. In case of error the already executed operations should be rolled back. However, there is no guarantee that the rollback will succeed. "BEST_EFFORT": In case of an error all further operations that are possible to execute are attempted to be executed. "STOP_ON_ERROR": In case of an error no further operations are attempted. allowedValues: - ATOMIC - BEST_EFFORT - STOP_ON_ERROR | type: ENUM multiplicity: 1 isInvariant: False isWritable: True |
| lastModifiedAt | M | The date and time at which the planned configuration descriptor was modified the last time by a MnS consumer. Upon creation of the planned configuration descriptor the value of the information element is set to the date and time at which the descriptor is created. | type: DateTime multiplicity: 1 isInvariant: False isWritable: False |
| lastValidatedAt | M | The date and time at which the planned configuration was validated (the last time). The information element is absent, when the planned configuration has not been validated yet. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| validationState | M | The outcome of the last validation of the planned configuration ("VALID", "INVALID"), or "NOT_VALIDATED" if the planned configuration has not been validated yet. The state is reset to "NOT_VALIDATED", if in another state, when "configChanges" is updated. allowedValues: - NOT_VALIDATED - VALID - INVALID | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |

### 7.1.3	Data types

#### 7.1.3.1	ConfigChange

# Table 7.1.3.1-1: Information elements of the "ConfigChange"

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| target | M | The target node of the operation. | type: String multiplicity: 1 isInvariant: False isWritable: False |
| modifyOperator | M | The modify operator applied to the target node. allowedValues: - create - merge - merge-create - delete | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| value | M | The value applied to the target node, which follows the NRM schema of the target. The "value" shall be present unless the "modifyOperator" is "DELETE". | type: Object multiplicity: 0..1 isInvariant: False isWritable: False |
| description | O | The textual human-readable description of the operation. | type: String multiplicity: 0..1 isInvariant: False isWritable: False |
| changeId | O | The identifier of the operation. It may or may not be provided If provided, it shall be unique within an instance of "configChanges" and it shall be provided for all changes in "configChanges". | type: String multiplicity: 0..1 isInvariant: False isWritable: False |

## 7.2	PlannedConfigurationGroupDescriptor

### 7.2.1	Definition

This definition represents a planned configuration group descriptor.

### 7.2.2	Information Elements

The following table specifies the information elements of a planned configuration group descriptor.

# Table 7.2.2-1: Information elements of the "PlannedConfigurationGroupDescriptor"

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the planned configuration group descriptor | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the planned configuration group. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| version | M | The version of the planned configuration group. Its format is implementation specific. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description | M | The textual human-readable description of the planned configuration group. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| customProperties | O | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. | type: Object multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |
| members | M | The identifiers of planned configurations or the planned configuration groups that are members of this planned configuration group. | type: Member multiplicity: 1..* isOrdered: True isUnique: True isInvariant: False isWritable: True |
| isOrdered | M | Specifies if the members of the planned configuration group are ordered. When ordered, the planned configuration group members shall be validated/activated in the specified order. When not ordered the planned configuration group members can be validated/activated in any order. | type: Boolean multiplicity: 1 isInvariant: False isWritable: True defaultValue: true |
| applyMode | M | Specifies if the members of the planned configuration group are treated as atomic, best effort, or if processing shall stop on the occurrence of the first error. allowedValues: - ATOMIC - BEST_EFFORT - STOP_ON_ERROR | type: ENUM multiplicity: 1 isInvariant: False isWritable: True defaultValue: BEST_EFFORT |
| isFailOnMemberConflicts | M | Specifies if the activation shall fail on detection of conflicts between planned configuration group members, or if the operations shall be processed as if there were no conflicts. | type: Boolean multiplicity: 1 isInvariant: False isWritable: True defaultValue: true |
| lastModifiedAt | M | The date and time at which the planned configuration group descriptor was modified the last time by a MnS consumer. Upon creation of the planned configuration group descriptor the value of the information element is set to the date and time at which the descriptor is created. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| lastValidatedAt | M | The date and time at which the planned configuration group was validated (the last time). The information element is absent, when the planned configuration has not been validated yet. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| validationState | M | The outcome of the last validation, or "NOT_VALIDATED" if the planned configuration group has not been validated yet. The state is reset to "NOT_VALIDATED", if in another state, when "members" is updated. allowedValues: - NOT_VALIDATED - VALID - PARTIALLY_VALID - INVALID | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |

### 7.2.3	Data types

#### 7.2.3.1	Member

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| planConfigDescrId | M | The identifiers of planned configuration descriptors. | type: String multiplicity: 1..* isInvariant: False isWritable: False |
| planConfigGroupDescrId | M | The identifiers of planned configuration descriptors. | type: String multiplicity: 1..* isInvariant: False isWritable: False |

## 7.3	FallbackConfigurationDescriptor

### 7.3.1	Definition

This definition represents a fallback configuration descriptor.

### 7.3.2	Information Elements

The following table specifies the information elements of a fallback configuration descriptor.

# Table 7.3.2-1: Information elements of the "FallbackConfigurationDescriptor"

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the fallback configuration descriptor | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the fallback configuration. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| version | M | The version of the fallback configuration. Its format is implementation specific. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description | M | The textual human-readable description of the fallback configuration. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| customProperties | M | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. | type: Object multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |
| activationJob | M | The identifier of the related activation job. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| configurationContentType | M | The format of the fallback configuration. For fallbacks of the "undo" type the same values shall be used as for PlannedConfigurationDescriptor.configChangesContentType. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| fallbackConfig | M | The fallback configuration in case the fallback configuration is of the "complete restore" type. Exactly one of this element or the configChanges shall be present. | type: Any multiplicity: 0..1 isInvariant: True isWritable: False |
| configChanges | M | The operation set specifying the configuration in case the fallback confiiguration is of the "undo" type. The format of "configChanges" is specified in "configurationContentType". Exactly one of this element or the fallbackConfig shall be present. | type: ConfigChange multiplicity: 0..* isInvariant: False isWritable: True |

## 7.4	TriggerConditionDescriptor

### 7.4.1	Definition

This definition represents a trigger condition descriptor.

### 7.4.2	Information Elements

The following table specifies the information elements of a trigger condition descriptor.

# Table 7.4.2-1: Information elements of the "TriggerConditionDescriptor"

| Information element name |  | S | Documentation and Allowed Values |  |  | Properties |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| id |  | M | The identifier of the trigger condition descriptor. |  |  | type: String multiplicity: 1 isInvariant: True isWritable: False |  |  |
| name | M |  |  | The name of the trigger condition. |  |  | type: String multiplicity: 0..1 isInvariant: False isWritable: True |  |
| version | M |  |  | The version of the planned configuration. Its format is implementation specific. |  |  | type: String multiplicity: 0..1 isInvariant: False isWritable: True |  |
| description |  | M | The textual human-readable description of the trigger condition. |  |  | type: String multiplicity: 0..1 isInvariant: False isWritable: True |  |  |
| customProperties |  | M | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. |  |  | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |  |  |
| conditionExpression |  | M | The condition expression. |  |  | type: String multiplicity: 1 isInvariant: False isWritable: True |  |  |
| evaluationPeriod |  | M | The evaluation period specifies the interval of time between two consecutive condition expression evaluations. The unit is seconds. |  |  | type: Integer multiplicity: 1 isInvariant: False isWritable: True |  |  |
| hysteresis |  | M | The hysteresis, when present, specifies that the trigger shall not be activated immediately, when the evaluation result changes from false to true, but only when the evaluation result is true for a specified number of times. allowedValues: Integers greater or equal to 1. |  |  | type: Integer multiplicity: 0..1 isInvariant: False isWritable: True |  |  |
| isTriggerOnce |  | M | The boolean indication, if the trigger is disarmed after the first firing. |  |  | type: Boolean multiplicity: 1 isInvariant: False isWritable: True defaultValue: true |  |  |
| activationJobs |  | M | The identifiers of one or more activation jobs that shall be triggered by this condition. |  |  | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |  |  |
| startEvaluationAt |  | M | The date and time at which the evaluation of the condition expression shall start. The evaluation result is set to "False" before that date and time. If the information element is not specified, evaluation of the trigger condition shall start immediately. |  |  | type: DateTime multiplicity: 0..1 isInvariant: True isWritable: True |  |  |
| stopEvaluationAt |  | M | The date and time at which the evaluation of the condition expression shall stop. The evaluation result is set to "False" after that date and time. If the information element is not specified, evaluation of the trigger condition shall continue until the deletion of the trigger condition descriptor. |  |  | type: DateTime multiplicity: 0..1 isInvariant: True isWritable: True |  |  |
| lastModifiedAt |  | M |  |  | The date and time at which the trigger condition was modified the last time by a MnS consumer. Upon creation of the trigger condition descriptor the value of the information element is set to the date and time at which the descriptor is created. |  |  | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| currentEvaluationResult |  | M | The current result of evaluating the "condition-expression". |  |  | type: Boolean multiplicity: 1 isInvariant: False isWritable: False |  |  |
| lastTriggeredAt |  | M | Th date and time at which the evaluation result of the trigger condition changed the last time from "False" to "True". The information element is absent or contains no information if the evaluation result never changed from "False" to "True". |  |  | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |  |  |
| isTriggerActive |  | M | The indication if the trigger can start activation jobs (trigger is active), or if the trigger cannot start activation jobs (trigger is inactive). |  |  | type: Boolean multiplicity: 1 isInvariant: False isWritable: False |  |  |

### 7.4.3	Data types

#### 7.4.3.1	Hysteresis

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| timeOfTrueEvaluations | M | The hysteresis, when present, specifies that the trigger shall not be activated immediately, when the evaluation result changes from false to true, but only when the evaluation results is true for a specified time (which must be a multiple of the evaluation period). Unit is seconds | type: Integer multiplicity: 0..1 isInvariant: False isWritable: True |
| numberOfTrueEvaluations | M | This information element, when present, specifies that the trigger shall not be activated immediately, when the evaluation result changes from false to true, but only when the evaluation results is true for a specified time (which must be a multiple of the evaluation period) or a specified number of times. | type: Integer multiplicity: 0..1 isInvariant: False isWritable: True |

## 7.5	ValidationJob

### 7.5.1	Definition

This definition represents a validation job.

### 7.5.2	Information Elements

The following table specifies the information elements of a validation job.

# Table 7.5.2-1: Information elements of the "ValidationJob"

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the validation job. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the validation job. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description | M | The textual human-readable description of the validation job. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| mnsConsumerId | M | The consumer that created the job. It may indicate a human user and/or one or more applications, for example ["userid:janedoe", "appid:12314"]. | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |
| planConfigDescrId | M | The identifier of the planned configuration descriptor, whose operation set is requested to be validated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigGroupDescrId | M | or, alternatively, the identifier of the planned configuration group descriptor, whose operation sets are requested to be validated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigDescr | O | or, alternatively, the planned configuration descriptor to be validated, if no planned configuration descriptor was created earlier. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigDescrGroup | O | or, alternatively, the planned configuration group descriptor to be validated, if no planned configuration group descriptor was created earlier. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| fallbackConfigDescrId | O | or, alternatively, the identifier of a fallback configuration. Only fallback configurations of the "undo" type can be validated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| validationMode |  | The validation mode. allowedValues: - CONTINUE_ON_ERROR - STOP_ON_ERROR | type: ENUM multiplicity: 1 isInvariant: True isWritable: True defaultValue: CONTINUE_ON_ERROR |
| cancelRequest | O | This boolean information element allows to request to cancel the activation process by setting its value to "True". Setting the value to "False" has no observable result. When the value is set to "True" it cannot be changed any more. | type: Boolean multiplicity: 1 isInvariant: False isWritable: True |
| jobState | M | The validation job state. allowedValues: - NOT_STARTED - RUNNING - CANCELLING - CANCELLED - COMPLETED - FAILED | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| jobDetails | M | Detailed information related to the job, including job related errors. | type: String multiplicity: * isInvariant: False isWritable: False |
| currentConfigTime | M | The date and time of the current configuration state against which the planned configuration or planned configuration group is validated. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| startedAt | M | The date and time at which the validation process started, i.e. the time when the job state transition from "NOT_STARTED" to "RUNNING" occurred. In the "NOT_STARTED" state the information element is absent or carries no information. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| stoppedAt | M | The date and time at which the validation process stopped, i.e. the job state transition from "RUNNING" to "COMPLETED" or "FAILED", or from "CANCELLING" to "CANCELLED". In the "NOT_STARTED", "RUNNING" or "CANCELLING" state the information element is absent or carries no information. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| validationState | M | The current validation state of the planned configuration or planned configuration group that is processed by the validation job. - UNKNOWN: validation result is unknown because the validation has not started yet or is still ongoing. The "jobState" provides further qualifications. - VALIDATION_SUCCEEDED: all operations are validated and are valid. VALIDATION_FAILED: at least one operation was not validated or is invalid. allowedValues: - UNKNOWN - VALIDATION_SUCCEEDED - VALIDATION_FAILED | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| validationDetails | M/ | Details of the validation of the operations that are contained in the planned configuration or planned configuration group. | type: ExecutionDetails multiplicity: 1 isInvariant: False isWritable: False |

==============Next change==============

#### 7.5.3.5	Error

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| type | M | High level error information. allowedValues: - SCHEMA_VALIDATION_ERROR - DATA_NODE_TREE_ERROR - MODIFICATION_NOT_ALLOWED - ACCESS_CONTROL_CONFLICT - APPLICATION_LAYER_ERROR - SERVER_ERROR - OTHER | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| title | M | A short, human-readable summary of the problem type. It shall not change from occurrence to occurrence of the problem. In other words, each type is mapped to one and only one title. | type: String multiplicity: 0..1 isInvariant: False isWritable: False |
| reason | M | Further qualification of the "type". allowedValues: - NEW_DATA_NODE_NAME_INVALID - NEW_DATA_NODE_VALUE_INVALID - NEW_DATA_NODE_CONTAINMENT_INVALID - FINAL_DATA_NODE_VALUE_INVALID - FINAL_DATA_NODE_UNIQUENESS_INVALID - FINAL_DATA_NODE_MULTIPLICITY_INVALID - FINAL_DATA_NODE_CARDINALITY_INVALID - TARGET_DATA_NODE_NOT_FOUND - TARGET_DATA_NODE_PARENT_NOT_FOUND - TARGET_DATA_NODE_FOUND - TARGET_DATA_NODE_NOT_WRITABLE - TARGET_DATA_NODE_INVARIANT - TARGET_DATA_NODE_CREATION_NOT_ALLOWED - TARGET_DATA_NODE_DELETION_NOT_ALLOWED - ACCESS_DENIED - OTHER | type: ENUM multiplicity: 0..1 isInvariant: False isWritable: False |
| detail | M | A human-readable explanation specific to this occurrence of the problem. | type: String multiplicity: 0..1 isInvariant: False isWritable: False |
| errorInfo | M | Any additional error information. | type: Any multiplicity: 0..1 isInvariant: False isWritable: False |
| badDataNode | M | The path identifying the data node to which the operation could not be applied. The value of "path" may be identical to the value of the "path" property in the request or may contain additional components identifying data nodes in the value of the "value" property of the request. | type: String multiplicity: 1 isInvariant: False isWritable: False |

==============Next change==============

#### 7.5.3.7	MemberOp

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| planConfigDescrId | M | The identifier of a planned configuration descriptor that contains on operation involved in a conflict. | type: String multiplicity: 1 isInvariant: False isWritable: False |
| changeId | M | The identification of the operation. It is the identifier ("changeId") provided by the MnS consumer. Exactly one of "changeId" or "changeIndex" shall be provided. | type: String multiplicity: 0..1 isInvariant: False isWritable: False |
| changeIndex | M | The identification of the operation. It is the positional index of the operation in the operation set ("changeIndex"). The positional index of the leftmost element is "0". Exactly one of "changeId" or "changeIndex" shall be provided. allowedValues: non-negative integer | type: Integer multiplicity: 0..1 isInvariant: False isWritable: False |
| target | M | Target of the change | type: String multiplicity: 1 isInvariant: False isWritable: False |

## 7.6	ActivationJob

### 7.6.1	Definition

This definition represents an activation job.

### 7.6.2	Information Elements

The following table specifies the information elements of an activation job.

# Table 7.6.2-1: Information elements of the "ActivationJob"

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the activation job. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the activation job. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description. | M | The textual human-readable description of the activation job. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| mnsConsumerId | M | The consumer that created the job. It may indicate a human user and/or one or more applications, for example ["userid:janedoe", "appid:12314"]. | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |
| planConfigDescrId | M | The identifier of the planned configuration descriptor, whose operation set is requested to be activated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigGroupDescrId | M | or, alternatively, the identifier of the planned configuration group descriptor, whose operation sets are requested to be activated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| fallbackConfigDescrId | M | or, alternatively, the identifier of the fallback configuration descriptor, whose operation sets are requested to be activated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigDescr | M | or, alternatively, the planned configuration descriptor to be activated, if no planned configuration descriptor was created earlier. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigGroupDescr | M | or, alternatively, the planned configuration group descriptor to be activated, if no planned configuration group descriptor was created earlier. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| createdFallbackConfigDescrId | O | Id of the created Fallback Plan Configuration Descriptor. If isFallbackEnabled is supported, this must be supported too. | type: String multiplicity: 0..1 isInvariant: False isWritable: False |
| isFallbackEnabled | O | This boolean attribute allows to specify if the MnS producer shall create a plan with the current configuration to allow for a later fallback. The plan-id of the fallback plan shall be returned to the MnS consumer. | type: Boolean multiplicity: 1 isInvariant: True isWritable: True defaultValue: false |
| serviceImpact | O | The service impact mode. allowedValues: - LEAST_SERVICE_IMPACT - SHORTEST_TIME | type: ENUM multiplicity: 1 isInvariant: True isWritable: True defaultValue: SHORTEST_TIME |
| isImmediateActivation | O | This boolean attribute specifies if the activation job shall start immediately (value is "True") or, alternatively, by conditional activation (value is "False"). | type: Boolean multiplicity: 1 isInvariant: True isWritable: True defaultValue: true |
| cancelRequest | O | This boolean attribute allows to request to cancel the activation process by setting its value to "True". Setting the value to "False" has no observable result. When the value is set to "True" it cannot be changed any more. | type: Boolean multiplicity: 1 isInvariant: False isWritable: True |
| jobState | M | The activation job state. allowedValues: - NOT_STARTED - QUEUED - RUNNING - CANCELLING - CANCELLED - COMPLETED - FAILED | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| jobDetails | M | Detailed information related to the job, including job related errors. | type: JodDetails multiplicity: 1 isInvariant: False isWritable: False |
| startedAt | M | The date and time at which the activation process started, i.e. the time when the job state transition from "NOT_STARTED" to "RUNNING" occurred. In the "NOT_STARTED" state the information element is absent or carries no information. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: FalseType: DateTime multiplicity: 0..1 isOrdered: NA isUnique: NA |
| stoppedAt | M | The date and time at which the activation process stopped, i.e. the job state transition from "RUNNING" to "COMPLETED" or "FAILED", or from "CANCELLING" to "CANCELLED". In the "NOT_STARTED", "RUNNING" or "CANCELLING" state the information element is absent or carries no information. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| activationState | M | The current activation state of the planned configuration or planned configuration group that is processed by the activation job. allowedValues: - NOT_STARTED - ACTIVATING - ACTIVATION_SUCCEEDED - ACTIVATION_SUCCEEDED_PARTIALLY - ACTIVATION_FAILED - ACTIVATION_FAILED_ROLLED_BACK - ACTIVATION_FAILED_ROLLBACK_FAILED | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| activationDetails | M | Details of the activation of the operations that are contained in the planned configuration or planned configuration group. | type: ExecutionDetails multiplicity: 1 isInvariant: False isWritable: False |

Annex A (normative):
Solution sets

## A.1	RESTful HTTP-based solution set

### A.1.1	General Considerations

Provisions for mapping from stage 2 to stage 3

The naming of information elements in stage-3 generally is the same as in stage-2, except for the following cases:

A "changeId" may or may not be provided for each change by MnS consumers according to stage 2 definitions. The following snippet shows an example where the "changeId" is provided.

"configChanges": [ { "changeId": "change-1", "modifyOperator": "create", "target": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": {} } }, { "changeId": "change-2", "modifyOperator": "create, "target": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME20", "value": { "id": "ME20", "attributes": {} } } ]

Provisions for OpenAPI defined data node trees

The following provisions apply when the planned configuration is for a current configuration that is defined with OpenAPI. The "configChangesContentType" is "OpenAPI". It is based on 3GPP JSON Patch defined in clause 6.4.3 of TS 32.158 [2].

The "target" parameter value is the concatenation of the canonical form of the target URI defined in clause 4.2.3 of TS 32.158 [2] with "scheme://" omitted, and the "path" parameter of 3GPP JSON Patch defined in clause 6.4.3 of TS 32.158 [2], for example:

"target": "example.org/SubNetwork=SN1/ManagedElement=ME1" "target": "example.org/SubNetwork=SN1/ManagedElement=ME1#/attributes/userLabel" "target": "example.org/SubNetwork=SN1/PerfMetricJob=PMJ1#/attributes/perfMetrics/2"

For Managed Object Plans the "target" parameter identifies always a manged object. Therefore, the fragment component (i.e. the part starting with "#") is always missing.

The "value" parameter value is either a JSON object, a JSON array, a string, a number or one of the following three literal names: false, null, true. It is exactly the JSON value of the node identified by "target" without any additional wrappers or containers, for example:

"value": 5 "value": "Bts10" "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" }

For Managed Object Plans the value of the "value" parameter shall comply to the following JSON schema snippet.

type: object properties: id: type: string objectClass: type: string objectInstance: $ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn' attributes: type: object

The allowed values of the "modifyOperator" are as defined in stage 2.

Provisions for YANG defined data node trees

The following provisions apply when the planned configuration is for a current configuration that is defined with YANG. The "configChangesContentType" is "YANG". It is based on YANG Patch defined in RFC 8072 [3].

The "target" parameter value is constructed based on clause 2.4 of RFC 8072 [3] with the following modifications:

-

-	The datastore resource "{+restconf}/data" shall be omitted.

-	The module prefix may be omitted, and the MnS producer shall accept it if the target without the module prefix is unambiguous.

The first example shows a target including the module prefix, and the second example shows the same target without module prefix. The third example shows the same target again without authority component.

"target": "example.org/3gpp-subnetwork:SubNetwork=SN1/3gpp-managed-element:ManagedElement=ME1" "target": "example.org/SubNetwork=SN1/ManagedElement=ME1" "target": "/SubNetwork=SN1/ManagedElement=ME1"

The "value" parameter value is encoded according to RFC 7951 [a]. The module prefix may also be omitted for the "value" parameter.

"value": { "count": 5 } "value": { "name": "Bts10" "value": { "ManagedElement": [ { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } ] }

The allowed values of the "modifyOperator" are as defined in stage 2.

Provisions for mapping the planned configuration in (3GPP) JSON Patch

In case the activation process requires to map the planned configuration into a 3GPP JSON Patch document the following provisions apply:

The "target" parameter value is assigned to the "path" property without modification.

The "value" parameter value is assigned to the "value" property without modification.

The "modifyOperator" parameter value is mapped to the "op" property according to the following rules:

-	The modify operator "create" is strict, the JSON Patch operation "add" is relaxed. Therefore, an "add" operation is only included in the JSON Patch document, if the node to be created does not exist in the current configuration. Otherwise, an error is raised.

-	The modify operator "delete" is relaxed, the JSON Patch operation "remove" is strict. Therefore, a "remove" operation is only generated in the JSON Patch document, if the target node exists in the current configuration, otherwise the "deleteMoi" operation in the plan is not included in the 3GPP JSON Patch document.

-	The modify operator "merge" is strict, the 3GPP JSON Patch operation "merge" is relaxed. Therefore, the "merge" operation is included in the 3GPP JSON Patch document only, if the target node exists in the current configuration. Otherwise an error is raised.

-	The modify operator "merge-create" is relaxed, the 3GPP JSON Patch operation "merge" is relaxed. Therefore, the "merge" operation is always included in the 3GPP JSON Patch document.

Table A.1.1-1 summarizes the property names used in "configChanges" and the corresponding names in 3GPP JSON Patch.

# Table A.1.1-1: Names in "configChanges" and 3GPP JSON Patch

| configChanges property name | 3GPP JSON Patch property name |
| --- | --- |
| target | path |
| modifyOperator | op |
| value | value |

Provisions for mapping the planned configuration into NETCONF

In case the activation process requires to map the planned configuration into NETCONF the following provisions apply:

The "target" parameter value is mapped into a series of nested XML elements with the innermost XML element being the target data node.

The "value" parameter value (in JSON encoding according to RFC 7951 [a]) is mapped to its corresponding XML representation.

The "modifyOperator" parameter value is mapped to the "operation" attribute according to the following rules:

-	The modify operator "create" is strict, NETCONF "create" operation is strict. Therefore, the "create" operation is always included in the NETCONF request.

-	The modify operator "delete" is relaxed, NETCONF "remove" operation is relaxed. Therefore, the "remove" operation is always included in the NETCONF request.

-	The modify operator "merge" is strict, NETCONF "merge" operation is relaxed. Therefore, the "merge" operation is included in the NETCONF request only, if the target data node exists in the current configuration. Otherwise an error is raised.

-	The modify operator "merge-create" is relaxed, NETCONF "merge" is relaxed. Therefore, the "merge" operation is always included in the NETCONF request.

### A.1.2	Resource structure

Table A.1.2-1 provides an overview of the resources on the MnS producer and applicable HTTP methods.

# Table A.1.2-1: Resources and methods

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
|  | …/plan-management/{/MnSVersion}/\ plan-descriptors/ | POST | Creates a new planned configuration descriptor |
|  |  | GET | Reads all plannd configuration descriptors |
|  | …/plan-management/{/MnSVersion}/\ plan-descriptors/{id} | GET | Reads one planned configuration descriptor |
|  |  | PUT | Replaces one planned configuration. descriptor. HTTP method is optional. |
|  |  | PATCH | Patches one planned configuration descriptor. HTTP method is optional. |
|  |  | DELETE | Deletes one planned configuration descriptor |
|  | …/plan-management/{/MnSVersion}/\ plan-group-descriptors | POST | Creates a new planned configuration group descriptor |
|  |  | GET | Reads all plannd configuration group descriptors |
|  | …/plan-management/{/MnSVersion}/\ plan-group-descriptors/{id} | GET | Read one planned configuration group descriptor |
|  |  | PUT | Replace one planned configuration group descriptor |
|  |  | DELETE | Delete one planned configuration group descriptor |
|  | …/{plan-management}/{/MnSVersion}/\ fallback-descriptors | GET | Read all fallback descriptors |
|  | …/{plan-management}/{/MnSVersion}/\ fallback-descriptors/{id} | GET | Read one fallback descriptor |
|  |  | DELETE | Delete one fallback descriptor |
|  | …/plan-management/{/MnSVersion}/\ trigger-descriptors | POST | Create a new trigger descriptor |
|  |  | GET | Read all trigger descriptors |
|  | …/plan-management/{/MnSVersion}/\ trigger-descriptors/{id} | GET | Read one trigger descriptors |
|  |  | PUT | Replace one trigger descriptors |
|  |  | DELETE | Delete one trigger descriptors |
|  | …/{plan-management}/{/MnSVersion}/\ plan-validation-jobs | POST | Create a new plan validation job |
|  |  | GET | Read all validation jobs |
|  | …/{plan-management}/{/MnSVersion}/\ plan-validation-jobs/{id} | GET | Read on validation job |
|  |  | DELETE | Delete one validation job |
|  | …/{plan-management}/{/MnSVersion}/\ plan-validation-jobs/{id}/validation-details | GET | Read validation details of one validation job |
|  | …/{plan-management}/{/MnSVersion}/\ plan-validation-jobs/{id} | PATCH | Cancel one validation job |
|  | …/{plan-management}/{/MnSVersion}/\ plan-validation-jobs | POST | Create a new plan activation job |
|  |  | GET | Read all activation jobs |
|  | …/{plan-management}/{/MnSVersion}/\ plan-activation-jobs/{id} | GET | Read on activation job |
|  |  | DELETE | Deleete one activation job |
|  | …/{plan-management}/{/MnSVersion}/\ plan-activation-jobs/{id}/activation-details | GET | Read validation details of one activation job |
|  | …/{plan-management}/{/MnSVersion}/\ plan-activation-jobs/{id} | PATCH | Cancel one activation job |

Creating, reading, updating and deleting planned configurations

The data node tree on the MnS producer is as follows upon system start up.

{ "plan-descriptors": {}, "plan-group-descriptors": {}, "fallback- descriptors": {}, "trigger- descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} }

### A.1.3	OpenAPI definitions

OpenAPI definitions for the management of planned configurations are specified in Forge, refer to clause 4.3 of TS 28.623 [16] for the Forge location. An example of Forge location is: "https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel18_SA105/".

Directory: OpenAPI

Files:

TS28572_PlanManagement.yaml

### A.1.4	Examples (informative)

The examples in this section are grouped per configuration changes content type (OPENAPI_BASED / YANG_BASED).

### OPENAPI_BASED

A new item of the collection resource "plan-descriptors" is created by MnS consumers using HTTP POST.

POST 3gpp/plan-management/v1/plan-descriptors HTTP/1.1 Host: exampleA.org Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "configChangesContentType": "OPENAPI_BASED", "activationMode": "ATOMIC", "configChanges": [ { "modifyOperator": "create", "target": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The MnS producer allocates the identifier "p1" for the new resource and returns the response. The location header contains the URI of the new resource. The response body contains the representation of the new resource which is equal to the representation received in the request with the "lastModifiedAt" and "validationState" properties added.

HTTP/1.1 201 Created Date: Wed, 21 Aug 2024 15:39:57 GMT Location: http://exampleA.org/3gpp/plan-management/plan-descriptors/p1 Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "activationMode": "ATOMIC", "configChangesContentType": "OPENAPI_BASED", "configChanges": [ { "modifyOperator": "create", "target": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ], "lastModifiedAt": "2025-03-06T16:50:26-08:00", "validationState": "NOT_VALIDATED" }

The resource structure on the MnS producer contains the new resource.

{ "plan-descriptors": { "p1": { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "activationMode": "ATOMIC", "configChangesContentType": "OPENAPI_BASED", "configChanges": [ { "modifyOperator": "create", "path": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ], "lastModifiedAt": "2025-03-06T16:50:26-08:00", "validationState": "NOT_VALIDATED" } }, "plan-group-descriptors": {}, "fallback-config-descriptors": {}, "trigger-condition-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} }

The next example shows how the value of the "location" attribute in the operation can be changed from "Castle Charlottenburg" to "Summer palace Charlottenburg" using JSON Patch.

PATCH 3gpp/plan-management/v1/plan-descriptors/p1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/configChanges/0/value/attributes/location", "value": "Summer palace Charlottenburg" } ]

The second example is for the case where a modified JSON Patch is used.

PATCH 3gpp/plan-management/v1/plan-descriptors/p1 HTTP/1.1 Host: example.org Content-Type: application/json [ { "op": "replace", "path": "/configChanges[changeId='pc1']/value/attributes/location", "value": "Summer palace Charlottenburg" } ]

To read the planned configuration "p1" a MnS consumer might send.

GET 3gpp/plan-management/v1/plan-descriptors/p1 HTTP/1.1 Host: exampleA.org Accept: application/json

The MnS producer may respond as follows.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2025 16:50:26 GMT Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "configChangesContentType": "OPENAPI_BASED", "currentConfigAddress": " example.org/3gpp ", "activationMode": "ATOMIC", "lastModifiedAt": "2025-03-06T16:50:26-08:00", "validationState": "NOT_VALIDATED", "configChanges": [ { "modifyOperator": "CREATE", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

To delete the planned configuration "p1" a MnS consumer might send.

DELETE 3gpp/plan-management/v1/plan-descriptors/p1 HTTP/1.1 Host: exampleA.org

In case of success, the MnS producer returns the following message.

HTTP/1.1 204 No Content Date: Wed, 21 Aug 2024 16:12:45 GMT

To read the planned configuration "p1" a MnS consumer might send.

GET 3gpp/plan-management/v1/plan-descriptors/p1 HTTP/1.1 Host: exampleA.org Accept: application/json

The MnS producer may respond as follows.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2025 16:50:26 GMT Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "configChangesContentType": "OPENAPI_BASED", "currentConfigAddress": " example.org/3gpp ", "activationMode": "ATOMIC", "lastModifiedAt": "2025-03-06T16:50:26-08:00", "validationState": "NOT_VALIDATED", "configChanges": [ { "modifyOperator": "create", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The following shows a snippet reporting member conflicts.

"memberConflicts": [ { "memberConflict": [ { "planConfigDescrId": "pcd-1", "changeId": "change-4" }, { "planConfigDescrId": "pcd-3", "changeId": "change-6" }, { "planConfigDescrId": "pcd-4", "changeId": "change-7" } ] }, { "memberConflict": [ { "planConfigDescrId": "pcd-5", "changeId": "change-1" }, { "planConfigDescrId": "pcd-6", "changeId": "change-9" } ] } ]

The following shows a snippet reporting activation details.

"activationDetails": { "summary": { "notfinished": 1, "succeeded": 1, "failed": 1, "rollBackSucceeded": 0, "rollBackFailed": 0 }, "results": [ { "planConfigDescrId": "pcd1", "changeIndex": 0, "state": "SUCCEEDED" }, { "planConfigDescrId": "pcd1", "changeIndex": 1, "state": "FAILED", "errors": { "type": "REQUEST_OBJECT_TREE_MISMATCH", "title": "The current state of the data node tree prevents activation.", "reason": "NEW_OBJECTS_ID_EXISTS", "detail": "The id of the new object to be created does exist already." } }, { "planConfigDescrId": "pcd1", "changeIndex": 2, "state": "NOT_STARTED" } ] }

### YANG_BASED

The examples in this section show how an operator may make changes to their configuration based on a YANG based schemas and related json encoding (as per A.1.1 Provisions for YANG defined data node trees).  Note that all the examples described in OPENAPI_BASED case above are compatible with the YANG_BASED option and will execute successfully.  The YANG_BASED option will support a broader range of YANG capabilities (e.g. namespace validation and other yang based schema constructs).

Usecase : Create a plan configuration (plan-descriptor)

POST plan-management/v1/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json Accept : application/json,application/problem+json { "name": "Rollout-5G-Dublin-East", "version": "1.0.0", "description": "This is the plan for the new 5G rollout in Dublin east.", "customProperties": { "technology-type": "NR", "location": "Dublin" }, "configChangesContentType": "YANG_BASED", "configChanges": { "modifyOperator": "create", "changeId": "add-nr-cell-001", "comment": "Add new NR cell for initial deployment in Dublin-4 area.", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/NRCellDU=4", "value": { "NRCellDU": [ { "id": "4", "attributes": { "userLabel": "Dublin-1-Cell-4", "administrativeState": "UNLOCKED", … } } ] } } }

A response for the above might look like below

Location: /plan-descriptors/Rollout-5G-Dublin-East-1.0.0 Content-Type: application/json Date: Thu, 02 Oct 2025 19:21:37 GMT { "id": "Rollout-5G-Dublin-East-1.0.0", "name": "Rollout-5G-Dublin-East", "version": "1.0.0", "description": "This is the plan for the new 5G rollout in Dublin east.", "creationTime": "2025-10-02T19:21:37Z", "status": "CREATED", "configChangesContentType": "YANG_BASED", "customProperties": { "technology-type": "NR", "location": "Dublin" }, "configChanges": { "modifyOperator": "create", "changeId": "add-nr-cell-001", "comment": "Add new NR cell for initial deployment in Dublin-4 area.", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/NRCellDU=4", "value": { "NRCellDU": [ { "id": "4", "attributes": { "userLabel": "Dublin-1-Cell-4", "administrativeState": "UNLOCKED" } } ] } } }

An example request with yang semantics including the module name is shown below :

{ "name": "Rollout-5G-Dublin-East", "version": "1.0.0", "description": "This is the plan for the new 5G rollout in Dublin east.", "customProperties": { "technology-type": "NR", "location": "Dublin" }, "configChangesContentType": "YANG_BASED", "configChanges": { "modifyOperator": "create", "changeId": "add-nr-cell-001", "comment": "Add new NR cell for initial deployment in Dublin-4 area.", "target": "/3gpp-common-subnetwork:SubNetwork=Irl/3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1/NRCellDU=4", "value": { # The value may contain any valid json encoded yang as per RFC 7951 "3gpp-nr-nrm-nrcelldu:NRCellDU": [ { "id": "4", "attributes": { "userLabel": "Dublin-1-Cell-4", "administrativeState": "UNLOCKED", … } } ] } } }

Usecase : Plan Activation Job usecases (plan-activation-jobs)

This section shows examples for plan activation job related resources. This example uses the 'inline' form of the plan descriptor:

# Create plan-activation-job POST {apiRoot}/plan-management/v1/plan-activation-jobs HTTP/1.1 Host: example.org Content-Type: application/json Accept : application/json,application/problem+json { "name" : "act-plan-001", "mnsConsumerId" : ["user:joesoap","app:213"], "description": "Activation job for inline 5G cell creation in Dublin-5.", "customProperties": { "requester": "AutomationTool", "deployment-phase": "pilot" }, "planConfigDescr": { "name": "Inline-Rollout-5G-Dublin-5", "version": "1.0.0", "description": "Plan defined inline for new cell deployment.", "customProperties": { "technology-type": "NR", "location": "Dublin-5" }, "isFallbackEnabled" : true, "configChangesContentType": "YANG_BASED", "configChanges": [ { "modifyOperator": "create", "changeId": "add-nr-cell-005", "comment": "Inline job: Add new NR cell 5 to GNBDUFunction 1.", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/NRCellDU=5", "value": { "NRCellDU": [ { "id": "5", "attributes": { "userLabel": "Dublin-5-Cell-5", "administrativeState": "UNLOCKED" } } ] }, { "modifyOperator": "create", "changeId": "add-nr-cell-006", "comment": "Inline job: Add new NR cell 6 to GNBDUFunction 1.", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/NRCellDU=6", "value": { "NRCellDU": [ { "id": "6", "attributes": { "userLabel": "Dublin-6-Cell-6", "administrativeState": "UNLOCKED" } } ] }, { "modifyOperator": "create", "changeId": "add-nr-cell-007", "comment": "Inline job: Add new NR cell 7 to GNBDUFunction 1.", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/NRCellDU=7", "value": { "NRCellDU": [ { "id": "7", "attributes": { "userLabel": "Dublin-7-Cell-7", "administrativeState": "UNLOCKED" } } ] } } ] } } Response HTTP/1.1 201 Created Location: /plan-activation-jobs/job-23451235153465vz5254 Content-Type: application/json { "id": "job-23451235153465vz5254", "name" : " act-plan-001", "description" : "Activation job for inline 5G cell creation in Dublin-5.", "isFallbackEnabled" : true, "planConfigDescrId" : "planxyz", "mnsConsumerId" : ["user:joesoap","app:213"], "jobState" : "RUNNING", "startedAt" : "<some-time>", "activationState" : "NOT_STARTED", "activationDetails": { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/activation-details" }, "cancelRequest" : false, "links" : { "self" : { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111"}, "descriptor" : "{apiRoot}/ProvMnS/1900/plan-descriptors/planxyz"}, "status": { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/status"}, "fallback": { "href" : "{apiRoot}/ProvMnS/1900/fallback-descriptors/fb-001"} } } # GET plan-activation-job status. Report all the relevant status information # related to the plan activation job GET {apiRoot}/plan-management/v1/plan-activation-jobs/myjob-111/status Response { "jobState": "COMPLETE", "activationState": "ACTIVATED", "startedAt": "2024-12-02T13:16:54.088Z", "stoppedAt": "2024-12-02T13:16:58.088Z" } # GET plan-activation-job activation-details # - jobState=COMPLETED / activationState=ACTIVATED # - All configuration edits/operations are successfully activated # - no detailed info on failed edits/operations are reported (none failed) GET {apiRoot}/ plan-management/v1/plan-activation-jobs/myjob-111/activation-details Response { "summary" : { "notfinished": 0, "succeeded": 3, "failed": 0 “rollbackSucceeded”: 0 “rollbackFailed”: 0 “conflicting” : 0 } } # GET plan-activation-job activation-details with verbose option (expand=all) # - jobState=COMPLETED / activationState=ACTIVATED # - All configuration edits/operations are successfully activated # - no detailed info on failed edits/operations are reported (none failed) GET {apiRoot}/plan-management/v1/plan-activation-jobs/myjob-111/activation-details?expand=all Response { "results" : [ { "planDescriptorId" : "plan-desc-001", "configChanges" : [ { "changeId" : "add-nr-cell-005", "state" : "SUCCEEDED", "target" : "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/ NRCellDU=5" }, { "changeId" : "add-nr-cell-004", "state" : "SUCCEEDED", "target" : "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/ NRCellDU=7" }, { "changeId" : "add-nr-cell-007", "state" : "SUCCEEDED", "target" : "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/ NRCellDU=7" } ] } ], "summary" : { "nofinished": 0, "succeeded": 3, "failed": 0, “rollbackSucceeded”: 0 “rollbackFailed”: 0 “conflicting” : 0 } } # GET plan-activation-job result-details with complete configuration edit/operation failure # - jobState=COMPLETED / activationState=ACTIVATION_FAILED # - All configuration edits/operations failed to activate # - Activation details on failed edits/operations are reported GET {apiRoot}/plan-management/v1/plan-activation-jobs/myjob-111/activation-details Response { "results" : [ { "planConfigDescrId" : "planxyz", "changeId" : "opId-001"", "target" : "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/ NRCellDU=1", "state" : "FAILED", "errors" : [ { "type": "ACCESS_CONTROL_CONFLICT ", "badDataNode": "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/_3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1/_3gpp-nr-nrm-nrcelldu:NRCellDU=1", "message": "Data already exists; cannot be created" } } }, { "planConfigDescrId" : "planxyz", "changeId" : "opId-002"", "target" : "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/ NRCellDU=2", "state" : "FAILED", "errors" : [ { "type": "ACCESS_CONTROL_CONFLICT ", "badDataNode": "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/_3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1/_3gpp-nr-nrm-nrcelldu:NRCellDU=2" } ] } }, { "planConfigDescrId" : "planxyz", "changeId" : "opId-003"", "target" : "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/ NRCellDU=3", "state" : "FAILED", "errors" : [ { "type": "ACCESS_CONTROL_CONFLICT", "badDataNode": "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/_3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1/_3gpp-nr-nrm-nrcelldu:NRCellDU=3" } ] } } ] } ], "summary" : { "notfinished": 0, "succeeded": 0, "failed": 3, “rollbackSucceeded”: 0 “rollbackFailed”: 0 “conflicting” : 0 } } # GET plan-activation-job result-details with failure (with expand=all option) # - jobState=COMPLETED / activationState=PARTIALLY_ACTIVATED # - Some configuration edits/operations failed to activate # - Activation details on failed edits/operations are reported # - Activation details on successful edits/operations are reported (expand=all) GET {apiRoot}/plan-management/v1/plan-activation-jobs/myjob-111/activation-details?expand=all Response { "results" : [ { "planDescriptorId" : "planxyz", "changeId" : "opId-001"", "target" : "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/ NRCellDU=1", "state" : "FAILED", "errors" : [ { "type": "DATA_NODE_TREE_ERROR", "badDataNode": "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1/_3gpp-nr-nrm-nrcelldu:NRCellDU=1", "message": "Data already exists; cannot be created" } } }, { "planDescriptorId" : "planxyz", "changeId" : "opId-002"", "target" : "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/ NRCellDU=2", "state" : "SUCCEEDED" }, { "planDescriptorId" : "planxyz", "changeId" : "opId-003"", "target" : "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1/ NRCellDU=3", "state" : "SUCCEEDED" } ] } ], "summary" : { "notfinished": 0, "succeeded": 2, "failed": 1, “rollbackSucceeded”: 0 “rollbackFailed”: 0 “conflicting” : 0 } }

==============Next change==============

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/2014 at commit b26999e0cbc919fe53d367c016a82318c2472ca4

*** START OF CHANGE 1 ***

*** OpenAPI/TS28572_PlanManagement.yaml ***

<CODE BEGINS>

openapi: 3.0.0

info:

title: 3GPP Plan Provisioning Management API

version: 19.1.0

description: API for managing network configuration plans and related jobs

externalDocs:

description: 3GPP TS 28.572; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.572/

servers:

- url: '{MnSRoot}/plan-management/{MnSVersion}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/cm

MnSVersion:

description: Version number of the OpenAPI definition

default: v1

paths:

/plan-descriptors:

post:

tags:

- Plan Descriptor Management

summary: Create a new plan descriptor

description: Creates a new configuration plan descriptor that can be later activated.

operationId: createPlanDescriptor

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PlanConfigurationDescriptorRequest'

example:

name: "Rollout-5G-Dublin-East"

version: "1.0.0"

description: "This is the plan for the new 5G rollout in Dublin east."

customProperties:

technology-type: "NR"

location: "Dublin"

configChangesContentType: YANG_BASED

configChanges:

- modifyOperator: "create"

changeId: "add-nr-cell-001"

description: "Add new NR cell for initial deployment in Dublin-4 area."

target: "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=1/GNBDUFunction=1"

value:

NRCellDU:

- id: '4'

attributes:

userLabel: "Dublin-1-Cell-4"

administrativeState: "UNLOCKED"

responses:

'201':

description: Plan descriptor created successfully

headers:

Location:

description: URI of the created plan descriptor.

schema:

type: string

format: uri-reference

example: "/plan-descriptors/my-plan11"

content:

application/json:

schema:

$ref: '#/components/schemas/PlanConfigurationDescriptor'

'400':

description: Invalid request parameters or malformed input.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'409':

description: A plan descriptor with the given ID already exists.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

get:

tags:

- Plan Descriptor Management

summary: Get plan configuration descriptors

description: Retrieve a list of  existing plan descriptors.

operationId: getPlanDescriptors

responses:

'200':

description: List of the plan configuration descriptors retrieved successfully.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/DescriptorListEntry'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/plan-descriptors/{id}:

parameters:

- in: path

name: id

schema:

type: string

description: Unique identifier of the plan descriptor.

example: "NewNetworkElement10-group-plan-001"

required: true

get:

tags:

- Plan Descriptor Management

summary: Get a specific plan descriptor by ID

description: Retrieve the details of a single plan descriptor using its unique identifier.

operationId: getPlanDescriptorById

responses:

'200':

description: Plan descriptor retrieved successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/PlanConfigurationDescriptorResponse'

'404':

description: Plan descriptor does not exist

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

put:

tags:

- Plan Descriptor Management

summary: Replace a plan descriptor

description: Replace a configuration plan descriptor

operationId: putPlanDescriptor

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PlanConfigurationDescriptorRequest'

example:

{

"name": "Rollout-5G-Dublin-East",

"description": "This is the plan for the new 5G rollout in Dublin east.",

"customProperties": {

"technology-type": "NR",

"location": "Dublin"

},

"configChanges": [

{

"modifyOperator": "merge",

"changeId": "Dublin-Center_DC-01_555777999",

"target": "/SubNetwork=Dublin-Center/ManagedElement=DC-001/NRCellDU=1",

"value": {

"administrativeState": "LOCKED"

}

}

]

}

responses:

'204':

description: Plan descriptor replaced successfully. No content is returned

'400':

description: Invalid request parameters or malformed input.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'404':

description: Plan descriptor not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

delete:

tags:

- Plan Descriptor Management

summary: Delete a plan descriptor by ID

description: Deletes a specific configuration plan descriptor using its unique identifier

operationId: deletePlanDescriptorById

responses:

'204':

description: Plan descriptor deleted successfully.

'404':

description: Plan descriptor not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'  ##EDITOR The example in swagger is very strange. The example cannot be the same for put and delete. TODO LATER

'409':

description: Conflict - The plan descriptor cannot be deleted due to its current state or dependencies (For example, it is referenced in a plan group descriptor).

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/plan-group-descriptors:

post:

tags:

- Plan Group Descriptor Management

summary: Create a new plan group descriptor

description: Creates a new configuration plan group descriptor that can be later activated.

operationId: createPlanGroupDescriptor

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PlanConfigurationGroupDescriptor'

responses:

'201':

description: Plan group descriptor created successfully

headers:

Location:

description: URI of the created plan descriptor.

schema:

type: string

format: uri-reference

example: "/plan-group-descriptors/mygroup-11"

content:

application/json:

schema:

$ref: '#/components/schemas/PlanConfigurationGroupDescriptorResponse'

'400':

description: Invalid request parameters or malformed input.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

get:

tags:

- Plan Group Descriptor Management

summary: Get plan configuration descriptors

description: Retrieve a list of existing plan descriptors.

operationId: getPlanGroupDescriptors

responses:

'200':

description: List of the plan configuration descriptors retrieved successfully.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/DescriptorListEntry'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/plan-group-descriptors/{id}:

parameters:

- in: path

name: id

schema:

type: string

description: Unique identifier of the plan group descriptor.

example: "NewNetworkElement10-group-plan-001"

required: true

get:

tags:

- Plan Group Descriptor Management

summary: Get a specific plan group descriptor by ID

description: Retrieve the details of a plan group descriptor using its unique identifier.

operationId: getPlanGroupDescriptorById

responses:

'200':

description: Plan group descriptor retrieved successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/PlanConfigurationGroupDescriptorResponse'

'404':

description: Plan group descriptor does not exist

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

put:

tags:

- Plan Group Descriptor Management

summary: Replace a plan group descriptor

description: Replace a configuration plan group descriptor

operationId: putPlanGroupDescriptor

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PlanConfigurationGroupDescriptor'

responses:

'200':

description: Plan group descriptor replaced successfully

content:

application/json:

schema:

$ref: '#/components/schemas/PlanConfigurationGroupDescriptorResponse'

'400':

description: Invalid request parameters or malformed input.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'404':

description: Plan group descriptor not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

delete:

tags:

- Plan Group Descriptor Management

summary: Delete a plan group descriptor by ID

description: Deletes a plan group descriptor using its unique identifier

operationId: deletePlanGroupDescriptorById

responses:

'204':

description: Plan descriptor deleted successfully.

'404':

description: Plan descriptor not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'409':

description: The plan group descriptor cannot be deleted due to its current state or dependencies (For example, it is referenced in a plan group descriptor).

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/fallback-descriptors:

get:

tags:

- Fallback Descriptor Management

summary: Get fallback configuration descriptors

description: Retrieve a list of  existing fallback descriptors.

operationId: getFallbackDescriptors

responses:

'200':

description: List of the fallback configuration descriptors retrieved successfully.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/DescriptorListEntry'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/fallback-descriptors/{id}:

parameters:

- in: path

name: id

schema:

type: string

description: Unique identifier of the fallback descriptor.

example: "Fallback-Dublin-plan-001"

required: true

get:

tags:

- Fallback Descriptor Management

summary: Get a specific fallback descriptor by ID

description: Retrieve the details of a single fallback descriptor using its unique identifier.

operationId: getFallbackDescriptorById

responses:

'200':

description: Fallback Plan descriptor retrieved successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/FallbackConfigurationDescriptorResponse'

'404':

description: Fallback descriptor does not exist

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

delete:

tags:

- Fallback Descriptor Management

summary: Delete a Fallback descriptor by ID

description: Deletes a specific fallback plan descriptor using its unique identifier

operationId: deleteFallbackDescriptorById

responses:

'204':

description: Fallback descriptor deleted successfully.

'404':

description: Fallback descriptor not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'  ##EDITOR The example in swagger is very strange. The example cannot be the same for put and delete. TODO LATER

'409':

description: Conflict - The fallback descriptor cannot be deleted due to its current state.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/trigger-descriptors:

post:

tags:

- Trigger Descriptor Management

summary: Create a new trigger descriptor

description: Creates a new trigger descriptor

operationId: createTriggerDescriptor

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TriggerDescriptorRequest'

responses:

'201':

description: Trigger descriptor created successfully

headers:

Location:

description: URI of the created trigger descriptor.

schema:

type: string

format: uri-reference

example: "/trigger-descriptors/mytrigger-11"

content:

application/json:

schema:

$ref: '#/components/schemas/TriggerDescriptorResponse'

'400':

description: Invalid request parameters or malformed input.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

get:

tags:

- Trigger Descriptor Management

summary: Get trigger descriptors

description: Retrieve a list of existing trigger descriptors.

operationId: getTriggerDescriptors

responses:

'200':

description: List of the trigger descriptors retrieved successfully.

content:

application/json:

schema:

type:  array

items:

$ref: '#/components/schemas/DescriptorListEntry'

/trigger-descriptors/{id}:

parameters:

- in: path

name: id

schema:

type: string

description: Unique identifier of the  descriptor.

example: "NewNetworkElement10-trigger-001"

required: true

get:

tags:

- Trigger Descriptor Management

summary: Get a specific trigger descriptor by ID

description: Retrieve the details of a single descriptor using its unique identifier.

operationId: getTriggerDescriptorById

responses:

'200':

description: Trigger descriptor retrieved successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/TriggerDescriptorResponse'

'404':

description: Trigger descriptor not found

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

put:

tags:

- Trigger Descriptor Management

summary: Replace a trigger descriptor

description: Replace a trigger descriptor

operationId: putTriggerDescriptor

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TriggerDescriptorRequest'

responses:

'204':

description: Trigger descriptor created successfully

'400':

description: Invalid request parameters or malformed input.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'404':

description: Trigger descriptor not found

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

delete:

tags:

- Trigger Descriptor Management

summary: Delete a trigger descriptor by ID

description: Deletes a specific trigger descriptor using its unique identifier

operationId: deleteTriggerDescriptorById

responses:

'204':

description: Trigger descriptor deleted successfully.

'404':

description: Trigger descriptor not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/plan-activation-jobs:

post:

tags:

- Activation Management

summary: Create a new plan activation job

description: Creates and starts a new plan activation job based on an existing plan descriptor (or plan group descriptor or fallback descriptor).  The new job's ID will be generated by the server and returned in the Location header.

operationId: createActivationJob

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ActivationJobWritableProperties'

responses:

'201':

description: Plan activation job created successfully.

The response body provides job details, and the Location header points to the new job.

headers:

Location:

description: URI of the created job resource.

schema:

type: string

format: uri-reference

example: "/plan-activation-jobs/myjob-111"

content:

application/json:

schema:

$ref: '#/components/schemas/ActivationJob'

'400':

description: Invalid request payload or parameters (e.g., malformed JSON, missing required fields).

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

get:

tags:

- Activation Management

summary: Get plan activation jobs

description: Retrieve a list of plan activation jobs.

operationId: getActivationJobs

parameters:

- in: query

name: job-state

schema:

$ref: '#/components/schemas/JobState'

description: Filter jobs by their current state.

example: "RUNNING"

responses:

'200':

description: List of plan activation jobs retrieved successfully.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/JobListEntry'

'400':

description: Invalid query parameters

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/plan-activation-jobs/{id}:

parameters:

- in: path

name: id

schema:

type: string

description: Unique identifier of the plan activation job.

example: "Dublin-plan-activation-001"

required: true

get:

tags:

- Activation Management

summary: Get plan activation job details by ID

description: Retrieve detailed information about a specific plan activation job using its unique identifier.

operationId: getActivationJobById

responses:

'200':

description: Job details retrieved successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ActivationJob'

'404':

description: Job not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

delete:

tags:

- Activation Management

summary: Delete a plan activation job by ID

description: Deletes a specific plan activation job, typically if it's not in a terminal state (e.g., running, completed, failed).

operationId: deleteActivationJobById

responses:

'204':

description: Job deleted successfully. No content is returned.

'404':

description: Job not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'409':

description:  Cannot delete job in its current state (e.g., already running).

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

patch:

tags:

- Activation Management

summary: Cancel the activation job

description: Cancel the activation job

operationId: cancelActivationJobById

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CancelRequest'

application/merge-patch+json:

schema:

$ref: '#/components/schemas/CancelRequest'

responses:

'202':

description: Job cancel request was accepted and cancellation is ongoing

'204':

description: Job cancel request was successfully completed

'404':

description: Job not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'422':

description: Semantic error - e.g. job was not in RUNNING state

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/plan-activation-jobs/{id}/status:

get:

tags:

- Activation Management

summary: Get job status

description: Retrieve the current status of a specific plan activation job using its unique identifier.

operationId: getJobStatus

parameters:

- in: path

name: id

schema:

type: string

description: Unique identifier of the plan activation job.

example: "Dublin-plan-activation-001"

required: true

responses:

'200':

description: Job status retrieved successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ActivationJobStatus'

'404':

description: Job not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/plan-activation-jobs/{id}/activation-details:

get:

tags:

- Activation Management

summary: Get activation details

description: Retrieve detailed information about the activation results of a job

operationId: getActivationDetails

parameters:

- in: path

name: id

schema:

type: string

description: Unique identifier of the plan activation job.

example: "Dublin-plan-activation-001"

required: true

- name: expand

in: query

description: >-

By default the activation details results shall include the FAILED operations only.  This parameter indicates whether to expand all details of the activation results to also include the 'SUCCEEDED' operations.

required: false

schema:

type: string

enum:

- all

responses:

'200':

description: Activation details retrieved successfully

content:

application/json:

schema:

$ref: '#/components/schemas/ExecutionDetails'

'404':

description: Job not found

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/plan-validation-jobs:

post:

tags:

- Validation Management

summary: Create a new plan validation job

description: Creates and starts a new plan validation job based on an existing plan descriptor.  The new job's ID will be generated by the server and returned in the Location header.

operationId: createValidationJob

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ValidationJobRequest'

responses:

'201':

description: Plan validation job created successfully.

The response body provides job details, and the Location header points to the new job.

headers:

Location:

description: URI of the created job resource.

schema:

type: string

format: uri-reference

example: "/plan-validation-jobs/myjob-111"

content:

application/json:

schema:

$ref: '#/components/schemas/ValidationJob'

'400':

description: Invalid request payload or parameters (e.g., malformed JSON, missing required fields).

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

get:

tags:

- Validation Management

summary: Get plan validation jobs

description: Retrieve a list of plan validation jobs.

operationId: getValidationJobs

parameters:

- in: query

name: job-state

schema:

$ref: '#/components/schemas/JobState'

description: Filter jobs by their current state.

example: "job-state=COMPLETED" # Example state for validation jobs

responses:

'200':

description: List of plan validation jobs retrieved successfully.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/JobListEntry'

'400':

description: Invalid query parameters

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/plan-validation-jobs/{id}:

parameters:

- in: path

name: id

schema:

type: string

description: Unique identifier of the plan validation job.

example: "Dublin-plan-validation-001"

required: true

get:

tags:

- Validation Management

summary: Get plan validation job details by ID

description: Retrieve detailed information about a specific plan validation job using its unique identifier.

operationId: getValidationJobById

responses:

'200':

description: Job details retrieved successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ValidationJob'

'404':

description: Job not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

delete:

tags:

- Validation Management

summary: Delete a plan validation job by ID

description: Deletes a specific plan validation job, typically if it's not in a terminal state (e.g., running, completed, failed).

operationId: deleteValidationJobById

responses:

'204':

description: Job deleted successfully. No content is returned.

'404':

description: Job not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

patch:

tags:

- Validation Management

summary: Cancel the validation job

description: Cancel the validation job

operationId: cancelValidationJobById

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CancelRequest'

responses:

'202':

description: Job cancel request was accepted and cancellation is ongoing

'204':

description: Job cancel request was successfully completed

'404':

description: Job not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'422':

description: Semantic error - e.g. job was not in RUNNING state

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/plan-validation-jobs/{id}/status:

get:

tags:

- Validation Management

summary: Get job status

description: Retrieve the current status of a specific plan validation job using its unique identifier.

operationId: getValidationJobStatus

parameters:

- in: path

name: id

schema:

type: string

description: Unique identifier of the plan validation job.

example: "Dublin-plan-validation-001"

required: true

responses:

'200':

description: Job status retrieved successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ValidationJobStatus'

'404':

description: Job not found.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

'500':

description: Internal server error.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

/plan-validation-jobs/{id}/validation-details:

get:

tags:

- Validation Management

summary: Get validation details

description: Retrieve detailed information about the validation results of a job

operationId: getValidationDetails

parameters:

- in: path

name: id

schema:

type: string

description: Unique identifier of the plan validation job.

example: "Dublin-South-plan-validation-001"

required: true

- name: details

in: query

description: Selects the level of details to return.

schema:

type: string

enum: [summary, all]

default: all

responses:

'200':

description: Validation details retrieved successfully

content:

application/json:

schema:

$ref: '#/components/schemas/ExecutionDetails'

'404':

description: Job not found

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ErrorDetail'

components:

schemas:

PlanConfigurationDescriptor:

allOf:

- $ref: '#/components/schemas/PlanConfigurationDescriptorBaseProperties'

- type: object

required:

- id

- activationMode

- lastModifiedAt

- validationState

- configChangesContentType

- configChanges

properties:

id:

type: string

description: The identifier of the planned configuration

example: plan-001

configChanges:

description: "The plan configuration changes"

type: array

items:

$ref: '#/components/schemas/ConfigChange'

validationState:

type: string

enum: [NOT_VALIDATED, VALID, INVALID]

default : NOT_VALIDATED

description: The validation state of the plan.

example: NOT_VALIDATED

lastModifiedAt:

type: string

format: date-time

description: the last time the plan was modified

example: 2025-03-06T16:50:26-08:00

lastValidatedAt:

type: string

format: date-time

description: last time the plan was validated

example: 2025-03-06T16:50:29-08:00

PlanConfigurationDescriptorRequest:

allOf:

- $ref: '#/components/schemas/PlanConfigurationDescriptorBaseProperties'

- type: object

required:

- configChanges

properties:

configChanges:

description: "The plan configuration changes"

type: array

items:

$ref: '#/components/schemas/ConfigChangeWritable'

PlanConfigurationDescriptorResponse:

allOf:

- $ref: '#/components/schemas/PlanConfigurationDescriptor'

- type: object

properties:

_links:

allOf:

- $ref: '#/components/schemas/SelfLink'

example:

self:

href: "{root-url}/plan-management/v1/plan-descriptors/pd-001"

templated: true

type: "application/json"

title: "The newly created PlanConfigurationDescriptor"

PlanConfigurationDescriptorBaseProperties:

type: object

properties:

name:

type: string

description: Descriptive name of the plan group configuration descriptor

example: Rollout-5G-Dublin-East

version:

type: string

description: The version of the planned configuration. Its format is implementation specific.

example: "1.0.0"

description:

type: string

description: Used to describe the purpose of the plan configuration

example: This is the plan for the new 5G rollout in Dublin east.

customProperties:

type: object

description: A dynamic set of custom properties provided by client

additionalProperties: true

example:

technology-type: NR

location: Dublin

configChangesContentType:

$ref: '#/components/schemas/ConfigChangesContentType'

activationMode:

type: string

enum: [ATOMIC, BEST_EFFORT, STOP_ON_ERROR]

default : BEST_EFFORT

description: Specifies the execution behavior when the plan is activated

example: BEST_EFFORT

PlanConfigurationGroupDescriptor:

type: object

required :

- members

properties:

id:

type: string

description: Unique id of the plan group configuration descriptor

example: "plan-001"

name:

type: string

description: Descriptive name of the plan group configuration descriptor

example: "Rollout-5G-Dublin-East"

version:

type: string

description: version of the plan group configuration descriptor

example: "1.0.0"

description:

type: string

description: Used to describe the purpose of the plan group configuration

example: "This is the plan for the new 5G rollout in Dublin east."

customProperties:

type: object

description: A dynamic set of custom properties provided by client

additionalProperties: true

example:

technology-type: NR

location: Dublin

isOrdered:

type: boolean

description: Specifies if the members of the planned configuration group are ordered. When ordered, the planned configuration group members shall be validated/activated in the specified order. When not ordered the planned configuration group members can be validated/activated in any order

default: false

isFailOnMemberConflicts:

type: boolean

description: Specifies if the activation shall fail on detection of conflicts between planned configuration group members, or if the operations shall be processed as if there were no conflicts

default: false

activationMode:

type: string

enum: [ATOMIC, BEST_EFFORT, STOP_ON_ERROR]

default : "BEST_EFFORT"

description: Specifies the execution behavior when the plan configuration group is activated

example: "BEST_EFFORT"

validationState:

type: string

enum: [NOT_VALIDATED, PARTIALLY_VALID, VALID, INVALID]

default : "NOT_VALIDATED"

description: The validation state for the last time plan configuration group was validated

example: "NOT_VALIDATED"

lastModifiedAt:

type: string

format: date-time

description: the last time the plan was modified

example: "2025-03-06T16:50:26-08:00"

lastValidatedAt:

type: string

format: date-time

description: last time the plan was validated

example: "2025-03-06T16:50:29-08:00"

members:

type: array

description: list of plan or plan group descriptor identifiers

items:

$ref: '#/components/schemas/Member'

example:

- planConfigDescrId: pcd-network-101

- planConfigGroupDescrId: pgc-europe-east-group

- planConfigDescrId: pcd-radio-tuning-005

- planConfigGroupDescrId: pgc-5g-core-rollout

PlanConfigurationGroupDescriptorResponse:

allOf:

- $ref: '#/components/schemas/PlanConfigurationGroupDescriptor'

- type: object

required:

- id

- activationMode

- validationState

- isOrdered

- isFailOnMemberConflicts

- members

properties:

_links:

type: object

description: Hypermedia links for plan descriptor

allOf:

- $ref: '#/components/schemas/SelfLink'

example:

self:

href: "{root-url}/plan-management/v1/plan-group-descriptors/pgd-001"

templated: true

type: "application/json"

title: "The newly created PlanConfigurationGroupDescriptor"

Member:

type: object

description: |

Defines a member identified by EITHER a single Plan Configuration Descriptor ID

OR a Plan Configuration Group Descriptor ID, but not both.

oneOf:

- type: object

required: [planConfigDescrId]

properties:

planConfigDescrId:

type: string

description: Unique id of the plan configuration descriptor.

example: "pcd-001"

- type: object

required: [planConfigGroupDescrId]

properties:

planConfigGroupDescrId:

type: string

description: Unique id of the plan configuration group descriptor.

example: "pgc-001"

FallbackConfigurationDescriptor:

type: object

properties:

id:

type: string

description: Unique id of the fallback configuration descriptor

example: "fallback-001"

name:

type: string

description: Descriptive name of the fallback configuration descriptor

example: "Fallback-Rollout-5G-Dublin-East"

description:

type: string

description: Used to describe the purpose of the fallback configuration

example: "Fallback for configuration plan Rollout-5G-Dublin-East."

version:

type: string

description: The version of the fallback configuration. Its format is implementation specific.

example: 1.0.0

customProperties:

type: object

description: A dynamic set of custom properties provided by client

additionalProperties: true

example:

technology-type: NR

location: Dublin

activationJob:

type: string

description: The identifier of the related activation job.

required:

- activationJob

oneOf:

- type: object

description: COMPLETE_RESTORE type fallback

required:

- fallbackConfig

properties:

fallbackConfig:

type: string

description: The fallback configuration associated with COMPLETE_RESTORE

- type: object

description: The configuration changes associated with UNDO type fallback

properties:

configChangesContentType:

$ref: '#/components/schemas/ConfigChangesContentType'

configChanges:

type: array

description: |

The configuration change set specifying the configuration in case the fallback configuration is of the "UNDO" type. The format of "configChanges" is specified in "configChangesContentType". Exactly one of this element or the fallbackConfig shall be present.

items:

$ref: '#/components/schemas/ConfigChangeWritable'

required:

- configChanges

- configChangesContentType

FallbackConfigurationDescriptorResponse:

allOf:

- $ref: '#/components/schemas/FallbackConfigurationDescriptor'

- type: object

properties:

_links:

type: object

description: Hypermedia links for fallback descriptor

allOf:

- $ref: '#/components/schemas/SelfLink'

example:

self:

href: "{root-url}/plan-management/v1/fallback-descriptors/pd-001"

templated: true

type: "application/json"

title: "The newly created FallbackConfigurationDescriptor"

TriggerDescriptorRequest:

allOf:

- $ref: '#/components/schemas/TriggerDescriptorBaseProperties'

TriggerDescriptorResponse:

allOf:

- $ref: '#/components/schemas/TriggerDescriptor'

- type: object

properties:

_links:

type: object

description: Hypermedia links for trigger descriptor

allOf:

- $ref: '#/components/schemas/SelfLink'

example:

self:

href: "{root-url}/plan-management/v1/trigger-descriptors/trigger-001"

templated: true

type: "application/json"

title: "The newly created TriggerDescriptor"

TriggerDescriptorBaseProperties:

type: object

required:

- conditionExpression

- evaluationPeriod

- activationJobs

properties:

name:

type: string

description: Descriptive name of the trigger descriptor

example: "Rollout-5G-Dublin-East"

description:

type: string

description: Used to describe the purpose of the trigger

example: "This is the plan for the new 5G rollout in Dublin east."

version:

type: string

description: an optional version for the Trigger Descriptor

example: "1.0.0"

customProperties:

type: object

description: A dynamic set of custom properties provided by client

additionalProperties: true

example:

technology-type: NR

location: Dublin

conditionExpression:

type: string

description: The condition expression

activationJobs:

type: array

items:

type : string

description: The identifiers of one or more activation jobs that shall be triggered by this condition.

startEvaluationAt:

type: string

format: date-time

description: The date and time at which the evaluation of the condition expression shall start. The evaluation result is set to "False" before that date and time. If the information element is not specified, evaluation of the trigger condition shall start immediately.

example: "2025-03-06T16:50:26-08:00"

stopEvaluationAt:

type: string

format: date-time

description: The date and time at which the evaluation of the condition expression shall stop. The evaluation result is set to "False" after that date and time. If the information element is not specified, evaluation of the trigger condition shall continue until the deletion of the trigger condition descriptor.

example: "2025-03-06T16:50:26-08:00"

isTriggerOnce:

type: boolean

default : true

description: The boolean indication, if the trigger is disarmed after the first firing.

example: false

evaluationPeriod:

type: integer

description: The evaluation period specifies the interval of time in seconds  between two consecutive condition expression evaluations.

hysteresis:

type: object

allOf:

- $ref: '#/components/schemas/Hysteresis'

description: The hysteresis, when present, specifies that the trigger shall not be activated immediately when the evaluation result changes from false to true or a specified number of times.  Values greater or equal to 1 are allowed

additionalProperties: true

Hysteresis:

type: object

properties:

timeOfTrueEvaluations:

type: integer

description: The hysteresis, when present, specifies that the trigger shall not be activated immediately, when the evaluation result changes from false to true, but only when the evaluation results is true for a specified time (which must be a multiple of the evaluation period). Unit is seconds

numberOfTrueEvaluations:

type: integer

description: This information element, when present, specifies that the trigger shall not be activated immediately, when the evaluation result changes from false to true, but only when the evaluation results is true for a specified time (which must be a multiple of the evaluation period) or a specified number of times.

TriggerDescriptor:

allOf:

- $ref: '#/components/schemas/TriggerDescriptorBaseProperties'

- type: object

properties:

id:

type: string

description: Unique id of the plan configuration descriptor

example: "trigger-001"

currentEvaluationResult:

type: boolean

default : false

description: The current result of evaluating the "condition-expression".

example: false

lastModifiedAt:

type: string

format: date-time

description: The date and time at which the trigger condition was modified the last time by a MnS consumer. Upon creation of the trigger condition descriptor the value of the information element is set to the date and time at which the descriptor is created.

example: "2025-03-06T16:50:29-08:00"

lastTriggeredAt:

type: string

format: date-time

description: The date and time at which the evaluation result of the trigger condition changed the last time from "False" to "True".

example: "2025-03-06T16:50:29-08:00"

isTriggerActive:

type: boolean

default : false

description: The indication if the trigger can start activation jobs (trigger is active), or if the trigger cannot start activation jobs (trigger is inactive).

example: false

required:

- conditionExpression

- evaluationPeriod

DescriptorListEntry:

type: object

properties:

id:

type: string

description: id of the descriptor.

example: plan-descriptor-1

name:

type: string

description: name of the descriptor.

example: Dublin East Rollout

version:

type: string

description: version of the descriptor.

example: "1.0.0"

description:

type: string

description: description/purpose of the descriptor.

example: "plan-descriptor-1"

required :

- id

JobListEntry:

allOf:

- type: object

properties:

id:

type: string

description: id of the job.

example: plan-job-1

name:

type: string

description: name of the job.

example: Dublin East Rollout Job

description:

type: string

description: description/purpose of the job.

example: Job to do dublin east rollout

jobState:

$ref: '#/components/schemas/JobState'

required :

- id

- jobState

ActivationJobWritableProperties:

type: object

properties:

name:

type: string

description: Name of the activation job

example: "Dublin East Cell Deployment"

description:

type: string

description: Human-readable description of the job

example: "Optimize the Dublin area network"

mnsConsumerId:

type: array

description: The consumer that created and/or started the job. It may indicated a human user and/or one or more applications initiating the job. E.g. ["userid:janedoe", "appid:12314"]

items:

type: string

isFallbackEnabled:

type: boolean

description: Whether fallback should be enabled for this job

default: false

serviceImpact:

type: string

enum: [LEAST_SERVICE_IMPACT, SHORTEST_TIME]

description: Additional job-specific parameters

default: "SHORTEST_TIME"

isImmediateActivation:

type: boolean

description: Specifies if the activation job shall start immediately or, alternatively, by conditional activation.

default: true

oneOf:

- $ref: '#/components/schemas/PlanRef'

- $ref: '#/components/schemas/PlanGroupRef'

- $ref: '#/components/schemas/EmbeddedPlan'

- $ref: '#/components/schemas/EmbeddedPlanGroup'

- $ref: '#/components/schemas/FallbackPlanRef'

example:

{

"name" : "5G-Dublin-East-Rollout",

"description" : "Optimize the 5G network in Dublin East",

"planConfigDescr" : {

"activationMode" : "ATOMIC",

"customProperties" : {

"technology-type": "NR",

"location": "Dublin"

},

"configChangesContentType" : "YANG_BASED",

"configChanges": [

{

"modifyOperator": "merge",

"changeId": "Dublin-Center_DC-01_555777999",

"target": "/SubNetwork=Dublin-Center/ManagedElement=DC-001/NRCellDU=1",

"value": {

"administrativeState": "LOCKED"

}

}

]

}

}

PlanRef:

type: object

required: [planConfigDescrId]

properties:

planConfigDescrId:

type: string

EmbeddedPlan:

type: object

required: [planConfigDescr]

properties:

planConfigDescr:

$ref: '#/components/schemas/PlanConfigurationDescriptorRequest'

PlanGroupRef:

type: object

required: [planConfigGroupDescrId]

properties:

planConfigGroupDescrId:

type: string

EmbeddedPlanGroup:

type: object

required: [planConfigGroupDescr]

properties:

planConfigGroupDescr:

$ref: '#/components/schemas/PlanConfigurationGroupDescriptor'

FallbackPlanRef:

type: object

required: [fallbackConfigDescrId]

properties:

fallbackConfigDescrId:

type: string

ActivationJobStatus:

allOf:

- type: object

properties:

jobState: { $ref: '#/components/schemas/JobState' }

activationState: { $ref: '#/components/schemas/ActivationState' }

startedAt: { type: string, format: date-time }

stoppedAt: { type: string, format: date-time }

required: [jobState, activationState]

ActivationJob:

allOf:

- $ref: '#/components/schemas/ActivationJobWritableProperties'

- type: object

properties:

id:

type: string

description: id of the activation job

example: "job-id-3985199134"

jobState:

allOf:

- $ref: '#/components/schemas/JobState'

example: "COMPLETED"

activationState:

allOf:

- $ref: '#/components/schemas/ActivationState'

example: "ACTIVATED"

startedAt:

type: string

format: date-time

example: "2024-12-02T13:16:54.088Z"

stoppedAt:

type: string

format: date-time

example: "2024-12-02T13:16:58.088Z"

jobDetails:

allOf:

- $ref: '#/components/schemas/JobDetails'

activationDetails:

allOf:

- $ref: '#/components/schemas/ExecutionDetails'

cancelRequest:

type: boolean

description: This boolean attribute allows to request to cancel the activation process by setting its value to "True". Setting the value to "False" has no observable result. Once the value is set to "True" it is immutable

default: false

createdFallbackConfigDescrId:

type: string

description: Id of the created Fallback Plan Configuration Descriptor

example: "fallback-plan-descriptor-001"

_links:

$ref: '#/components/schemas/ActivationJobLinks'

required: [id, jobState, jobDetails, activationState, activationDetails, _links]

additionalProperties : true

ValidationJobRequest:

allOf:

- $ref: '#/components/schemas/ValidationJobBaseProperties'

example:

{

"name" : "5G-Dublin-East-Rollout",

"description" : "Optimize the 5G network in Dublin East",

"planConfigDescr" : {

"activationMode" : "ATOMIC",

"customProperties" : {

"technology-type": "NR",

"location": "Dublin"

},

"configChangesContentType" : "YANG_BASED",

"configChanges": [

{

"modifyOperator": "merge",

"changeId": "Dublin-Center_DC-01_555777999",

"target": "/SubNetwork=Dublin-Center/ManagedElement=DC-001/NRCellDU=1",

"value": {

"administrativeState": "LOCKED"

}

}

]

}

}

ValidationJobBaseProperties:

type: object

properties:

name:

type: string

description: Name of the validation job

example: "Dublin East Cell Deployment"

description:

type: string

description: Human-readable description of the job

example: "Optimize the Dublin area network"

mnsConsumerId:

type: array

description: The consumer that created and/or started the job. It may indicated a human user and/or one or more applications initiating the job. E.g. ["userid:janedoe", "appid:12314"]

items:

type: string

validationMode:

type: string

enum: [CONTINUE_ON_ERROR, STOP_ON_ERROR]

default : "CONTINUE_ON_ERROR"

description: Specifies the execution behavior when the plan is activated

oneOf:

- $ref: '#/components/schemas/PlanRef'

- $ref: '#/components/schemas/PlanGroupRef'

- $ref: '#/components/schemas/EmbeddedPlan'

- $ref: '#/components/schemas/EmbeddedPlanGroup'

- $ref: '#/components/schemas/FallbackPlanRef'

example:

{

"name" : "5G-Dublin-East-Rollout",

"description" : "Optimize the 5G network in Dublin East",

"planConfigDescr" : {

"activationMode" : "ATOMIC",

"customProperties" : {

"technology-type": "NR",

"location": "Dublin"

},

"configChangesContentType" : "YANG_BASED",

"configChanges": [

{

"modifyOperator": "merge",

"changeId": "Dublin-Center_DC-01_555777999",

"target": "/SubNetwork=Dublin-Center/ManagedElement=DC-001/NRCellDU=1",

"value": {

"administrativeState": "LOCKED"

}

}

]

}

}

additionalProperties : true

ValidationJob:

allOf:

- $ref: '#/components/schemas/ValidationJobBaseProperties'

- $ref: '#/components/schemas/ValidationJobStatus'

- type: object

properties:

id:

type: string

description: id of the validation job

example: "job-id-3985199134"

currentConfigTime:

type: string

format : date-time

description: The date and time of the current configuration state against which the planned configuration or planned configuration group is validated.

cancelRequest:

type: boolean

description: boolean indicating the request of a job cancellation

default: false

example: true

jobDetails:

$ref: '#/components/schemas/JobDetails'

validationDetails:

$ref: '#/components/schemas/ExecutionDetails'

_links:

$ref: '#/components/schemas/ValidationJobLinks'

required:

- id

- jobState

- jobDetails

- cancelRequest

- validationMode

- validationState

- validationDetails

- _links

ValidationJobStatus:

type: object

required: [jobState, validationState]

properties:

jobState:

allOf:

- $ref: '#/components/schemas/JobState'

example: "COMPLETED"

validationState:

allOf:

- $ref: '#/components/schemas/ValidationState'

example: "NOT_VALIDATED"

startedAt:

type: string

format: date-time

example: "2024-12-02T13:16:54.088Z"

stoppedAt:

type: string

format: date-time

example: "2024-12-02T13:16:58.088Z"

ConfigChangeWritable:

type: object

properties:

modifyOperator:

type: string

enum: [create, merge, merge-create, delete]

description: The operation to perform

example: "create"

description:

type: string

description: text describing the change

example: "modify NR cell for optimisation"

target:

type: string

description: Target data node path

example: "/_3gpp-common-subnetwork:SubNetwork=Irl/3gpp-common-mecontext:MeContext=Dublin-1/_3gpp_nrm_managedelement:ManagedElement=1/_3gpp_nrm_nrcelldu:NRCellDU=4"

changeId:

description:  Unique identifier for this change.  Its value must be unique in any list of changes in a PlanDescriptor.

MnSProducer may ignore this property even if provided in favour of an auto-positional array index matching this configChange array entry.

If not provided the response will default to changeIndex

example: "cell-operation-001"

value:

type: object

additionalProperties: true # Allows any nested properties within 'value'

description: Value to apply (for create/merge/merge-create operations)

additionalProperties: true # Allows for additional properties to be provided

required:

- modifyOperator

- target

ConfigChange:

type: object

properties:

modifyOperator:

type: string

enum: [create, merge, merge-create, delete]

description: The operation to perform

example: "create"

description:

type: string

description: text describing the change

example: "modify NR cell for optimisation"

target:

type: string

description: Target data node path

example: "/_3gpp-common-subnetwork:SubNetwork=Irl/3gpp-common-mecontext:MeContext=Dublin-1/_3gpp_nrm_managedelement:ManagedElement=1/_3gpp_nrm_nrcelldu:NRCellDU=4"

value:

type: object

additionalProperties: true # Allows any nested properties within 'value'

description: Value to apply (for create/merge/merge-create operations)

oneOf:

- required: [changeId]

properties:

changeId:

type: string

description: |

Unique identifier for this change as originally provided in the request.

The MnS Producer may ignore it and use positional index instead.

If not present in the request, the response will use `changeIndex`.

example: "cell-operation-001"

- required: [changeIndex]

properties:

changeIndex:

type: integer

description: |

Zero-based index of this change in the original `configChanges` array.

Used when no `changeId` was provided or when the producer prefers positional addressing.

example: 0

additionalProperties: true # Allows for additional properties to be provided

required:

- modifyOperator

- target

ActivationJobLinks:

allOf:

- type: object

properties:

self:

allOf:

- $ref: '#/components/schemas/LinkObject'

- type: object

properties:

href:

type: string

default: "{apiRoot}/plan-management/v1/plan-activation-jobs/{activationJobId}"

title:

type: string

enum:

- "Link to the plan activation job"

method:

type: string

enum:

- "GET"

example:

href: "{apiRoot}/plan-management/v1/plan-activation-jobs/activation-job-001"

title: "Link to the plan activation job"

type: "application/json"

templated: true

method: GET

descriptor:

description: A URI reference to the plan or plan group configuration descriptor

allOf:

- $ref: '#/components/schemas/LinkObject' # Any other link will conform to LinkObject schema

- type: object

properties:

href:

type: string

example: "{apiRoot}/plan-management/v1/plan-descriptors/{id}"

title:

type: string

enum:

- "Link to the plan (or plan group) configuration descriptor"

method:

type: string

enum:

- "GET" # This means the method MUST be "GET"

example:

href: "{apiRoot}/plan-management/v1/plan-descriptors/plan-descriptor-001"

title: "Link reference to the plan (or plan group) configuration descriptor"

type: "application/json"

templated: true

method: GET

status:

allOf:

- $ref: '#/components/schemas/LinkObject' # Any other link will conform to LinkObject schema

- type: object

properties:

href:

type: string

default: "{apiRoot}/plan-management/v1/plan-activation-jobs/{id}/status"

title:

type: string

enum:

- "Link to GET the job status"

method:

type: string

enum:

- "GET" # This means the method MUST be "GET"

description: A URI reference to the status information

example:

href: "{apiRoot}/plan-management/v1/plan-activation-jobs/myjob-111/status"

title: "Link to GET the job status"

type: "application/json"

templated: true

method: GET

validationDetails:

allOf:

- $ref: '#/components/schemas/LinkObject' # Any other link will conform to LinkObject schema

- type: object

properties:

href:

type: string

default: "{apiRoot}/plan-management/v1/plan-activation-jobs/{id}/activation-details"

title:

type: string

enum:

- "Link to GET the job details"

method:

type: string

enum:

- "GET"

description: A URI reference to the status information

example:

href: "{apiRoot}/plan-management/v1/plan-activation-jobs/myjob-111/status"

title: "Link to GET the job status"

type: "application/json"

templated: true

method: GET

cancel:

allOf:

- $ref: '#/components/schemas/LinkObject' # Any other link

- type: object

properties:

href:

type: string

default: "{apiRoot}/plan-management/v1/plan-activation-jobs/{id}"

title:

type: string

enum:

- "Link to cancel the job"

method:

type: string

enum:

- "PATCH"

description: A URI reference to cancel the job

example:

href: "{apiRoot}/plan-management/v1/plan-activation-jobs/myjob-111"

title: "Link to cancel the job"

type: "application/json"

templated: true

method: PATCH

fallback:

allOf:

- $ref: '#/components/schemas/LinkObject'

description: A URI reference to the fallback plan descriptor

example:

href: "{apiRoot}/plan-management/v1/fallback-descriptors/fallback-myjob-111"

templated: true

type: "application/json"

title: "A URI reference to the fallback plan descriptor"

method: GET

# additionalProperties to allow any other dynamic links

additionalProperties:

$ref: '#/components/schemas/LinkObject' # Any other link will conform to LinkObject schema

required:

- self

- descriptor

- status

- validationDetails

- cancel

ValidationJobLinks:

allOf:

- type: object

properties:

self:

allOf:

- $ref: '#/components/schemas/LinkObject'

- type: object

properties:

href:

type: string

default: "{apiRoot}/plan-management/v1/plan-validation-jobs/{validationJobId}"

title:

type: string

enum:

- "Link to the plan validation job"

method:

type: string

enum:

- "GET"

example:

href: "{apiRoot}/plan-management/v1/plan-validation-jobs/val-job-001"

title: "Link to the plan validation job"

type: "application/json"

templated: true

method: GET

descriptor:

description: A URI reference to the plan (or plan group) configuration descriptor

allOf:

- $ref: '#/components/schemas/LinkObject' # Any other link will conform to LinkObject schema

- type: object

properties:

href:

type: string

example: "{apiRoot}/plan-management/v1/plan-descriptors/{id}"

title:

type: string

enum:

- "Link to the plan (or plan group) configuration descriptor"

method:

type: string

enum:

- "GET"

example:

href: "{apiRoot}/plan-management/v1/plan-descriptors/plan-descriptor-001"

title: "Link reference to the plan (or plan group) configuration descriptor"

type: "application/json"

templated: true

method: GET

status:

allOf:

- $ref: '#/components/schemas/LinkObject' # Any other link will conform to LinkObject schema

- type: object

properties:

href:

type: string

default: "{apiRoot}/plan-management/v1/plan-validation-jobs/{id}/status"

title:

type: string

enum:

- "Link to GET the job status"

method:

type: string

enum:

- "GET"

description: A URI reference to the status information

example:

href: "{apiRoot}/plan-management/v1/plan-validation-jobs/myjob-111/status"

title: "Link to GET the job status"

type: "application/json"

templated: true

method: GET

validationDetails:

allOf:

- $ref: '#/components/schemas/LinkObject' # Any other link will conform to LinkObject schema

- type: object

properties:

href:

type: string

default: "{apiRoot}/plan-management/v1/plan-validation-jobs/{id}/activation-details"

title:

type: string

enum:

- "Link to GET the job details"

method:

type: string

enum:

- "GET"

description: A URI reference to the status information

example:

href: "{apiRoot}/plan-management/v1/plan-validation-jobs/myjob-111/status"

title: "Link to GET the job status"

type: "application/json"

templated: true

method: GET

cancel:

allOf:

- $ref: '#/components/schemas/LinkObject' # Any other link

- type: object

properties:

href:

type: string

default: "{apiRoot}/plan-management/v1/plan-validation-jobs/{id}"

title:

type: string

enum:

- "Link to cancel the job"

method:

type: string

enum:

- "PATCH"

description: A URI reference to cancel the job

example:

href: "{apiRoot}/plan-management/v1/plan-validation-jobs/myjob-111"

title: "Link to cancel the job"

type: "application/json"

templated: true

method: PATCH

# additionalProperties to allow any other dynamic links

additionalProperties:

$ref: '#/components/schemas/LinkObject' # Any other link will conform to LinkObject schema

required:

- self

- descriptor

- status

- validationDetails

- cancel

JobState:

type: string

enum: [NOT_STARTED, QUEUED, RUNNING, CANCELLING, CANCELLED, COMPLETED, FAILED]

default: NOT_STARTED

example: "COMPLETED"

JobDetails:

properties:

message:

type: string

errors:

type: array

items:

$ref: '#/components/schemas/ErrorDetail'

additionalProperties: true

ExecutionDetails:

type: object

description: Details of the execution of the operations that are contained in the planned configuration or planned configuration group referenced in the job.

properties:

summary:

$ref: '#/components/schemas/SummaryStatus'

results:

type: array

items:

$ref: '#/components/schemas/Result'

memberConflicts:

type: array

items:

$ref: '#/components/schemas/MemberConflict'

example:

summary:

notFinished: 0

succeeded: 1

failed: 0

rollbackSucceeded: 0

rollbackFailed: 0

conflicting: 0

results: []

memberConflicts: []

ValidationState:

type: string

enum: [UNKNOWN, VALIDATION_SUCCEEDED, VALIDATION_FAILED]

example: "VALIDATION_SUCCEEDED"

ActivationState:

type: string

enum: [NOT_STARTED, ACTIVATED, ACTIVATION_FAILED, PARTIALLY_ACTIVATED, ACTIVATION_FAILED_ROLLED_BACK, ACTIVATION_FAILED_ROLLBACK_FAILED]

example: "ACTIVATED"

Result:

type: object

description: Details of the execution of the operations that are contained in the planned configuration.

properties:

planConfigDescrId:

type: string

description : If planned configuration groups are activated, this information elements specifies the planned configuration descriptor identifier, for which error details are reported. If a planned configuration is activated or validated, this information element is absent.

example: "descriptor-001"

changeId:

type: string

description: The identification of the operation. It is the identifier ("changeId") provided by the MnS consumer. Exactly one of "changeId" or "changeIndex" shall be provided.

example: "changeId-001"

changeIndex:

type: integer

description: The identification of the operation. It is the positional index of the operation in the operation set ("changeIndex"). The positional index of the leftmost element is "0". Exactly one of "changeId" or "changeIndex" shall be provided.

example: 0

state:

type: string

enum: [NOT_STARTED, PROCESSING, SUCCEEDED, FAILED]

default: NOT_STARTED

description: The state of the operation activation.

example: "SUCCEEDED"

target:

type: string

example: "/SubNetwork=1/MeContext=2/ManagedElement=3"

errors:

type: array

items:

$ref: '#/components/schemas/ErrorDetail'

required:

- target

- state

example:

changeId: changeId-002-update-config

target: /SubNetwork=1/MeContext=2/ManagedElement=3

state: FAILED

errors:

- type: VALIDATION_ERROR

title: Validation Error

reason: VALUE_INVALID

detail: The provided value 'some-value' for 'someAttr' is not allowed. Valid values are 'some-value-x', 'some-value-y'.

path: /SubNetwork=1/MeContext=2/ManagedElement=3/attributes/someAttr

errorInfo:

attributeName: someAttr

invalidValue: some-value

SummaryStatus:

type: object

properties:

notFinished:

type: integer

example: 1

succeeded:

type: integer

example: 3

failed:

type: integer

example: 3

rollbackSucceeded:

type: integer

example: 0

rollbackFailed:

type: integer

example: 0

conflicting:

type: integer

example: 0

MemberConflict:

type: object

properties:

memberConflict :

type: array

items:

$ref: '#/components/schemas/MemberOp'

MemberOp:

type: object

description: The identification of two or more operation members that have a conflict.

properties:

planConfigDescrId:

type: string

description: The plan configuration descriptor Id.

target:

description: The path of the target change resource.

type: string

required:

- planConfigDescrId

- target

oneOf:

- required:

- changeId

properties:

changeId:

type: string

description: Provided if the changeId was included in the original request.

example: "change-cell-001-1"

- required:

- changeIndex

properties:

changeIndex:

type: integer

description: Location index of the change. Reported only if changeId is NOT available.

example: 0

ConfigChangesContentType:

type: string

description: The supported configuration content types.

enum:

- YANG_BASED  # confliguration format is aligned with RFC 9751 modelling specifications with some relaxed rules

- OPENAPI_BASED  # confliguration format is aligned with an OpenAPI model definition

default: YANG_BASED

CancelRequest:

type: object

properties:

cancelRequest:

type: boolean

description: Must be set to true to initiate cancellation.

# Restricts the boolean to only allow the 'true' value

enum: [true]

required:

- cancelRequest

# This ensures no other fields (like status or priority) can be sent in this specific PATCH

additionalProperties: false

ErrorDetail:

type: object

properties:

title:

type: string

description: A short, human-readable summary of the problem type

example: "Data already exists; cannot be created"

type:

type: string

description: The type of the error

enum:

- SCHEMA_VALIDATION_ERROR

- DATA_NODE_TREE_ERROR

- MODIFICATION_NOT_ALLOWED

- ACCESS_CONTROL_CONFLICT

- APPLICATION_LAYER_ERROR

- SERVER_ERROR

- OTHER

reason:

type: string

enum:

- NEW_DATA_NODE_NAME_INVALID

- NEW_DATA_NODE_VALUE_INVALID

- NEW_DATA_NODE_CONTAINMENT_INVALID

- FINAL_DATA_NODE_VALUE_INVALID

- FINAL_DATA_NODE_UNIQUENESS_INVALID

- FINAL_DATA_NODE_MULTIPLICITY_INVALID

- FINAL_DATA_NODE_CARDINALITY_INVALID

- TARGET_DATA_NODE_NOT_FOUND

- TARGET_DATA_NODE_PARENT_NOT_FOUND

- TARGET_DATA_NODE_FOUND

- TARGET_DATA_NODE_NOT_WRITABLE

- TARGET_DATA_NODE_INVARIANT

- TARGET_DATA_NODE_CREATION_NOT_ALLOWED

- TARGET_DATA_NODE_DELETION_NOT_ALLOWED

- ACCESS_DENIED

- OTHER

example: "NEW_DATA_NODE_CONTAINMENT_INVALID"

detail:

type: string

description: A human-readable explanation specific to this occurrence of the problem.

example: "NRCellDU=1234 already exists in the network"

badDataNode:

type: string

example: "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1"

errorInfo :

type: object

description: additional error info (e.g. stackdump)

additionalProperties: true

required:

- type

- badDataNode

LinkObject:

type: object

description: Defines the structure of a single hypermedia link.

properties:

href:

type: string

format: uri-reference # Use uri-reference for relative paths (allows for absolute or relative uri)

description: The target URI of the link.

templated:

type: boolean

description: Indicates if the href is a URI Template (RFC 6570).

default: true

type:

type: string

description: The content type expected when following this link (MIME type).

title:

type: string

description: A human-readable title that describes the link's purpose.

method:

type: string

enum: [GET, POST, PUT, DELETE, PATCH]

description: The HTTP method to use for this action link.

required:

- href

SelfLink:

type: object

description: Hypermedia links for this resource, including fixed and dynamic relations.

properties:

self:

allOf:

- $ref: '#/components/schemas/LinkObject'

description: A link to the resource itself.

# additionalProperties to allow any other dynamic links

additionalProperties:

$ref: '#/components/schemas/LinkObject' # Any other link will conform to LinkObject schema

required:

- self

example: # demonstrates a typical _links object in an actual response

self:

href: "{root-url}/plan-management/v1/plan-descriptors/pd-001"

templated: true

type: "application/json"

title: "The newly created PlanConfigurationDescriptor"

help:

href: "{root-url}/help-service/v1/topics/plan-descriptors"

templated: true

type: "application/json"

title: "online help for the plan descriptor"

<CODE ENDS>

*** END OF CHANGE 1 ***