3GPP TSG-SA5 Meeting #165	S5-260073

Goa, INDIA 9 - 13 February 2026

Source:	Huawei

Title:	Pseudo-CR on TR 28.881 Update Use case #5 Invariant Guidance in Intent Contexts

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	1.0.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This contribution proposes to address the following issues in the clause 4.5 Use case #5: Invariant Guidance in Intent Contexts.

- There is no normal definition for intent handler, should be changed to intent handling function.

- contextInvariant is configured by MnS consumer, however, the property “isWritable” is False.

- The default value for attribute contextInvariant is missing.

Proposed Changes

* * * First Change * * *

4.5	Use case #5: Invariant Guidance in Intent Contexts

4.5.1	Description

3GPP TS 28.312 [1] describes a scenario where for an instantiated intent, the intent handling function may need to decompose the intent into multiple derivative intents, each to be fulfilled by a separate other intent handling function. There may be contexts in the intent which the MnS consumer desires that the other intent handling functions understand them exactly as they were provided, i.e., without any modification by the first intent handling function in the decomposition process.

As an example, the MNO may, for some decomposition use-cases, want to only use certain certified hardware (e.g., with certain security, privacy, energy consumption, other types of quality guarantees) to be used in the fulfilment of RAN or CN operations. This additional guidance to use this specified hardware or the quality constraints on the resources may be provided as part of the intent context.

The MnS consumer should be enabled to indicate the context which should be transmitted to other intent handling functions without modification.

4.5.2	Potential requirements

REQ-Intent_InvarGui-1: The intent driven MnS should include a capability enabling the MnS consumer to indicate the requirements, goals and contexts which the MnS consumer recommends to be copied into decomposed intents and transmitted to other intent handling functions without modification.

4.5.3	Potential solutions

The "context" dataType, as defined in 3GPP TS 28.312 [1], can be extended to include information about invariant guidance.

-	Add a description for invariant guidance e.g. as: The Context can be used to indicate invariant guidance, e.g., which context is recommended to be propagated to all intent handling functions in the case where the intent is decomposed into multiple sub intents. The attribute contextInvariant is a flag to indicate if a specific context is recommended to be considered as in invariant guidance or not. The flag applies to context at the expectation level and intent target level.

-	The following attribute would be added to the Context <<dataType>>, i.e.:

Addition to Table 6.2.1.3.4.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| contextInvariant | O | T | T | T | T |

The new attribute can be defined as:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| contextInvariant | It indicates whether the specific context recommended to be considered as invariant guidance to be transferred to subsequent intent handling functions without any modification. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |

* * * End of Changes * * * *