3GPP TSG-SA5 Meeting #165	S5-260099

Goa, India 09 – 15 February  2026

Source:	Nokia

Title:	Pseudo-CR on Conclusion on Intent Interpretation Assistance

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR28.881

Version:	1.0.0

Work Item:	Intent driven management services for mobile network phase 4

Comments

This pCR is to conclude  the use case#6 (Section 4.6) in TR28.881 on Intent Interpretation Assistance Information.

Proposed Changes

* * * First Change * * * *

## 4.6	Use case #6: Intent Interpretation Assistance Information

### 4.6.1	Description

A similar intent can be instantiated on several intent handlers, e.g., on several management systems each responsible for a for a different domain. 3GPP TS 28.312 [1] supports intent negotiation that enables interaction between the MnS producer and MnS consumer to clarify the ability to fulfil intent requirements that may otherwise not be realizable owing to their abstract nature. For the same intents that are sent to several MnS producers, negotiation with the several MnS producers to clarify the intents can be resource wasteful.

The intent driven management service can minimize the number of negotiation exchanges while allowing effective interpretation of intents by reusing the interpretation information. The MnS consumer compiles prior interactions, e.g. previous IntentNegotiationReports or intentNegotiationConsumerFeedback, and shares those prior interactions as intent interpretation assistance information with the handler. The Mns consumer should be enabled to provide this intent interpretation assistance information that can be used by the intent handler to support the interpretation of the intent. As an example, if during intent negotiation, a set of options were provided by MnS producer and the MnS consumer selects one option, the MnS consumer may want the MnS producer to reuse this information, so the MnS consumer can provide it to the MnS producer for reuse.

NOTE:	The intent interpretation assistance information can be visualized as a "frequently asked questions (FAQ)" section used in many human-oriented service descriptions (e.g. on service providers' websites).

### 4.6.2	Potential requirements

REQ-Intent_Interpretation_Assistance_1: The intent driven MnS producer should have the capability enabling the MnS consumer to provide information containing prior intent negotiations to the handler that can then be used by the intent handler to support the interpretation of the intent.

### 4.6.3	Potential solutions

-	Add to the Intent IOC a description of the information about intent interpretation e.g., as:

a)	The intent IOC includes intentInterpretationAssistanceInfo which contains prior negotiation interactions from the MnS consumer that are provided to the intent handling function and can be used by the intent handling function to support the interpretation of the intent. The intentInterpretationAssistanceInfo is a pair that captures the previous MnS producer questions, requests or reports and the corresponding consumer responses. It may include following information related to a timestamp, applicable intent context, and the previous MnS producer to MnS consumer interactions.

1)	For example, the pair may include a key that is "the set of previous intent fulfilment options that were provided by MnS producer" as well as the corresponding value which is "the option that was selected by the MnS consumer".

-	Introduce a data type and a corresponding attribute on the Intent IOC for intent interpretation assistance information say named intentInterpretationAssistanceInfo. The attribute is readable and writable.

-	The intentInterpretationAssistanceInfo is a pair that captures the previous questions, requests or reports and the corresponding consumer responses. Examples can include the following attributes/ information:

a)	A timestamp.

b)	Conditions: the conditions under which the intent was previously sent.

c)	MnS Producer report: the request sent by the MnS producer.

d)	MnS Consumer response: the responses provided by the MnS consumer for the specific MnS producer report.

example of the exchanged information may be a combination of 1) an MnS Producer report containing the same consumer's previous expectation target and the achieved outcomes together with 2) MnS Consumer response that contains the corresponding MnS consumer satisfaction index.

### 4.6.4	Evaluation of potential solutions

Only one potential solution provided in clause 4.6.3 is identified. This potential solution proposes enhancing the existing intent IOC by adding a datatype that captures the previous MnS-producer to MnS consumer interactions. The implementation of this potential solution is not complex as it proposes enhancements to existing intent IOC and reuses existing attributes.

Therefore, the potential solution described in clause 4.6.3 is a feasible solution to satisfy the requirements in clause 4.6.2.

* * * Next Change * * * *

# 5	Conclusions and Recommendations

## 5.6	Use case #6: Intent Interpretation Assistance Information

The use case description, requirements and a potential solution for Intent Interpretation Assistance Information are described in clause 4.6. This use case enables an MnS consumer to provide information containing prior interactions that can then be used by the intent handler to support the interpretation of the intent.

The potential solution described in clause 4.6.3, which proposes enhancing the existing intent IOC with adding a new intent interpretation assistance info can be used as baseline for normative work.

* * * End of Changes * * * *