3GPP TSG SA5 Meeting #165	S5-260116

Goa, India, 9-13 February 2026

Source:	ZTE Corporation

Title:	Pseudo-CR TR 28.882 Add Solution for Management Support to Training for UE-side Model

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	0.2.0

Work Item:	FS_AIML_MGT_Ph3

Comments

In SP-251687, SA has reached the following agreement:
“There shall not be any further work on AI/ML UE sided data collection for 5G-A Rel-20.”

This contribution proposes to add different option of solution on UE sided data collection to close the discussion for AI/ML UE sided data collection in SA5 5GA Rel-20.

Proposed Changes

* * * First Change * * * *

5.1	ML model training

5.1.1	Use cases

5.1.1.1	Management support to training for UE-side model

5.1.1.1.1	Management support to AI/ML-based beam management

5.1.1.1.1.1	Description

To support AI/ML-based beam management defined in TS 38.300 [2], for beam prediction management, UE can send the data of beam prediction management to UE-side training entity (e.g. a server inside MNO or an OTT server) via gNB and 3GPP management system for UE-side model training (see NOTE 1). 3GPP management system needs to collect data from gNB and report it to UE-side training entity to management support of training for UE-side model.

Figure 5.1.1.1.1.1-1: Management of UE-side data collection and reporting for UE-side model training

Being a MnS consumer, the UE-side training entity requests 3GPP management system to collect the UE-side data of beam prediction management, then 3GPP management system configure the gNB(s) to collects data from specified UEs, and gNB shall send configuration information to the UE(s). The request from UE-side training entity shall include the type of data (e.g. based on UE/UE Group, location, time) to be produced and the gNB(s) where the data shall be collected.

After the UE-side training data such as beam prediction data are produced, UE(s) shall report the UE-side data to gNB(s), and 3GPP management system shall collect data from the gNB(s). 3GPP management system report the collected data to UE-side training entity depending on access information specified by operator (see NOTE 2).

NOTE 1:	The UE-side data are subject to further discussion, pending ongoing correspondence and confirmation by RAN2 and RAN1.

NOTE 2:	Access control information is for further discussion.

5.1.1.1.1.2	Potential requirements

REQ-ML_UESIDE-01: The 3GPP management system should have a capability allowing the authorized UE-side training entity to request the UE-side training data for the UE-side model training.

REQ-ML_UESIDE-02: The 3GPP management system should have a capability to request and get the UE-side training data from gNB(s) for the UE-side model training.

REQ-ML_UESIDE-03: The 3GPP management system should have a capability to report the UE-side data to authorized UE-side training entity for UE-side model training.

5.1.1.1.1.3	Possible solutions

Option 1: Solution described in high level

Following are the potential management capabilities which can be further enhanced for UE-side data collection:

-	For the management of UE-side data collection, it can be achieved by enhance the ManagementDataCollection IOC in clause 4.3.47 in TS 28.622 and TraceJob IOC in clause 4.3.30 in TS 28.622[x2], i.e., the UE-side data.

Note: The content of UE-side data depends on RAN1 & RAN 2, and is out the scope of SA5.

-	For the management of UE-side data reporting to UE training entity, the existing Streaming or File based data reporting service can be reused.

Option 2: No Solution is identified in Rel-20 5GA study.

5.1.1.1.1.4	Possible solutions evaluation

Option 1:

The possible solution summarizes current management capability that can be further enhanced for UE-side data collection. However, according to the guideline from SA that no further work is planned in Rel-20 5GA, details are not investigated.

Option 2:

No Solution is identified.

* * * Next Change * * * *

9	Conclusion and recommendations

9.1	ML model training

For management support to training for UE-side model, according to the SA decision in SP-251687, no normative work is planned in 5GA Rel-20.