3GPP TSG-SA5 Meeting #165	S5-260129

Goa, India, 09 - 13 February 2026

Source:		China Mobile

Title:	Pseudo-CR on TR 32.801 Add cloud native management scenario for 6G resiliency

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

Based on the clause 5.6.3 in TR 22.870, this contribution proposes to add cloud native Management scenario for 6G resiliency.

Proposed Changes

* * * First Change * * * *

2	References

[X] 	TR 22.870 Study on 6G Use Cases and Service Requirements.

* * * Next Change * * * *

6	6G Management Scenarios

6.2 Management scenarios for support of 6G services

6.2.1 System and Operation Aspects

6.2.1.X Management Scenario #<X>: <cloud native for 6G resiliency>

6.2.1.X.1 Description

To achieve objective of resiliency use case  for 6G in the clause 5.6.3 of [X] TR 22.870,  the 3GPP mangement system should follow cloud nativeness principle to design and achieve management capabilities. The usecase assumes that 6G capabilities are be designed as a collection of workloads on top of the cloud native infrastructure, and cloud native infrastructure and applications are decoupled from the lifecycles of vertical monoliths.

The 3GPP management system can collect real-time resource status information from the cloud-native infrastructure , and combined with AIML capability to realize prediction and detection of network anomalies.

In the various disruptions scenarios, such as physical damage, cyber-attacks, and natural disasters, the 3GPP management system supports elastic management of cloud-native workloads, such as implementing dynamic scheduling of  6G NF workloads and infrastructure resources, activating redundant workloads, rerouting service traffic, and automatically ensuring service continuity, then improving network resilience and O&M efficiency.

This scenario focuses on cloud-native management capabilities within the scope of SA5, and achieves synergy with SA2's architectural design and SA1's use case requirements.

6.2.1.1.2 Potential Requirements

[PR 6.2.1.X-1]  The 6G 3GPP management system should be able to interact with cloud-native infrastructure to collect resource status information.

[PR 6.2.1.X-2]  The 6G 3GPP management system should be able to dynamically reallocates workloads to maintain service continuity and performance.

[PR 6.2.1.X-3]  The 6G 3GPP management system should be able to adapt the decoupling characteristics of cloud-nativeness, and support elastic scaling and independent upgrading of the OAM functions themselves.

* * * End of Changes * * * *