3GPP TSG-SA5 Meeting #165	S5-260155

Goa, India, 09 - 13 February 2026

Source:	ZTE Corporation

Title:	Pseudo-CR on TR 32.801-01 Add New Management Scenario on 6G Data Management Framework in the Management System

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution proposes to add New Management Scenario on 6G Data Management Framework in the Management System.

Proposed Changes

* * * First Change * * * *

6.1 New 6G management scenarios

Editor's note 1: This clause will contain new 6G management scenarios and the corresponding requirements.

6.1.y Data Management Framework

6.1.y.2 Management Scenario #2: 6G Data Management Framework in the Management System

6.1.y.2.1 Description

In 5G and 5G‑Advanced, the 3GPP management system provides mechanisms for the control and reporting of various types of management data, including configuration, performance, fault, trace/MDT, QoE, and signalling traffic data. Each type of management data is collected and controlled through its corresponding management job or instance. Here is a brief summary of the management data:

- Configuration data. The MnS Consumer can use generic provisioning management service (getMOIAttributes operation) to obtain the managed information represented by an instance of an IOC defined in NRMs, e.g. Generic NRM, NR and NG-RAN NRM, or 5GC NRM as defined in TS 28.532.

- Performance data. The MnS Consumer can request PM Producer to create a PerfMetricJob instance to collect specified performance metrics as defined in TS 28.622.

- Fault data. The MnS Consumer can receive fault relevant data from alarm notification as defined in TS 28.111.

- Trace/MDT data. The MnS Consumer can request Trace/MDT Producer to create a TraceJob instance to collect Trace/MDT data as defined in TS 32.422.

- QoE data. The MnS Consumer can request QMCJob to create a QMCJob instance to collect QoE data as defined in TS 28.405.

- Signaling traffic data. The MnS Consumer can request STM Data Producer to create a StmCtrl instance to collect signaling traffic data as defined in TS 28.560.

For reporting, both streaming‑based and file‑based mechanisms are supported in the 3GPP management system as specified in TS 28.532.

However, these capabilities result in a producer‑centric management paradigm, where each MnS producer independently collects, stores, processes and exposes its own data. Consequently, data pipelines remain fragmented, and unified lifecycle management across producers is not supported. This fragmentation limits the operator’s ability to perform deep analytics. For example, an operator may require correlating gNB energy consumption measurements with the QoE experienced by served UEs to support energy‑saving decisions, or correlating historical configuration data with abnormal events for network optimization purposes. Such multi‑source operations are difficult to achieve under the current data management architecture.

To meet the data‑intensive and AI‑driven operational needs of 6G networks, a standardized Data Management Framework (DMF) is required. The DMF provides unified and policy‑governed lifecycle management of management data, covering data collection, processing, storage, and exposure. By enabling harmonized data handling across producers, the DMF supports breaking data silos, ensuring data sovereignty and secure sharing, and supplying trustworthy and consistently described data and datasets for intelligence‑enabled network functions. The key management capabilities to be supported by the DMF include:

- Data Collection: To provide operators with timely and reliable management data for real‑time analytics and AI‑driven functions, the DMF collects data from heterogeneous producers using unified streaming‑based and file‑based mechanisms, and supports QUIC‑based transport to efficiently deliver large‑volume and latency‑sensitive data.

- Data Processing: To ensure that management data can be directly consumed by automation and intelligence functions, the DMF processes collected data into standardized, coherent, and machine‑ready forms through operations such as data labelling, dataset creation, and other preparation steps required for analytics and automation usage.

- Data Storage: unified storage management for processed data, supporting:

  - Data Registration. To provide operators with a unified view of available management data, it enables the registration and cataloguing of management data, including collection granularity, reporting mechanism, and data format (e.g., JSON, CSV), forming a comprehensive directory for subsequent discovery and use.

  - Data Retrieval. To support efficient and reusable access to management data, it provides discovery and query capabilities, enabling consumers to efficiently identify and obtain required data.

- Data Exposure: To supply authorized consumers with validated management data in the required timeliness and format, the DMF exposes data through unified file‑based and streaming‑based mechanisms, including Kafka‑based delivery for high‑throughput, low‑latency real‑time operational needs.

By providing these unified data lifecycle management capabilities, the DMF addresses the limitations of the existing producer-centric paradigm and supports the needs of advanced, AI-enabled network operations in 6G.

6.1.y.2.2 Potential Requirements

REQ‑DMF‑1: The DMF within the management system shall enable standardized lifecycle management of management data.

REQ‑DMF‑2: The DMF within the management system shall provide standardized mechanisms for the collection, processing, storage, and exposure of management data.

REQ‑DMF‑3: The DMF within the management system should support data collection by stream‑based approach based on QUIC for real-time data collection requirements.

REQ‑DMF‑4: The DMF within the management system shall support data processing including data labelling and dataset creation for model training purpose.

REQ‑DMF‑5: The DMF within the management system shall support unified data storage including data registration and data retrieval.

REQ‑DMF‑6: The DMF within the management system should support data exposure by stream‑based approach based on Kafka for real-time data exposure requirements.

* * * End of Changes * * * *