3GPP TSG-SA5 Meeting #165	S5-260172

Goa, India, 9 – 13 February 2026

Source:	Rakuten Mobile Inc.

Title:	pCR on TR 28.885 Use case for ES policy

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.3.0

Work Item:	FS_Energy_Ph4_OAM

Comments

As per SID for FS_Energy_Ph4_OAM, WT-3 is expected to study use case(s), requirement(s) and solution(s) to improve energy saving, energy efficiency and reducing carbon footprint of 5G network. The contribution below proposes the introduction of Network Energy Savings policy in NRM for WT-3.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or nonspecific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[4]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[5]	ETSI GS OEU 020 (v1.1.1): "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[6]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

[7] 	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[8]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[9]	3GPP TR 23.700-67: "Study on Energy Efficiency and Energy Saving; Phase 2".

[10]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[11]	3GPP TS 32.130: " Telecommunication management; Network sharing; Concepts and requirements".

[y]	3GPP TS 38.300: "NR and NG-RAN Overall description; Stage-2".

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

EC	Energy Consumption

EE	Energy Efficiency

EIF	Energy Information Function

S-NSSAI	Single Network Slice Selection Assistance Information

MNO				Mobile Network Operator

* * * Next Change * * * *

## 5.3	Improve energy saving, energy efficiency and reducing carbon footprint of 5G network

### 5.3.X	Use case #<X>: NES Policy framework use case

#### 5.3.X.1	Description

The objective of this use case is to enable MnS Consumers to configure the NES policies to MnS Producers associated with NG-RAN and/or core network functions or NE(s) by utilizing the existing OAM Provisioning framework and NRMs.  The RAN energy-saving features are specified for NG-RAN functions in 3GPP TS 38.300 [y] (clause 15.4). Potential new energy saving use cases could be introduced to be supported by NG-RAN functions and core networks. These features can be implemented via NR NRM.

Currently, the policies in NR NRM framework include CESManagementFunction and DESManagementFunction, as specified in 3GPP TS 28.310 [3], clause 6. These are based on requirements defined in 3GPP TS 38.300 [y], clause 15.4.3.  The corresponding attributes for these NRM functions are specified in 3GPP TS 28.541[7]. However, some attributes exhibit policy like characteristics. For example, attributes such as intraRatEsActivationCandidateCellsLoadParameters and intraRatEsDeactivationCandidateCellsLoadParameters exhibit traits like policies. However, for new NES features introduced in Release 17, 18, 19, and onwards, MNOs currently lack a standardized management mechanism to convey policies that influence their behaviour and performance. This absence of a unified NES policy framework prevents MNOs from dynamically adjusting these individual features based on real-time network conditions, leading to increased operational complexity and hindering the efficient deployment and optimization of advanced energy-saving strategies.

To address the above challenges a unified NES policy framework is essential to enable MNO to efficiently deploy and manage energy saving features that are introduced in Release 17, 18, 19, and onwards, while ensuring streamlined and scalable management of both current and future energy-saving capabilities.

The advantages of this use case are given below.

Autonomous NES Decision-making: While cell-level ES is addressed, a consolidated policy framework for all NES features is currently lacking. Hence, there is a critical need to extend NRM framework to specify a consolidated NES policy framework to enable the autonomous NG-RAN and core network operation for energy saving. Here the NES policy would be provided to NG-RAN and/or core network functions, enabling autonomous operation for various NES features.

NG-RAN and core network functions can accept and utilize the MNO-defined policies to make autonomous decisions regarding NES features, provided they do not violate overarching network policies.

#### 5.3.X.2	Potential requirements

PREQ-NES_Policy-1: The 3GPP management system should enable an authorized consumer to retrieve NES policy related capability information of each NES feature of the Network Element/Network Function.

PREQ-NES_Policy-2: The 3GPP managed system should enable an authorized consumer to provide NES policy(ies) to MnS Producer for one or more NES features.

PREQ-NES_Policy-3: The 3GPP managed system should be able to enable an authorized consumer to provide conditions and thresholds for the policy to be enforced.

#### 5.3.X.3	Potential solutions

##### 5.3.X.3.i              Potential solution #1: Unified NES Policy framework

###### 5.3.X.3.i.1	Introduction

This potential solution defines the unified NES policy framework for potential new NES features that are supported by a Network Function/Network Element. This policy can be extensible, allowing for the addition of new attributes when new NES features are defined. MnS consumers may configure this policy using the existing OAM Provisioning framework, ensuring consistency and avoiding fragmentation.

###### 5.3.X.3.i.2	Description

The high-level unified NES policy framework solution is illustrated in Figure 5.3.X.3.i.2-1.

Figure 5.3.X.3.i.2-1 – Unified NES policy framework for potential new NES features

- NF provisioning MnS Consumer receives counters and KPIs from other sources

- The NF Provisioning MnS Consumer analyses the PM counters and KPIs received and prepares a NES policy for the NF Provisioning MnS Producer of NF. Here the NF Provisioning MnS Consumer would prepare the NES policy at the granularity of cell or NF.

- The NF Provisioning MnS Consumer uses the createMOI operation to configure the NES policy and associated attributes to NF Provisioning MnS Producer.

For example, the policy can comprise attributes that dictate critical parameters for NES feature operation, including:

| Attribute name | Description |
| --- | --- |
| Conditions | Specific network or environmental conditions that trigger or influence NES operations. |
| Thresholds | Conditions under which NES features should be activated or deactivated. |
| Targets | Desired energy saving outcomes or performance levels. |
| Applicable time period | The time window or period for which a NES policy is applicable or valid. |
| Additional Information/vendor extension | Any additional parameters necessary for NG-RAN functions to autonomously handle various NES features effectively. Some of these can be vendor specific extensions. |

- The MnS Producer associated with the NF can analyse the above NES policy and take necessary actions such preparing for the activation and deactivation of one or more NES features in NF.

- If the conditions mentioned in the NES Policy are satisfied in the applicable time window or period, the NF Provisioning MnS Producer can perform the following

5a.	The MnS Producer associated with the NF provisions the NES features that is to be handled by that NF depending on various NES scenario. For example, NF activates or deactivates the NES feature(s) upon the conditions mentioned in the NES Policy is satisfied.

5b. The NF Provisioning MnS Producer sends notifyMOIChanges to NF Provisioning MnS Consumer to indicate the NES Policy provided by the MnS Consumer in Step 3 has been enforced.

- If the conditions mentioned in the NES Policy are not satisfied at the applicable time period, the NF Provisioning MnS Producer can perform the following

  - 6a. The NF Provisioning MnS Producer uses the notifyMOIChanges to indicate to the NF Provisioning MnS Consumer that the provisioned policy was not enforced during the applicable time window indicated in the NES policy. This is to enable MnS Consumer to be aware that the NES policy that was prepared by it is not suitable to be applied appropriately in the current network condition.

  - 6b. After receiving the notification sent in Step 6a, the MnS Consumer monitors and analyses the latest network conditions. This can result in MnS consumer preparing a new or updated NES policy.

6c. The NF Provisioning MnS Consumer uses the modifyMOIAttributes to send updated NES Policy to NF Provisioning MnS Producer. Once the conditions mentioned in the updated policy are satisfied then MnS Consumer and MnS Producer can follow the Steps 5a and 5b.

The above unified NES policy framework will facilitate to configure appropriate NES policies based on specific network deployments, traffic patterns, and operational conditions. A fundamental principle of this NES policy framework is to ensure that the implementation of NES features does not compromise user service quality. The policy attributes and NG-RAN and/or core network functions behaviour will be designed to prioritize maintaining agreed-upon service levels.

The NG-RAN and/or core network may examine the policy conditions and target configurations and may perform the necessary actions. For example, NG-RAN function may perform the following based on the OAM provided policy:

- Activate or deactivate cells and associated carriers.

- Adjust Cell DTX/DRX configurations

- Adjust traffic scheduling patterns to reduce radio equipment power consumption

- Dynamically adjust transmission through spatial and power domain adaptation.

- Utilize SSBLess SCell functionality.

The NG-RAN function activates or deactivates the specific energy-saving feature or a set of energy saving features, when the conditions mentioned in the MnS Consumer's policy are met.

* * * End of Changes * * * *