3GPP TSG-SA5 Meeting #165	S5-260173

Goa, India, 9 – 13 February 2026

Source:	Rakuten Mobile Inc.

Title:	Pseudo-CR on TR 28.884 Improvements to message bus solution

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	0.2.0

Work Item:	FS_SBMA_Ph4

Comments

To address WT-1 of Study on Service Based Management Architecture enhancement phase 4, this contribution proposes to improve the management data streaming solution based on message bus.

Proposed Changes

Address remaining editor’s notes by adding relevant procedure

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.532: " Management and orchestration; Generic management services".

[4]	3GPP TS 28.537: " Management and orchestration; Management capabilities".

[5]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: " Management and orchestration; 5G end to end Key Performance Indicators (KPIs)".

[7]	3GPP TS 32.423: " Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

[8]	https://datatracker.ietf.org/doc/html/rfc6455

[9]	https://websocket.org/guides/websocket-protocol/

[10]	3GPP TS 28.111: "Management and orchestration; Fault management (FM)".

[11]	3GPP TS 32.531: "Telecommunication management; Software management (SwM); Concepts and Integration Reference Point (IRP) Requirements".

[12]	3GPP TS 32.532: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Information Service (IS)".

[13]	3GPP TS 32.533: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)".

[14]	3GPP TS 28.631: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[15]	3GPP TS 28.632: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 28.633: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[17]	SP-250863: Study on SBMA enhancement phase 4

[18]	3GPP TS 32.158: " Design rules for REpresentational State Transfer (REST) Solution Sets (SS)"

[19]	RFC 6241 Network Configuration Protocol (NETCONF)

[20]	3GPP TS 32.101: "Telecommunication management;Principles and high level requirements".

[x]	3GPP TS 28.622: “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

* * * Next Change * * * *

5.2.3.1	Solution #1: Multipoint-to-Multipoint management data streaming based on message broking MnF

This potential solution proposes introduction of a new MnF implementing the message broker(s) that mediates data with the help of new MnS services, namely, message bus data reporting MnS and message bus data retrieval MnS (for data subscribers) to enable multipoint-to-multipoint streaming data reporting. This solution is meant for use at the management level and not at the managedElement/NF level. This solution also does not impact existing MnS services and can coexist beside such services as an additional option.

Figure 5.2.3.1-1: Potential solution for multipoint-to-multipoint management data streaming based on message bus

As shown in Figure 5.2.3.1-1, in this solution, Message bus data reporting MnS producers send management data to Message broking MnF using the message bus data reporting service based on message bus publishing protocol. Message bus data retrieval MnS consumers receive the management data using the message bus data retrieval service based on message bus retrieval protocol. This approach allows the same copy of management data produced by the Message bus data reporting MnS producer(s) to be shared across multiple Message bus data retrieval MnS consumers thus enabling multipoint-to-multipoint communication and a publishing-and-subscribing based communication pattern.

Editor’s note: The following aspects are FFS

- What specific message bus-based communication protocol to be supported (e.g. Kafka) and why?

The high-level procedures for enabling data flowing from Message bus data reporting MnS producers to Message bus data retrieval MnS consumers via the Message broking MnF for performance metric data streaming is illustrated in Figure 5.2.3.1-2 below. 

Note: The message bus-based management data streaming method applies to performance metric data only.

Figure 5.2.3.1-2: Procedures for establishing management data streaming based on message bus

- Message bus data retrieval MnS consumer creates a new PerfMetricJob MOI by invoking the createMOI operation to the Message bus data retrieval MnS producer including information about performance metrics requested (performanceMetrics, TS 28.622[14]), scope (objectInstances, TS 28.622[14]),) and data producing period (granularityPeriod, TS 28.622[14]),) etc.

- The Message bus data retrieval MnS producer responds back a createMOI response which contains information about the message bus topic names and message broker endpoints (reportingCtrl.topicNames and reportingCtrl.brokerEndpoints, TS 28.622[14]),).The Message bus data retrieval MnS consumer then establishes data subscription from the message broker.

- Message bus data reporting MnS consumer creates a new PerfMetricJob MOI by invoking the createMOI operation to the Message bus data reporting MnS producer including information about performance metrics requested (performanceMetrics), scope (objectInstances) and data producing period (granularityPeriod) etc. In addition, information about the topic names and broker endpoints (reportingCtrl.topicNames and reportingCtrl.brokerEndpoints) for the Message bus data reporting MnS producer to establish data communication to the message broker, is conveyed.

- Message bus data reporting MnS producer sends back a createMOI PerfMetricJob response indicating the acceptance of the data production request.

- Message bus data retrieval MnS consumer then subscribes to the topics using the topic names and message broker endpoints provided by the Message bus data retrieval MnS producer in step 2, using message bus implementation specific subscription protocol.

- Message bus data reporting MnS producer reports data to the message broker using message bus implementation specific data publishing protocol

- Message bus data retrieval MnS consumer receives the data from the message broker using message bus implementation specific data retrieval protocol.

Note: The message bus data subscription, publishing and retrieval protocols are implementation specific allowing any industry solution to be used, e.g. Kafka.

* * * End of Changes * * * *