3GPP TSG SA5 Meeting #165	S5-260201

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	pCR TR 28.884 Solution for software management

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	0.2.0

Work Item:	FS_SBMA_Ph4

Comments

This contribution proposes the detailed solution for software management which focuses on the NRM modelling of software for PNF.

Proposed Changes

* * * First Change * * * *

### 5.4.3	Potential solutions

#### 5.4.3.x	Potential solutions for PNF software management

#### 5.4.3.x.1	NRMs for PNF software management

New IOCs SwPnfPackage, SwPnfJob, SwPnfProcess are introduced to satisfy the requirements given in clause 5.4.2. Relationship and inheritance diagrams are captured in 5.4.3.x -1 and 5.4.3.x -2 respectively.

Figure 5.4.3.x.1 -1: Relationship UML diagram for software management

Figure 5.4.3.x.1 -2: Inheritance UML diagram for software management

The SwPnfPackage IOC includes attributes inherited from Top IOC and the following attributes in Table 5.4.3.x-1.

The attribute swPackageName is a human readable name of a software package.

The attribute swPackageVersion is a version identifier of the software package.

The attribute swPackageType represents the type of software. It is a string. The examples of software type could be COLD_PATCH, HOT_PATCH, PLUGIN_PACKAGE, etc.

The attribute swPackageSize represents the size of software. It is an integer. The unit is KB.

The attribute swPackageLocation represents the location of software. It is a string of the software file path on a server.

The attribute IntegrityCheck is a hash value which is used for software package integrity verification. It helps to ensure the downloaded software package is not corrupted during the transmission or storage.

The attribute vendorName is a name of inventory unit vendor (or vendors may provide manufacturer name)

Table 5.4.3.x.1-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| swPackageName | M | T | T | T | T |
| swPackageVersion | M | T | T | T | T |
| swPackageType | O | T | T | T | T |
| swPackageSize | O | T | T | T | T |
| swPackageLocation | M | T | T | T | T |
| IntegrityCheck | O | T | T | T | T |
| vendorName | O | T | T | T | T |

The SwPnfJob IOC represents MnS consumer's requirements for PNF software management. It includes attributes inherited from Top IOC and the following attributes in Table 5.4.3.x-2.

The attribute nEInformation specifies the RAN NEs or RAN NE types for which this SwPnfJob instance is valid. For a RAN NE starting a software management process, there shall be no ambiguity which SwPnfJob is valid for the RAN NE. Therefore, the attribute nEInformation of different SwPnfJob instances shall not intersect.

The attribute swJobId is a unique identifier of a software management job.

The attribute swJobName is a human readable name of a software management job.

The attribute swJobType represents the actions for which this software management job asks. Its type is string. Examples of allowed value could be UPGRADE, FALLBACK, etc.

The attribute swJobStartTime represents the start time of a software management job.

The attribute swJobStartTime represents the end time of a software management job.

Table 5.4.3.x.1-2

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nEInformation | M | T | T | T | T |
| swJobId | M | T | T | T | T |
| swJobName | M | T | T | T | T |
| swJobType | M | T | T | T | T |
| swJobStartTime | O | T | T | T | T |
| swJobStopTime | O | T | T | T | T |
| Attribute related roles |  |  |  |  |  |
| swPnfPackageListRef | M | T | T | T | T |

The SwPnfProcess IOC represents the software management process for a RAN NE, which allows the MnS consumer to configure and be informed about the current situation of the software management process. It includes attributes inherited from Top IOC and the following attributes in Table 5.4.3.x-3.

The SwProcessMonitor attribute represents the status of software management process for a single RAN NE and includes information the MnS consumer can use to monitor the progress and result of the software management process. The data type of this attribute is ProcessMonitor in TS 28.622 [x]. The following specializations are provided for this data type for the software management process:

-	The value of attribute status are "NOT_STARTED", "RUNNING", "CANCELLING","FINISHED", "SUSPENDED","FAILED" and "CANCELLED". The values "PARTIALLY_FAILED" is not used.

-	The attribute progessPercentage indicates progress of the process as percentage.

-	When the attribute status is equal to "RUNNING" the attribute progressStateInfo attribute shall indicate one of the following states: "PRECHECK_BEFORE_DOWNLOAD", "SW_DOWNLOAD", "PRECHECK_BEFORE_ACTIVATION", "SW_ACTIVATION". Vendor specific information may be provided though.

-	For the case that the attribute status is equal to "FAILED" the attribute resultStateInfo shall indicate one of the following failure reasons: "UNKNOWN", " NE_HEALTH_CHECK_FAILED", "SW_DOWNLOAD_FAILED", " SW_ACTIVATION_FAILED", "DISCONNECTION_BETWEEN_NE_AND_OAM", "RESPONSE_TIMEOUT_BETWEEN_NE_AND_OAM","FTPS_CONNECTTION_FAILED_BETWEEN_NE_AND_OAM". Vendor specific information may be provided though.

The attribute stopPointSetIndication represents a True or False indication on whether this software management process instance will stop when the progressStateInfo attribute changes its value. If the value of stopPointSetIndication is True, when the progressStateInfo attribute changes its value, e.g., from "SW_DOWNLOAD" to "PRECHECK_BEFORE_ACTIVATION", this software management process will automatically stop and the value of stopPointStatus becomes "STOPPED". In this case only when the MnS consumer sets the value of stopPointStatus becomes "CONTINUED", this software management process will continue. If the value of stopPointSetIndication is False, this software management process will not automatically stop.

The attribute stopPointStatus represents whether the software management process is stopped or continued. It is applicable and mandatory when the value of stopPointSetIndication is True.

MnS consumer can terminate a software management process which is currently ongoing (the value of the attribute status is "RUNING" and the attribute progressStateInfo is not "SW_ACTIVATION") by configuring attribute cancelScProcess to "True". Then the attribute status will change from "RUNGING" to "CANCELLED" when MnS producer have terminated the software management process as MnS consumer requested.

Table 5.4.3.x.1-3

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nEId | M | T | F | T | T |
| swProgressMonitor | M | T | F | T | T |
| stopPointSetIndication | O | T | T | T | T |
| stopPointStatus | CM | T | T | T | T |
| cancelSwProgress | O | T | T | T | T |
| Attribute related roles |  |  |  |  |  |
| swPnfJobRef | M | T | F | T | T |

#### 5.4.3.x.2	Procedures for PNF software management

Figure 5.4.3.x.2 -1: Procedures for software management

1. MnS consumer sends a request to create a SwPnfPackage instance (see createMOI operation defined in TS 28.532 [3]) to MnS producer with software package information.

2. Based on the received request, the MnS producer creates the concrete SwPnfPackage instance (i.e. instance of SwPnfPackage).

3. MnS Producer sends a response (see createMOI operation defined in TS 28.532 [3]) to the MnS Consumer with attribute "objectInstance" of the created SwPnfPackage instance.

4. MnS consumer sends a request to create a SwPnfJob instance (see createMOI operation defined in TS 28.532 [3]) to MnS producer with software package information.

5. Based on the received request, the MnS producer creates the concrete SwPnfJob instance (i.e. instance of SwPnfJob) for NE(s) or NE type(s) specified by attribute "neInformation".

6. MnS Producer sends a response (see createMOI operation defined in TS 28.532 [3]) to the MnS Consumer with attribute "objectInstance" of the created SwPnfJob instance.

7. For each NE (specified in the created SwPnfJob) starting its software management process, MnS producer creates an SwPnfProcess instance for the started software management process and configure the created SwPnfProcess instance with DN of the associated SwPnfJob.

8. MnS producer notifies (see notifyMOICreation notification defined in TS 28.532 [3]) the MnS consumer about the creation of a new SwPnfProcess instance, including DN of SwPnfProcess instance.

9. In the case that MnS consumer manually controls the software management process, the MnS consumer requests to configure attribute stopPointSetIndication of the SwPnfProcess instance with value "TRUE", which means when the attribute progressStateInfo changes, the process will stop and wait for MnS consumer’s command to continue.

10. When the value of attribute progressStateInfo changes, MnS producer stops handling the software management process.

11. MnS producer notifies the MnS consumer about the value change of the SwPnfProcess instance.

12. MnS consumer requests to configure attribute stopPointStatus of the SwPnfProcess instance with value "CONTUNUED" to continue the process.

The following step 13 describes the procedures for MnS consumer to monitor software management progress and result. These steps can happen any time after the SwPnfProcess instance is created until the SwPnfProcess instance is deleted

13a)	The MnS producer sends notification (see notifyMOIAttributeValueChanges defined in TS 28.532 [3]) to MnS consumer to notify the progress and result for software management process (see attributes in SwPnfProcess IOC).

13b)	The MnS consumer sends query request to MnS producer to query the attribute values of SwPnfProcess instance (see getMOIAttributes operation defined in TS 28.532 [3]) to obtain the progress and result (including DN of the SwPnfProcess instance and other attributes of SwPnfProcess instance) for software management process.

Following Steps 14 and 15 are the steps for MnS consumer request to terminate an ongoing software management process.

14. The MnS consumer sends a request (see modifyMOIAttributes operation defined in TS 28.532 [3]) to MnS producer to configure cancelScProcess attribute of the SwPnfProcess instance with value "TRUE" to terminate an ongoing software management process.

15. The MnS producer sends a response (see modifyMOIAttributes operation defined in TS 28.532 [3]) for terminating an ongoing software management process to MnS consumer.

16. MnS producer deletes the SwPnfProcess instance when software management process is completed or terminated.

17. MnS producer notifies the MnS consumer about the deletion of a SwPnfProcess instance.

The following steps describes the procedures for MnS consumer request to delete a SwPnfJob instance to ultimate deactivation of requirements for software management management for a set of RAN NEs.

18. MnS consumer sends a request to delete a SwPnfJob instance (see deleteMOI operation defined in TS 28.532 [3]) to MnS producer with the DN of the SwPnfJob instance.

19. Based on the received request, the MnS producer delete the concrete SwPnfJob instance

20. MnS Producer sends a response (see deleteMOI operation defined in TS 28.532 [3]) to the MnS Consumer.

* * * Next Change * * * *

# Annex A (informative): PlantUML source code

## A.1	Relationship UML diagram for PNF software management

@startuml

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

skinparam  NoteBackgroundColor White

class "<<ProxyClass>> \n  ManagedEntity " as ManagedEntity{}

class "<<InformationObjectClass>>\n SwPnfJob " as SwPnfJob {}

class "<<InformationObjectClass>>\n SwPnfProcess" as SwPnfProcess{}

class "<<InformationObjectClass>>\n SwPnfPackage" as SwPnfPackage{}

ManagedEntity "1" *-- "*" SwPnfJob : <<names>>

ManagedEntity "1" *-- "*" SwPnfProcess : <<names>>

SwPnfJob "*" <-left-> "*"SwPnfPackage

SwPnfJob "1" <-right-> "*"SwPnfProcess

note left of ManagedEntity

Represents the folllowing IOCs:

SubNetwork

end note

@enduml

## A.2	Inheritance UML diagram for PNF software management

@startuml

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

skinparam  NoteBackgroundColor White

class "<<InformationObjectClass>>\n Top" as Top{}

class "<<InformationObjectClass>>\n SwPnfPackage " as SwPnfPackage {}

class "<<InformationObjectClass>>\n SwPnfJob" as SwPnfJob{}

class "<<InformationObjectClass>>\n SwPnfProcess" as SwPnfProcess{}

Top <|--SwPnfPackage

Top <|--SwPnfJob

Top <|--SwPnfProcess

@enduml

## A.3	Procedures UML diagram for PNF software management

@startuml

title " Procedures for PNF software management "

participant "SWM MnS Consumer" as SC

participant "SWM MnS Producer" as SP

group Create a SwPnfPackage instance

SC -> SP: 1. Request to create a SwPnfPackage instance \n (list of attributes of SwPnfPackage IOC)

SP -> SP: 2. Create and configure SwPnfPackage MOI

SP -> SC: 3. Response for creating an SwPnfPackage instance

end

group Create a SwPnfJob instance

SC -> SP: 4. Request to create a SwPnfJob instance \n (list of attributes of SwPnfJob IOC)

SP -> SP: 5. Create and configure SwPnfJob MOI

SP -> SC: 6. Response for creating an SwPnfJob instance

end

loop  [Corresponding RAN NE start its software management process]

SP -> SP: 7. Create an SwPnfProcess instance for the software management process \nand configure the created SwPnfProcess instance with \n DN of associated SwPnfJob instance

SP -> SC: 8. Notify the creation of SwPnfProcess instance

opt Configure the Stop Point for SwPnfProcess

SC -> SP: 9. Request to configure attribute stopPointSetIndication\n of the SwPnfProcess instance with value "TRUE"

loop

SP -> SP: 10. Stop hanlding the SwPnfProcess \nwhen the value of attribute progressStateInfo is changed

SP -> SC: 11. Notify the value change of SwPnfProcess instance

SC -> SP: 12. Request to configure attribute stopPointStatus\n of the SwPnfProcess instance to continue the process

end

end

alt  [obtain the progress and result by subscribe-notification method]

loop

SP -> SC: 13a. Notify the progress and result information

end

else [[obtain the progress and result by query method]]

SC -> SP: 13b-1. Request to query the SwPnfProcess information (DN of SwPnfProcess)

SP -> SC: 13b-2. Response with the SwPnfProcess information

end

opt Terminate a SwPnfProcess

SC -> SP: 14. Request to configure "cancelScProcess" attribute \n of the SwPnfProcess with value "TRUE"

SP -> SC: 15. Response with SwPnfProcess configuration result

end

|||

SP -> SP: 16. Delete the SwPnfProcess MOI when \n software management process is completed or terminated

SP->SC: 17.Notify SwPnfProcess deletion (DN of SwPnfProcess MOI)

end

group Delete a SwPnfJob instance

SC -> SP: 18. Request to delete a SwPnfJob instance \n (DN of SwPnfJob MOI)

SP -> SP: 19. Delete SwPnfJob MOI

SP -> SC: 20. Response for deleting an SwPnfJob instance

end

skinparam sequenceActorBackgroundColor #FFFFFF

skinparam sequenceParticipantBackgroundColor #FFFFFF

skinparam noteBackgroundColor #FFFFFF

autonumber "#'.'"

skinparam monochrome true

skinparam shadowing false

hide footbox

@enduml

# Annex B: Change history

* * * End of Changes * * * *