3GPP TSG-SA5 Meeting #165	S5-260203

Goa, India, 9 - 13 February 2026

Source:	Huawei

Title:	pCR TR 28.884 Add solution for message bus data service

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	0.2.0

Work Item:	FS_SBMA_Ph4

Comments

This contribution proposes to add solution for message bus data service in SBMA for 5G.

Proposed Changes

* * * First Change * * * *

## 5.2	Use case #2: Management data streaming based on message bus

### 5.2.1	Description

The 5G network employs a distributed, service-oriented architecture, leading to scenarios where multiple consumers may request the same management data for multiple uses. In a typical network performance data reporting scenario, an MnS producer collects and transmits performance data to multiple MnS consumers within the network management system (e.g., real-time analytics, network optimization engines, visualization dashboards, and audit services).

Using point-to-point WebSocket communication protocol, which operates over a single TCP connection between a client and a server, a separate connection would be established between each consumer and the producer. For the case of multiple consumers requesting the same performance data, it causes the producer to retrieve and duplicate the relevant performance data from its cache for each connection individually.

By adopting a message bus architecture, the producer establishes connection(s) with the message bus and publishes the performance data only once. Consumers retrieve the data whenever they want (i.e., the consumer either pulls the data from the message bus or subscribes and receives messages pushed by the bus) - hence the principle of loose coupling. No additional connections are initiated by the producer, and no duplicate data reporting occurs on its side for the case that multiple consumers request the same performance data. As a comparison example between WebSocket and message bus for the case that multiple consumers request the same performance data as an assumption:

- WebSocket option: duplicate data reporting occurs N times on the producer's side for sending the data to N consumers who request the same performance data.

- Message bus option: depending on the collection job, no duplicate data reporting occurs on the producer's side for sending the data to N consumers (regardless of the number of consumers)who request the same performance data because the only one copy of data buffered on the message bus broker (i.e., a middleware that facilitates communication between different participants by acting as an intermediary for messages) can be polled by all the consumers.

While the message bus architecture introduces certain operational considerations, such as the need for proper message category segmentation and consumer coordination management to avoid throughput bottlenecks. These aspects are addressed through established internal mechanisms and configuration practices. Such implementation details are outside the scope of this document. The message bus interface is not used on ManagedElement/NF level but only on management function level.

### 5.2.2	Potential requirements

### 5.2.3	Potential solutions

5.2.3.1	Solution #1: Multipoint-to-Multipoint management data streaming based on message broking MnF

This potential solution proposes introduction of a new MnF implementing the message broker(s) that mediates data with the help of new MnS services, namely, message bus data reporting MnS and message bus data retrieval MnS (for data subscribers) to enable multipoint-to-multipoint streaming data reporting. This solution is meant for use at the management level and not at the managedElement/NF level. This solution also does not impact existing MnS services and can coexist beside such services as an additional option.

Figure 5.2.3.1-1: Potential solution for multipoint-to-multipoint management data streaming based on message bus

As shown in Figure 5.2.3.1-1, in this solution, message bus data reporting MnS producers send management data to Message broking MnF using the message bus data reporting service based on message bus publishing protocol. Message bus data retrieval MnS consumers receive the management data using the message bus data retrieval service based on message bus retrieval protocol. This approach allows the same copy of management data produced by the message bus data reporting MnS producer(s) to be shared across multiple message bus data retrieval MnS consumers thus enabling multipoint-to-multipoint communication and a publishing-and-subscribing based communication pattern.

Editor’s note: The following aspects are FFS

- What types of management data (e.g. PM) to be supported.

- What specific message bus-based communication protocol to be supported (e.g. Kafka) and why?

3.    How to do data access control.

4.    How the information related to the message bus (e.g., end-point URIs) is conveyed or configured to the MnS producers and MnS consumers needs to be clarified.

5.    Whether new MnSs is needed is to be studied

For message bus data reporting service, the MnS consumer is the message broker MnF, and the MnS producer is the MnF who is going to send the streaming data to the message broker MnF.

The following operations in table 5.2.3.1-1 are considered for message bus data reporting service:

Table 5.2.3.1-1: Operations of message bus data reporting service

| Operations | Inputs | Outputs |
| --- | --- | --- |
| establishMBusConnection (M) | brokerEndpoint (M) producerId (O) msgBodySerializationMethod (O) | connectionId (M) status (M) |
| terminateMBusConnection (M) | connectionId (M) | status (M) |
| addMBusStream (O) | connectionId (M) <topicName, streamInfoList> (M) | status (M) |
| deleteMBusStream (O) | connectionId (M) <topicName, streamIdList> (M) | status (M) |
| reportMBusData (M) | connectionId (M) producerRecord <topicName, msgBody> (M) | status (M) |

- establishMBusConnection (M): This operation enables the MnS producer to establish a connection to the MnS consumer (i.e. message broking MnF). Parameter brokerEndpoint indicates the IP address and port number of the consumer. Optionally, the producer informs the consumer about its own identity by parameter producerId. Optionally, the producer informs the consumer about some configurations of the message bus-based transportation (e.g. serialization method of msgBody). Upon successful connection establishment, the consumer returns connectionId to identify this connection.

- terminateMBusConnection (M): This operation enables the MnS producer to terminate the connection to the MnS consumer (i.e. message broking MnF). Upon successful termination of the connection, the MnS producer stops reporting data to the MnS consumer on this connection.

- addMBusStream (O): This operation enables the MnS producer to add one or more reporting streams (defined in TS 28.532 [3]) to a specific topic on an already established message bus connection. The producer informs the consumer about the tuple of the topic name and the meta-data (i.e. streamInfo defined in TS 28.532 [3]) of the data stream to be reported. This operation provides an optional way for the consumer (i.e. message broking MnF) to know what streaming data to be reported (i.e. streamInfo) and forward the information to the message bus data retrieval service consumer (i.e. real data consumer). This operation is not required when the message broking MnF is not interested in streamInfo or the real data consumer can obtain streamInfo by other means.

- deleteMBusStream (O): This operation allows the MnS producer to remove one or more reporting streams (specified by streamIdList) from a specific topic on an already established message bus connection. The producer stops reporting the specific streaming data to the specific topic.

- reportMBusData (M): This operation enables the MnS producer to send a unit of producerRecord to the MnS consumer. The producerRecord is composed of topicName and msgbody (i.e. streamingData defined in TS 28.532 [3]). Parameter topicName indicates the target topic that the streaming data to be reported to.

For message bus data retrieval service, the MnS producer is the message broker MnF, and the MnS consumer is the MnF who is going to poll the streaming data from the message broker MnF.

The following operations in table 5.2.3.1-2 are considered for message bus data retrieval service:

Table 5.2.3.1-2: Operations of message bus data retrieval service

| Operations | Inputs | Outputs |
| --- | --- | --- |
| establishMBusConnection (M) | brokerEndpoint (M) consumerGroupId (O) consumerId (O) msgBodyDeserializationMethod (O) | connectionId (M) status (M) |
| terminateMBusConnection (M) | connectionId (M) | status (M) |
| addMBusTopic (M) | connectionId (M) topicName (M) | status (M) <topicName, streamInfoList> (O) |
| deleteMBusTopic (M) | connectionId (M) topicName (M) | status (M) |
| pollMBusData (M) | connectionId (M) timeout (O) | consumerRecord <topicName, msgBody, partition(O), offset(O)> (M) status (M) |

- establishMBusConnection (M): This operation enables the MnS consumer to establish a connection to the MnS producer (i.e. message broking MnF). Parameter brokerEndpoint indicates the IP address and port number of the producer. Optionally, the MnS producer is aware of the MnS consumer's group identity by parameter consumerGroupId, so that the producer can identify multiple consumers within the same consumer group to implement balance loading. Optionally, the consumer informs the producer about its own identity by parameter consumerId. Optionally, the consumer informs the producer about some configurations of the message bus-based transportation (e.g. deserialization method of the msgBody). Upon successful connection establishment, the producer returns connectionId to identify this connection.

- terminateMBusConnection (M): This operation enables the MnS consumer to terminate the connection to the MnS producer (i.e. message broking MnF). Upon successful termination of the connection, the MnS consumer stops polling data from the MnS producer on this connection.

- addMBusTopic (M): This operation enables the consumer to subscribe one or more topics from the producer. Upon successful subscription, the producer knows the subscription information of the consumer and may implement load balancing. This operation also provides an optional way for the consumer to retrieve streamInfoList (i.e. meta-data of the streaming data) of all data streams under the subscribed topic, when the producer can provide the stream information.

- deleteMBusTopic (M): This operation enables the consumer to unsubscribe one or more topics from the producer. The consumer stops polling any streaming data under the specific topic.

- pollMBusData (M): This operation enables the MnS consumer to poll one or more units of consumerRecord from the MnS producer with a max waiting time optionally specified by parameter timeout. The consumerRecord is composed of topicName, partition(optional), offset(optional) and msgBody (i.e. streamingData defined in TS 28.532 [3]).

A basic sequence flow of starting performance data streaming based on message bus is as figure 5.2.3.1-2. Step 3, 4, 7 are operations of message bus data reporting service, and step 8~10 are operations of message bus data retrieval service.

Figure 5.2.3.1-2: Basic sequence flow of performance data streaming based on message bus

- Measurement control MnS consumer requests the measurement control MnS producer to start the measurement collection (see step 1 of D.1.1 Sequence flow in TS 28.550 [YY]), conveying message bus target information (e.g. topic and message broker MnF information as mentioned in S5-25xxxx 5.2.3.x).

- Measurement control MnS producer configures the message bus data reporting service producer with the streaming information (see step 2 of D.1.1 Sequence flow in TS 28.550 [YY]). The mechanism of this step is vendor specific.

- Message bus data reporting service producer establish the message bus streaming connection to message broker MnF by invoking the establishMBusConnection operation. If the connection exists, this step can be skipped.

- Message bus data reporting service producer optionally informs message broker MnF of the data streams to be reported (i.e. topicName and streamInfo) by invoking the addMBusStream operation.

- Measurement control MnS producer collects measurement data (see step 5 of D.1.1	Sequence flow in TS 28.550 [YY]). The mechanism of this step is vendor specific.

- Measurement control MnS producer reports collected measurement data to message bus data reporting service producer (see step 6 of D.1.1	Sequence flow in TS 28.550 [YY]). The mechanism of this step is vendor specific.

- Message bus data reporting service producer reports data to message broker MnF by invoking the reportMBusData operation.

- Message bus data retrieval service consumer establish the message bus streaming connection to message broker MnF by invoking the establishMBusConnection operation. If the connection exists, this step can be skipped.

- Message bus data retrieval service consumer informs message broker MnF of the subscription information by invoking the addMBusTopic operation. If streamInfo is reported in step 4, the consumer can retrieve all streamInfo under subscribed topic by this operation.

- Message bus data retrieval service consumer retrieves data from message broker MnF by invoking the pollMBusData operation.

Since the reporting and retrieving action are decoupled, there is no strict requirement for chronological order between step 2~7 and step 8~10.

* * * End of Changes * * * *