3GPP TSG SA5 Meeting #165	S5-260223

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR on TR 28.881 Solution, evaluation and conclusion on intent expectation satisfied information

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	0.3.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This contribution proposes to add solution, evaluation and conclusion for the requirements documented in clause 4.14.2 regarding the enhancement of intent and intent report.

Proposed Changes

* * * First Change * * * *

4.14 Use case #14:  intent expectation satisfied information

4.14.1	Description

In 3GPP TS 28.312 [1], the existing intentReport allows the IDMS consumer to obtain the values of KPI as indicated by the corresponding expectation targets. However, this information may not accurately reflect the actual situation. For example, considering an intent that includes a radio network expectation with the target aveDLRANUEthptTarget, the existing intent report provides the average value of the downlink RAN UE throughput and tells whether the intent expectation is fulfilled or not based on the average value. However, in the case where the average value is impacted by some extreme values, the fulfilled intent report will miss the information that the majority of UE experience doesn’t fulfil the UE throughput specified in the target. Such information matters because it helps the network operator to have a better understanding on the intent expectation satisfaction. The report can include the information besides the average information, which can be entity based, or time based, and is derived through statistical measurement or calculation, such as distributions, range or std deviation.

4.14.2	Potential requirements

REQ-Intent_IESI-1: The intent driven MnS producer should have a capability providing information besides average value in the intent report.

4.14.3	Potential solutions

4.14.3.1 Solution 1: Enhance Intent Report and Intent Report Control

This solution proposes to enhance the intent feasibility check report defined in 3GPP TS 28.312 [1] clause 6.2.1.3.10, expectation exploration result included in intent exploration report defined in clause 6.2.1.3.20, expectation fulfilment results included in intent fulfilment report defined in clause 6.2.1.3.6, and intentReportControl defined in clause 6.2.1.3.16.

Enhancement Aspect #1: In order to enable the MnS consumer to receive more feasibility check information besides the binary check result, the detailed satisfied information is introduced in the existing data type IntentFeasibilityCheckReport, which reflects how comprehensive and consistent the expectation objects achieve the target requirements during the intent feasibility check phase.

The data type IntentFeasibilityCheckReport is enhanced with:

(1)  Add an allowed value "PARTIAL_FEASIBLE" for feasibilityCheckResult.

(2) The detailed satisfied information is conveyed through the following three new attributes.

- objectSatisfiedRatio: an integer attribute indicating the proportion of expectation objects that achieve all target requirements among the total expectation objects within a specified intent expectation. The numerator is the number of objects (i.e., specific performance measurement object within the scope of expectation object, e.g., UE, cell) that meet all target requirements, and the denominator is the total number of expectation objects involved in the specified intent expectation.

The Support Qualifier is CO: In case of the feasibilityCheckResult is "PARTIAL_FEASIBLE" and the satisfied information is described in the form of expectation objects, this attribute needs to be supported.

- durationSatisfiedRatio: an integer attribute indicating the proportion of time during which all target requirements are continuously achieved by the expectation objects within the feasibility check period. The numerator is the cumulative length of time during which the expectation objects continuously meet all target requirements, and the denominator is the total length of the entire feasibility check period.

The Support Qualifier is CO: In case of the feasibilityCheckResult is "PARTIAL_FEASIBLE" and the satisfied information is described in the form of time, this attribute needs to be supported.

- statisticsInfo: a string which contains the statistical measurement or calculation, such as distributions, range or std deviation.

The Support Qualifier is CO: In case of the feasibilityCheckResult is "PARTIAL_FEASIBLE" and the satisfied information requested by the MnS consumer is statistics information, this attribute needs to be supported.

Enhancement Aspect #2: In order to enable the MnS consumer to receive more exploration information besides the average value, the detailed satisfied information is introduced in the existing data type ExpectationExplorationResult, which reflects how comprehensive and consistent the expectation objects achieve the target requirements during the intent exploration phase.

The data type ExpectationExplorationResult is enhanced with:

(1) The detailed satisfaction information is conveyed through three attributes, similar to Enhancement Aspect 1. The only differences are that the “feasibility check period” is replaced by the “exploration period” in the definition of durationSatisfiedRatio, and that the Support Qualifier is specified as Optional.

Enhancement Aspect #3:  In order to enable the MnS consumer to receive more fulfilment information besides the average value, the detailed satisfied information is introduced in the existing data type ExpectationFulfilmentResult, which reflects how comprehensive and consistent the expectation objects fulfil the target requirements during the intent fulfilment phase.

The data type ExpectationFulfilmentResult is enhanced with:

(1) The detailed satisfaction information is conveyed through three attributes, similar to Enhancement Aspect 1. The only differences are that the “feasibility check period” is replaced by the “fulfilment period” in the definition of durationSatisfiedRatio, and that the Support Qualifier is specified as Optional.

Enhancement Aspect #4: In order to enable the MnS consumer to customize the intent report on satisfied information, a new attribute includeSatisfiedInfo is introduced in the existing data type intentReportControl.  It indicates whether the satisfied information of a specific intent expectation is included in the intent report and whether it is object based or time based or pure statistics information.

4.14.4	Evaluation of potential solutions

Enhancement Aspect #1, #2, and #3, i.e., adding three new attributes to cover satisfied information in IntentFeasibilityCheckReport, ExpectationExplorationResult, ExpectationFulfilmentResult respectively, are all feasible and independently to address the requirement REQ-Intent_IESI-1 from different perspectives. They are not mutually exclusive and can be considered complementary. It is recommended to proceed to normative phase. Enhancement Aspect #4, i.e., adding a new attribute in intentReportControl to indicate the satisfied information to be reported, enhances the intent control capability by enabling customization of satisfied information reporting, which aligns with the overall objective of intent expectation satisfied information and is considered beneficial. Therefore, Enhancement Aspect #4 is also recommended to proceed to the normative phase.

* * * Next Change * * * *

# 5	Conclusions and Recommendations

Editor's note: this clause will contain conclusions and recommendations for corresponding key issues identified in clause 4.

## 5.14 Use case #14: intent expectation satisfied information

The description, requirements and potential solutions for intent expectation satisfied information are described in clause 4.14.  This use case proposes to update intent report to support satisfied information. It is recommended to enhance the IntentFeasibilityCheckReport, ExpectationExplorationResult, ExpectationFulfilmentResult and intentReportControl defined in 3GPP TS 28.312 [1] to support above management requirement. The detailed solution in clause 4.1.3 is used as baseline for normative work.

* * * End of Changes * * * *