3GPP TSG SA5 Meeting #165	S5-260228

Goa, India, 9-13 February 2026

Source:	China Telecom

Title:	Pseudo-CR on Solution for KI#1.1: Distributed Charging Architecture with Local CHF for Edge Applications

Document for:	Approval

Agenda item:	7.6.1

Spec:	3GPP TR 32.801-02

Version:	0.2.0

Work Item:	FS_6G_CH

Comments

This pCR proposes to introduce the solution for KI#1.1 based on distributed charging architecture with Local CHF for edge applications in TR 32.801-02 “Study on Charging Aspects of 6G System”.

Proposed Changes

* * * First Change * * * *

5	Topics

5.1	Topic 1: Charging Architecture

5.1.1	General description

This topic focuses on the 6G charging architecture which defines the charging network functions and interfaces. The 6G charging architecture will need to handle increased traffic and data with diverse value generated by new services, and to be in line with the overall 6G system architecture studied in TR 23.801-01 [4]. The charging architecture covers the following aspects:

-	6G charging architecture supporting across diverse networks, services and resources

-	6G charging interface

-	CDRs transmission

-	Interworking

Table 5.1.1-1: Mapping of Use cases, Potential charging requirements, Key issues and Solutions for Topic 1

| Use cases | Potential charging requirements | Key issues | Solutions |
| --- | --- | --- | --- |
| #1.1 | ARC-01 | #1.1 | #1.X |

Editor’s Note: Addition of service/feature into the table is ffs.

5.1.2	Use cases

5.1.2.1	Use Case #1.1: Charging for edge applications

In the 6G era, the MNOs can provide the customized industry/vertical private network services to enterprises/companies (e.g., smart factories, mines, ports). For example, in the low-latency charging scenario, edge applications demand extremely fast response times and low-latency for charging control from 6G charging system.

The potential charging requirements for this UC is: REQ-3GPP6GCH-ARC-01.

5.1.3	Potential charging requirements

The following are potential charging requirements for Use Case #1.1:

-	REQ-3GPP6GCH-ARC-01: The 6G charging system may support charging for edge applications.

5.1.4	Key issues

5.1.4.1	Key Issue #1.1: Charging for edge applications

This key issue addresses the REQ-3GPP6GCH-ARC-01, to investigate charging for edge applications, including

-	The interaction and coordination for low-latency charging scenario;

5.1.5	Solutions

5.1.5.X		Solution #1.X: Distributed Charging Architecture with Local CHF for Edge Applications

5.1.5.X.1	General description

This solution addresses Key Issue #1.1 by introducing a distributed charging architecture. The traditional centralized CHF is divided into a Central CHF (C-CHF) and a Local CHF (L-CHF).

The L-CHF is a delegated function deployed at the network edge (e.g., near the UPF or within the Edge Data Center). It performs real-time quota management and charging control locally, reducing the transmission time between the CTF and the charging system.

C-CHF is located in the central core network and maintains the master subscriber account balance, manages long-term charging data, and coordinates with L-CHFs.

5.1.5.X.2	Procedure description

The interaction between L-CHF and C-CHF follows these steps:

1. Quota Delegation: When a 6G UE initiates an edge service, the C-CHF allocates a bulk quota to the L-CHF based on the subscriber's profile and credit status.

2. Local Control: The CTF interacts directly with the L-CHF via a localized Nchf interface. The L-CHF performs real-time credit control and quota granting.

3. Synchronization: The L-CHF periodically or upon specific triggers (e.g., quota exhaustion, session termination) reports the used units to the C-CHF and requests a new bulk quota if necessary.

4. CDR Generation: L-CHF may generate partial CDRs or charging records, which are later synchronized to the C-CHF.

* * * End of Changes * * * *