3GPP TSG SA5 Meeting #165	S5-260239

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR 32.872 Corrections to TR 32.872

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	TR V0.2.0

Work Item:	FS_RoamRE_CH

Comments

This pCR proposes to make some amendments to the entire TR 32.872 before submitting it for approval.

Proposed Changes

* * * First Change * * * *

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

N40	Reference point between SMF and the CHF.

N16	Reference point between two SMFs.

N47	Reference point between V-SMF and H-CHF.

N107	Reference point between V-CHF and H-CHF.

* * * Next Change * * * *

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CHF	Charging Function

CTF	Charging Trigger Function

CGF	Charging Gateway Function

NF	Network Function

SMF	Session Management Function

* * * Next Change * * * *

#### 5.1.1.1	Use case #1.1: Failures detected between the CTF and the V-CHF

In the LBO N40+N107 charging architecture, for a specific PDU session, there is a charging session between the V-SMF(CTF) and V-CHF, and there is also a charging session between the V-CHF and H-CHF. When a failure detected between V-SMF(CTF) and V-CHF, How the charging system handles the failure should be studied.

Figure 5.1.1.1-1: An example of failure detected between V-SMF(CTF) and V-CHF

The potential charging requirements for this UC are: REQ-3GPPCH-LBIC-1.

* * * Next Change * * * *

#### 5.1.1.2	Use case #1.2: Failures detected between the CTF and the V-CHF

In the LBO N40+N107 architecture, for a specific PDU session, there is a charging session between the V-SMF(CTF) and V-CHF, and there is also a charging session between the V-CHF and H-CHF. When a failure detected between V-CHF and H-CHF, How the charging system handles the failure should be studied.

Figure 5.1.1.2-1: An example of failure detected between V-CHF and H-CHF

The potential charging requirements for this UC are: REQ-3GPPCH-LBIC-2.

* * * Next Change * * * *

#### 5.1.3.1	Key issue #1.1: Failure handling enhancement for failures detected between the CTF and the V-CHF

This key issue is for investigating how to support REQ-3GPPCH-LBIC-1. This investigation covers the following:

-	Identification of the failure handling enhancement for the scenario in which a failure is detected between the CTF and the V-CHF;

* * * Next Change * * * *

5.1.3.2	Key issue #1.2: Failure handling enhancement for failures detected between the V-CHF and the H-CHF

This key issue is for investigating how to support REQ-3GPPCH-LBIC-2. This investigation covers the following:

-	Identification of the failure handling enhancement for the scenario in which a failure is detected between the V-CHF and the H-CHF;

* * * Next Change * * * *

#### 5.1.4.1	Solution #1.1: Handling of the charging session between V-CHF and H-CHF when V-CHF detected failure of charging session with CTF

A possible solution for key issue #1.1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF.

When the V-CHF detects that an expected charging data request for a particular session has not been received within a period of time, the charging session between the V-CHF and CTF may be maintained or released based on local configuration.

-	In the case of the charging session between the V-CHF and CTF is released, the V-CHF sends a Charging Data Request [Termination] to H-CHF to release the charging session between the V-CHF and H-CHF.

Figure 5.1.4.1-1: V-CHF terminates the charging session with H-CHF

-	In the case of the charging session between the V-CHF and CTF is maintained, the V-CHF maintains the charging session between the V-CHF and H-CHF.

* * * Next Change * * * *

#### 5.1.4.2	Solution #1.2: Handling of abnormal messages received by V-CHF

A possible solution for key issue #1.1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF.

When a Charging Data Request [Update, or Termination] received by the V-CHF, which cannot be associated to any existing charging session (i.e., resource in V-CHF), should be handled as specified in clause 5.5.1.2 of 3GPP TS 32.290[3].

* * * Next Change * * * *

#### 5.1.4.3	Solution #1.3: CTF detected failure

A possible solution for key issue #1.1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF. This solution only applicable on application level failure handling.

When the V-CHF is determined not reachable by V-SMF(CTF), V-SMF(CTF) uses application level failure handling (Terminate, Continue, Retry_and_terminate) as specified in clause 5.5.1.1 of 3GPP TS 32.290 [3].

When failure handling is set to 'Continue' and a failover procedure is supported in the V-CHF and the V-SMF(CTF), the V-SMF(CTF) re-sends the request with the ‘Remote CHF resource’ to an alternative V-CHF, then the alternative V-CHF replaces the original V-CHF to interact with the H-CHF.

Figure 5.1.4.3-1: V-CHF re-sends the request to an alternative V-CHF

* * * Next Change * * * *

#### 5.1.4.4	Solution #1.4: V-CHF release the charging session with CTF when V-CHF detected failure of charging session with H-CHF

A possible solution for key issue #1.2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When the V-CHF, acting as an NF consumer, detects a failure of the H-CHF, it determines how to handle the charging session with the NF (CTF) based on the operator's agreement.

In the case of V-CHF decides to release the charging session between the V-CHF and CTF, the V-CHF sends a Charging Data Response or Charging Notify Request to CTF to indicate that the charging session should be released.

Figure 5.1.4.4-1: V-CHF terminates the charging session with CTF

* * * Next Change * * * *

5.1.4.5	Solution #1.5: V-CHF suspend quota management when V-CHF detected failure of charging session with H-CHF

A possible solution for key issue #1.2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF. This solution only applicable for online charging.

When the V-CHF, acting as an NF consumer, detects a failure of the H-CHF, it determines how to handle the charging session with the NF (CTF) based on the operator's agreement.

In the case of V-CHF decides to continue the charging session between the V-CHF and CTF in online charging scenario, the V-CHF instructs the NF consumer (CTF) to suspend quota management as specified in clause 5.3.2.5 of 3GPP TS 32.290[3], and stores the Charging Data Request(s) or charging information marked with ‘QUOTA_MANAGEMENT_SUSPENDED’ from the NF consumer (CTF).

* * * Next Change * * * *

5.1.4.6	Solution #1.6: Handling of abnormal messages received by H-CHF

A possible solution for key issue #1.2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When a Charging Data Request [Initial] is received by the H-CHF, which can be associated to an existing charging session, the H-CHF handles the Charging Data Request as specified in clause 5.5.1.2 of 3GPP TS 32.290[3].

When a Charging Data Request [Update or Termination] is received by the H-CHF, which cannot be associated to any existing charging session, the H-CHF handles the Charging Data Request as specified in clause 5.5.1.2 of 3GPP TS 32.290[3].

* * * Next Change * * * *

#### 5.1.4.7	Solution #1.7: Handling of messages received by H-CHF from alternative V-CHF

A possible solution for key issue #1.2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF. This solution only applicable for application level failure handling.

When the V-CHF is determined not reachable by V-SMF(CTF), the V-SMF(CTF) may re-sends the request to an alternative V-CHF when a failover procedure is supported in the V-CHF and the V-SMF(CTF).

In the scenario of switching from V-CHF to an alternative V-CHF, when a Charging Data Request is received by the H-CHF from alternative V-CHF, which can be associated to an existing charging session (i.e., resource in CHF), but the associated NF consumer identification in the Charging Data Request is changed, should be handled as a valid request, with Charging Data Response including the charging session id (i.e., resource id).

* * * Next Change * * * *

#### 5.2.1.1	Use case #2.1: Failure of one charging session

In the HR roaming charging architecture, for a specific PDU session, there are two charging sessions simultaneously. When a failure is detected in one of the charging sessions, How the charging system handles the failure should be studied.

Figure 5.2.1.1-1: An example of the failure between H-SMF and H-CHF in HR roaming

The potential charging requirements for this UC are: REQ-3GPPCH-LBHR-1.

* * * Next Change * * * *

### 5.2.2	Potential charging requirements

REQ-3GPPCH-LBHR-1: Charging system shall support failure handling for the Home Routed charging scenario in which a failure is detected in one of the charging sessions.

* * * Next Change * * * *

#### 5.2.3.1	Key issue #2.1: Failure is detected in one of the charging sessions

This key issue is for investigating how to support REQ-3GPPCH-LBHR-1. This investigation covers the following:

-	Whether and how to handle when failure is detected in one of the charging sessions;

* * * Next Change * * * *

#### 5.2.4.1	Solution #2.1: Failures detected between the H-CHF and the H-CTF

A possible solution for key issue #2.1 covering requirements REQ-3GPPCH-LBHR-1, failure is detected in one of the charging sessions. This solution only applicable for application level failure handling.

When the H-CHF detects that an expected charging data request for a particular session has not been received within a period of time, the H-CHF handles as specified in clause 5.5.1.2 of 3GPP TS 32.290[3].

When the H-CHF is determined not reachable by H-SMF (CTF), the H-SMF (CTF) uses application level failure handling (Terminate, Continue, Retry_and_terminate) and may store Charging Data Request(s) or charging information as specified in clause 5.5.1.1 of 3GPP TS 32.290 [3]. if the failure handling indicates 'Terminate', the service will be terminated, and the associated charging sessions will be terminated accordingly.

* * * Next Change * * * *

5.2.4.2	Solution #2.2: Failures detected between the V-CHF and the V-CTF

A possible solution for key issue #2.1 covering requirements REQ-3GPPCH-LBHR-1, failure is detected in one of the charging sessions. This solution only applicable for application level failure handling.

When the V-CHF detects that an expected charging data request for a particular session has not been received within a period of time, the V-CHF handles as specified in clause 5.5.1.2 of 3GPP TS 32.290[3].

When the V-CHF is determined not reachable by V-SMF (CTF), the V-SMF (CTF) uses application level failure handling (Terminate, Continue, Retry_and_terminate) and may store Charging Data Request(s) or charging information as specified in clause 5.5.1.1 of 3GPP TS 32.290[3]. if the failure handling indicates 'Terminate', the service will be terminated, and the associated charging sessions will be terminated accordingly.

* * * Next Change * * * *

#### 5.3.1.1	Use case #3.1: Failures detected between the CTF and the V-CHF

In the LBO N40+N47 charging architecture, for a specific PDU session, a charging session between the V-SMF (CTF) and V-CHF and a charging session between the V-SMF (CTF) and H-CHF is to be initiated, updated or terminated. When a failure detected between V-SMF (CTF) and V-CHF, How the charging system handles the failure should be studied.

Figure 5.3.1.1-1: Failure detected between V-SMF(CTF) and V-CHF

The failure can happen at any time in a session, bellow are two cases described either when the V-SMF cannot reach the V-CHF or when the V-CHF cannot reach V-SMF. If the V-SMF (CTF) trigger the H-CHF in the case of V-CHF failure, should be studied.

Figure 5.3.1.1-2: Failure at request from V-SMF(CTF) to V-CHF

Figure 5.3.1.1-3: Failure at response from V-CHF to V-SMF(CTF)

The potential charging requirements for this UC are: REQ-3GPPCH-LBHC-1.

* * * Next Change * * * *

#### 5.3.1.2	Use case #3.2: Failures detected between the CTF and the H-CHF

In the LBO N40+N47 charging architecture, for a specific PDU session, a charging session between the V-SMF (CTF) and V-CHF and a charging session between the V-SMF (CTF) and H-CHF is to be initiated, updated or terminated. When a failure detected between V-SMF (CTF) and H-CHF. How the charging system handles the failure should be studied.

Figure 5.3.1.2-1: Failure detected between V-SMF(CTF) and H-CHF

The failure can happen at any time in a session, bellow are two cases described either when the V-SMF cannot reach the H-CHF or when the H-CHF cannot reach V-SMF.

Figure 5.3.1.2-2: Failure at request from V-SMF(CTF) to H-CHF

Figure 5.3.1.2-3: Failure at request from V-SMF(CTF) to H-CHF

The potential charging requirements for this UC are: REQ-3GPPCH-LBHC-2.

* * * Next Change * * * *

#### 5.3.3.1	Key issue #3.1: Failure handling enhancement for failures detected between the CTF and the V-CHF

This key issue is for investigating how to support REQ-3GPPCH-LBHC-1. This investigation covers the following:

-	Identification of the failure handling enhancement, if any, for the scenario in which a failure is detected between the CTF and the V-CHF;

* * * Next Change * * * *

5.3.3.1	Key issue #3.2: Failure handling enhancement for failures detected between the CTF and the H-CHF

This key issue is for investigating how to support REQ-3GPPCH-LBHC-2. This investigation covers the following:

-	Identification of the failure handling enhancement, if any, for the scenario in which a failure is detected between the CTF and the H-CHF;

* * * End of Changes * * * *