3GPP TSG SA5 Meeting #165	S5-260270

Goa, India, 9-13 February 2026

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Add potential solution for Estimation of NG-RAN EC per PLMN-ID granularity

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.3.0

Work Item:	FS_Energy_Ph4_OAM

Comments

To introduce a potential solution and update the evaluation of the potential solutions, conclusions and recommendations for the following use case defined in clause 5.4.3 of TR 28.885.

-	Use case #3: Support estimation of EC and EE KPIs at per PLMN-ID granularity in Network sharing scenario

Proposed Changes

* * * First Change * * * *

##### 5.4.3.3.a	Potential solution #<a>: Estimation of NG-RAN EC per PLMN-ID granularity

###### 5.4.3.3.a.1	Introduction

This potential solution describes estimation of energy consumption of a NG-RAN at a per PLMN-ID granularity.

###### 5.4.3.3.a.2	Description

The gNB EC KPI defined in clause 6.7.3.4.2 of TS 28.554 [4] can be enhanced to optionally support reporting at PLMN-ID granularity as follows:

a)	EC_gNB_PLMNID

b)	This KPI describes the energy consumption of an NG-RAN at operator-specific, i.e., at a per PLMN-ID granularity. It is obtained using the of the PNF Energy Consumption measurement.

b-1)	Real, kilowatt-hours (kWh).

b-2)	RATIO

c)	EC_gNB_PLMNID =   PEE.Energy * W

Where,

-	PEE.Energy is PNF Energy consumption as defined in clause 5.1.1.19.3 of TS 28.552 [10].

-	W is the measurement for the weighted average, it is one of the following:

-	proportion of combined DL and UL data volume of the NR Cells of the gNB delivered for the particular PLMN, over the same observation period, as described below;

-	for non-split gNBs.

WPLMN=SamplesDRB.PdcpSduVolumeUl_PLMN+DRB.PdcpSduVolumeDl_PMNSamples(DRB.PdcpSduVolumeUL+DRB.PdcpSduVolumeDL)

Where,

-	Measurements DRB.PdcpSduVolumeDL and DRB.PdcpSduVolumeDL_PLMN are DL Cell PDCP SDU Data Volume (defined in clause 5.1.2.1.1.1 of TS 28.552 [10])

-	Measurements DRB.PdcpSduVolumeUL and DRB.PdcpSduVolumeUL_PLMN are UL Cell PDCP SDU Data Volume (defined in clause 5.1.2.1.1.2 of TS 28.552 [10]).

-	for split gNBs.

WPLMN=SamplesDRB.F1uPdcpSduVolumeUl_PLMN+DRB.XnuPdcpSduVolumeUl_PLMN+DRB.X2uPdcpSduVolumeUl_PLMN+DRB.F1uPdcpSduVolumeDl_PLMN+DRB.XnuPdcpSduVolumeDl_PLMN+DRB.X2uPdcpSduVolumeDl_PLMNSamples(DRB.F1uPdcpSduVolumeUl+DRB.XnuPdcpSduVolumeUl+DRB.X2uPdcpSduVolumeUl+DRB.F1uPdcpSduVolumeDl+DRB.XnuPdcpSduVolumeDl+DRB.X2uPdcpSduVolumeDl)

Where,

-	Measurements DRB.F1uPdcpSduVolumeDl (EP_F1U (F1-U interface)) and DRB.F1uPdcpSduVolumeDl_PLMN (PLMN filter) are DL PDCP SDU Data Volume per interface (F1-U interface) defined in clause 5.1.3.6.2.3 of TS 28.552 [10].

-	Measurements DRB.F1uPdcpSduVolumeUl (EP_F1U (F1-U interface)) and DRB.F1uPdcpSduVolumeUl_PLMN (PLMN filter) are UL PDCP SDU Data Volume per interface (F1-U interface) defined in clause 5.1.3.6.2.4 of TS 28.552 [10].

-	Measurements DRB.XnuPdcpSduVolumeDl (EP_XnU (Xn-U interface)) and DRB. XnuPdcpSduVolumeDl_PLMN (PLMN filter) are DL PDCP SDU Data Volume per interface (EP_XnU (Xn-U interface)) defined in clause 5.1.3.6.2.3 of TS 28.552 [10].

-	Measurements DRB.XnuPdcpSduVolumeUl (EP_XnU (Xn-U interface)) and DRB. XnuPdcpSduVolumeUl_PLMN (PLMN filter) are UL PDCP SDU Data Volume per interface (EP_XnU (Xn-U interface)) defined in clause 5.1.3.6.2.4 of TS 28.552 [10].

-	Measurements DRB.X2uPdcpSduVolumeDl (EP_X2U (X2-U interface)) and DRB. X2uPdcpSduVolumeDl_PLMN (PLMN filter) are DL PDCP SDU Data Volume per interface (EP_X2U (X2-U interface)) defined in clause 5.1.3.6.2.3 of TS 28.552 [10].

-	Measurements DRB. X2uPdcpSduVolumeUl (EP_X2U (X2-U interface)) and DRB. X2uPdcpSduVolumeUl_PLMN (PLMN filter) are UL PDCP SDU Data Volume per interface (EP_X2U (X2-U interface)) defined in clause 5.1.3.6.2.4 of TS 28.552 [10].

-	a weight defined by the consumer of KPI

NOTE:	An example of the weight factor (W) defined by the consumer of the KPI can be based on the agreement between the operators on the proportion of the NG-RAN resources shared between the PLMNs or based on the energy consumption of the gNB without traffic. These examples are not subject to standardization.

d)	ManagedElement

e)	This KPI is applicable when a ManagedElement is used to represent the combined functionality of 3GPP defined gNB and designed to run on dedicated hardware, and the NG-RAN is shared in network sharing scenarios as described in 32.130 [11].

* * * Next Change * * * *

#### 5.4.3.4	Evaluation of potential solutions

The potential solution #<a> Estimation of NG-RAN EC per PLMN-ID granularity (described in clause 5.4.3.3.a), provides enhancements to the gNB EC KPI (defined in clause 6.7.3.4.2 of TS 28.554 [4]) to optionally support reporting at PLMN-ID granularity. This potential solution utilizes measurements with PLMN filters (defined in TS 28.552 [10]).

The potential solution #<a> fulfils the requirement PREQ-Energy_Consumption_Network_Sharing-1. This is a feasible candidate as input to the normative phase.

* * * Next Change * * * *

### 6.4.A	Use case #<A>: Support estimation of EC and EE KPIs at per PLMN-ID granularity in Network sharing scenario

The use case, requirements (PREQ-Energy_Consumption_Network_Sharing-1) and potential solutions to support estimation of EC and EE KPIs at per PLMN-ID granularity in Network sharing scenario are described in clause 5.4.3. The evaluation and feasibility of the potential solution is described in clause 5.4.3.4. It is proposed to enhance the gNB EC KPI (defined in clause 6.7.3.4 of TS 28.554 [4]) to optionally support reporting at PLMN-ID granularity in the normative phase.

* * * End of Changes * * * *