3GPP TSG SA5 Meeting #165	S5-260272

Goa, India, 9-13 February 2026

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Add potential solution for Enhancements to 5GC NF Profile to support ES and EE

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.3.0

Work Item:	FS_Energy_Ph4_OAM

Comments

To introduce a potential solution and update the evaluation of the potential solutions, conclusions and recommendations for the following use case defined in clause 5.2.2 of TR 28.885.

-	Use case #2: Enhancements to 5GC NF Profile to support energy saving and energy efficiency

Proposed Changes

* * * First Change * * * *

### 5.2.2	Use case #2: Enhancements to 5GC NF Profile to support energy saving and energy efficiency

#### 5.2.2.1	Description

Several solutions identified for the key issues in TR 23.700-67 [9] include enhancements to 5GC NF Profile (see TS 23.501 [8]) to support energy saving and energy efficiency.

5GC NF Profile related attributes are modelled in ManagedNFProfile data type (see clause 5.3.54 of TS 28.541 [7]).

This use case is to identify the enhancements to ManagedNFProfile data type to support the energy saving and energy efficiency in the 5GC.

#### 5.2.2.2	Potential requirements

PREQ-NFProfile_Energy_Conf-1: The 3GPP management system should be able to support energy saving and energy related configurations in the managed 5GC NF Profile.

#### 5.2.2.3	Potential solutions

##### 5.2.2.3.a	Potential solution #<>: Enhancements to  ManagedNFProfile data type

###### 5.2.2.3.a.1	Introduction

The conclusions for KI#3 of SA WG2 study are documented in clause 8.3 of TR 23.700-67 [9]. This includes the potential enhancements for NF Profile to include parameters "Energy Priority Information", "Schedule of Energy State" and "Energy State" (see Table 8.3-1 of TR 23.700-67 [9]).

###### 5.2.2.3.a.2	Description

5GC NF Profile related attributes are modelled in ManagedNFProfile data type (see clause 5.3.54 of TS 28.541 [7]). This data type needs enhancements to include additional attributes to support the parameters "Energy Priority Information", "Schedule of Energy State" and "Energy State" studied by SA WG2 (see Table 8.3-1 of TR 23.700-67 [9]).

NOTE:	The attributes to be introduced in ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]) needs to be aligned with the SA WG2 and CT WG4 during the normative phase.

##### 5.2.2.3.i	Potential solution #<i>: <Potential Solution i Title>

###### 5.2.2.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

###### 5.2.2.3.i.2	Description

Editor's Note:	This clause further details the potential solution, including all of its aspects and any assumptions made.

#### 5.2.2.4	Evaluation of potential solutions

The potential solution #<a> Enhancements to  ManagedNFProfile data type (described in clause 5.2.2.3.a), identifies enhancements to introduce new attributes to ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]). The potential solution #<a> fulfils the requirement PREQ-NFProfile_Energy_Conf-1. This is a feasible candidate as input to the normative phase.

NOTE:	The attributes to be introduced in ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]) needs to be aligned with the SA WG2 and CT WG4 during the normative phase.

* * * Next Change * * * *

### 6.2.A	Use case #2: Enhancements to 5GC NF Profile to support energy saving and energy efficiency

The use case, requirements (PREQ-NFProfile_Energy_Conf-1) and potential solutions enhancements to introduce new attributes to ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]) are described in clause 5.2.2. The evaluation and feasibility of the potential solution is described in clause 5.2.2.4. It is proposed to enhance ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]) in the normative phase.

NOTE:	The attributes to be introduced in ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]) needs to be aligned with the SA WG2 and CT WG4 during the normative phase.

* * * End of Changes * * * *