3GPP TSG SA5 Meeting #165	S5-260283

Goa, India, 9-13 February 2026

# Source:	China Unicom

# Title:	Discussion paper on management of Ambient IoT features

# Document for:	Information, Discussion, Endorsement

Agenda Item:

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

- 3GPP TR 38.848: "Technical Specification Group Radio Access Network; Study on Ambient IoT (Internet of Things) in RAN".

- 3GPP TR 38.769: "Study on solutions for ambient IoT (Internet of Things) (Release 19)

- R3-258882 AIOT Ph2_38.300 BL CR

- 3GPP TR 23.700-30 Study on Architecture support of Ambient power-enabled Internet of Things (AIoT); Phase 2.

- 3GPP TS 28.552: "Management and orchestration;5G performance measurements(Release 20)".

- 3GPP RP-251885: "New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2".

3	Rationale

3.1	Progress in SA and RAN

Progress in SA

SA5 has already initiated support for AIoT Deployment Scenario 1 and Topology 1 in Rel-19.

3GPP TR 23.700-30[4] developed by SA2 provides guidance to SA5, which encompasses the management support for AIoT services under the RRC-based option for UE Reader connectivity, as well as the management support for feature of DO-A capable AIoT devices.

Progress in RAN

RAN#108 approved the RAN new WID on Rel-20 Ambient IoT[6], with scope including Deployment Scenario 2, Topology 2, and enhancements for active devices and outdoor scenarios.

3.2	Enhancement on management supporting AIoT

3.2.1	NR NRM enhancement

Topology 2 represents the way where the Ambient IoT device communicates bidirectionally with an intermediate node between the device and a base station introduced in TR 38.848 [1]. The following is a logical system architecture defined in TR 38.769 [2].

"

6.4.2	Support of Topology 2

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function in topology 2, which is able to communicate with the A-IoT-enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoTdevice via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

For Topology 2, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 2, XX interface is NG interface.

In Topology 2, the RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers.

An A-IoT-enabled gNB could support both topology 1 and topology 2, this is an implementation matter.

"

Observation#1: The logical system architecture is composed of the RAN supporting A-IoT feature which performs by gNB-readers or UE-readers.

Potential management aspects#1: According to Observation#1, it is proposed to enhance NR NRM to configure the RAN with gNB-readers or UE-readers.

3.2.2	5GC NRM enhancement

SA2 TR 23.700-30[4] concludes that AIoT System Architecture is introduced to support Topology 2.

"

### 7.1.1	Agreed Principles for KI#1

'Figure 7.1.1-1 depicts the AIoT System Architecture for Topology 2.

Figure 7.1.1-1: AIoT System Architecture for Topology 2

Figure 7.1.1-2 depicts the AIoT system architecture, using the reference point representation.

Figure 7.1.1-2: AIoT System Architecture for Topology 2 in reference point representation

"

Observation#2: AIoT System Architecture for Topology 2 with UE readers will be introduced in Rel-20.

Potential management aspects#2: According to Observation#2, it is proposed to enhance the 5GC NRM to configure certain CN NFs (e.g., AIOTF, AMF, NEF, ADM) that support UE reader function.

SA2 TR 23.700-30 [4] also studies DO-A capable device registration for Topology 1 and DO-A data transfer aspects, and reaches several consensuses in this regard.

Observation#3: A new AIoT Devices that support the DO-A traffic type will be introduced in Rel-20.

Potential management aspects#3: According to Observation#3, it is proposed to enhance the 5GC NRM to configure certain CN NFs (e.g., AIOTF, AMF, NEF, ADM) that support the feature of DO-A capable devices.

3.2.3	Performance measurements for AIoT

SA5 TS 28.522 [5] currently includes no relevant research on the performance measurements of AIoT.

Observation#4: Some performance measurements supporting A-IoT will be added in Rel-20.

Potential management aspects#4: Based on Observation #4, it is proposed to enhance existing measurements or introduce new measurements for the RAN and 5GC to support AIoT, including but not limited to AIOTF performance measurements, RRC connection-related measurements and UE-reader function-related measurements.

4	Detailed proposal

It proposes to endorse following statements:

Proposal#1. Propose AIoT WID to capture management enhancement and performance measurements supporting Rel-20 AIoT feature.

Proposal#2. Endorse Potential management aspects#1, #2 , #3 and #4 as guidance for management enhancement and performance measurements supporting Rel-20 A-IoT features based on the normative progress of RAN WGs and SA2.