3GPP TSG SA5 Meeting #165	S5-260343

Goa, India, 9-13 February 2026

# Source:	China Unicom

# Title:	New SID on management enhancements related to NR MIMO

# Document for:	Approval

# Agenda Item:	6.2.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on management enhancements related to NR MIMO

Acronym:	FS_NR_MIMO_OAM

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X |  |  |
| No | X | X |  | X | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

3	Justification

NR MIMO is a key 5G technology that enhances data transmission rates and overall system performance through the use of multiple transmit and receive antennas. The evolution of MIMO has not only improved spectrum efficiency but also significantly increased signal reliability and quality of service.

With the rapid emergence of new services such as UAV and NTN scenarios , an efficient and flexible NR MIMO management mechanism is essential to support 3-D coverage optimization.

As the NR construction increases cost for operators , it is necessary to balance capex and opex in development. On the basis of traditional coverage optimization goals, an efficient and flexible NR MIMO management mechanism is essential to support and improve both radio resource and energy efficiency.

Since Rel-15, RAN working groups have been developing NR MIMO-related work items, with NR_MIMO_Ph6 further studied in Rel-20. O-RAN working groups have also contributed through technical reports such as O-RAN.WG1.MMIMO-USE-CASES, which outlines requirements, key challenges, proposed solutions, and benefits of massive MIMO enhancements, alongside identified impacts on OAM.

NR MIMO management features have remained largely unchanged since Rel-15, limiting management flexibility and constraining radio resource and energy scheduling. This study item therefore aims to investigate potential use cases, requirements, and solutions to enhance existing NR MIMO management frameworks.

4	Objective

The study item focuses on potential management enhancements related to NR MIMO, including:

WT-1: Analyze and identify NR MIMO network management enhancement requirements for different scenarios.

This work task includes following aspects:

• Identify different scenarios for NR MIMO management enhancement:

- 3-D Coverage optimization to support UAV and NTN scenarios

- Beamforming and SSB optimization to support radio resource efficiency

- Beam power optimization to support energy optimization

• Identify whether and how existing 5G management Specifications can support these identified scenarios. .

WT-2:Investigate potential enhancements of configuration and performance management to support fine-grained and flexible evaluation and optimization.

WT-3: Coordination with O-RAN Alliance: Investigate whether and how existing use cases, requirements and solutions of Massive MIMO optimization from O-RAN can apply to existing NR MIMO management.

5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 28.xyz | Study on management enhancements related to NR MIMO | SA#112 | SA#112 |  |

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA Dependency (Yes/No/Maybe) | Non-3GPP Dependency |
| --- | --- | --- | --- | --- | --- |
| WT-1 | 0.3 | 0.3 | Y | N | Maybe |
| WT-2 | 0.2 | 0.2 | Y | N | Maybe |

Total TU estimates for the study phase:  0.5

Total TU estimates for the normative phase: 0.5

Total TU estimates: 1

6	Work item Rapporteur(s)

7	Work item leadership

SA5

8	Aspects that involve other WGs

Potential coordination with RAN WGs

9	Supporting Individual Members

Supporting IM name

China Unicom