3GPP TSG-SA5 Meeting #165	S5-260357

Goa, India, 9 - 13 February 2026

Source:	Ericsson (China)

Title:	Add solution on Energy Rationing Information Management

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.3.0

Work Item:	FS_Energy_Ph4_OAM

Comments

Reason for change: Energy rationing information in the network is crucial for maintaining network performance and achieving energy-saving goals. In order to enable authorized users to provide energy rationing-related information in operators' networks, similar as corresponding energy related information which defined in in clause 8 of TS 28.310 (e.g. EnergySupplyInfo), 3GPP management system should provide a solution to allowed operator to configure energy rationing information which obtained from internal 3GPP NEs or NFs(e.g. off-grid base stations) or external to 3GPP (e.g an energy supplier).

Proposed Changes

* * * First Change * * * *

### 5.1.3	Use case #3: Energy Rationing Information Management

#### 5.1.3.1	Description

Clause 3.1 in TS 22.261 [2] defines energy rationing as a situation in which the availability of energy either across the network or at a particular network element or function is limited or reduced. This use case is for enabling management of energy rationing and energy saving in a network. The configuration of energy rationing information in a network can be used to apply controlled energy usage and energy-saving in an operator’s network. This information could be crucial for maintaining network performance and achieving energy saving objectives. This information can help a consumer to know which services can be modified if a node which has energy rationing applied on it is used.

The energy rationing-related information in the network can assist in prioritizing a node for selection over others in a given scenario. The cause due to which the energy rationing is applied in an operator’s network can be related to regulatory requirements, economic reasons, electricity brownouts, or energy blackouts (full outage of energy supplier). Economic reason refers to raising the price of power (energy units) by an energy supplier during certain periods or locations to force operators to use less energy. In such cases, the MNO might need to reduce energy usage to keep its services economical. Electricity brownout refers to partial power reduction, energy blackout refers to a total energy outage at the energy supplier’s end (e.g., due to faults), requiring the MNO to rely on local power backup sources (battery banks, generators, etc.) and potentially reduce energy consumption to continue providing services until power from the energy supplier resumes.

#### 5.1.3.2	Potential requirements

REQ-Energy_Rationing-CON-1: The 3GPP management system should enable its authorised consumers to provide energy rationing related information in an operator’s network.

#### 5.1.3.3	Potential solutions

##### 5.1.3.3.A	Potential solution #<A>: Add the EnergyRationingInfo IOC in TS 28.310

###### 5.1.3.3.A.1	Introduction

It is proposed to make the following changes to TS 28.310 [3] clause 8.3.1:

- Add the EnergyRationingInfo <IOC>. This IOC defines the information related to energy rationing. This information is configured by the operator with the information obtained from internal 3GPP NEs/ NFs (e.g. off-grid base stations) or external to 3GPP (e.g. an energy supplier).

###### 5.1.3.3.A.2	Description

Add the EnergyRationingInfo <IOC> as the child of energyInfoGroup MOI. Introduce information of Energy Rationing IOC, this Information obtained from internal 3GPP NEs/ NFs (e.g. off-grid base stations) or external to 3GPP (e.g an energy supplier). It may contain:

- An attribute, i.e energyRationingReason, that indicates the reason to apply energy rationing. Reasons for energy rationing include, but are not limited to:

- regulatory requirements

- economic reasons

- electricity brownouts

- energy blackouts

- An attribute of impacted elements describing particular network elements or network functions in operator network which be impacted by the energy rationing.

- An attribute denoting the period of time to apply energy rationing.

- An attribute, i.e. energyAvailibility indicate the remaining amount of energy (e.g. in kWh) locally available for consumption. It available only for this feature be supported by the device.

#### 5.1.3.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of all potential solutions listed in 5.1.3.3

* * * End of Changes * * * *