3GPP TSG-SA5 Meeting #165	S5-260389

Goa, India, 09 - 13 February 2026

Source:	L.M. Ericsson Limited

Title:	Rel-20 pCR TR 28.883 Solution for Defining the Lifecycle and Runtime Behaviour of NDT Jobs

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	V0.2.0

Work Item:	FS_NDT_Ph2

Comments

Solution for the definition of the lifecycle and runtime behaviour of NDTJobs.

Proposed Changes

* * * First Change * * * *

# 5	Use cases

## 5.10	Use Case #7 Defining the Lifecycle and Runtime Behaviour of NDT Jobs

### 5.10.1	Description

The NDTJob lifecycle is not clear, there are some items open to interpretation and some items missing which are described below.

The MnS Consumer should have a method to know which configuration of NDTJob has produced a given report. In the current specification NDTReport refer to the NDT job (i.e NDTJobRef). In the existing specification, it is possible to modify the NDTJob which may result in the different NDTReport. This will result in loosing the link between produced NDT Report and the original NDT Job. The MnS Consumer would benefit from understanding the implications of reconfiguring the NDTJob. At the moment, the NDTReports refer to the DN of the NDTJob – if a NDTJob is reconfigured, this means the same DN is applied to each report, even if the simulation has changed. Likewise, there is no clear procedure described in Clause 6.4, despite the “Modify” use-case being possible for the NDTJob Instances.

Excerpt: Table 6.2.1.3.8.2-1 describes the attributes associated with the NDTReport <<IOC>>

Excerpt: From 6.3 Attribute definitions, the explanation of the ndtJobRef of the NDTReport <<IOC>> refers to

### 5.10.2	Potential requirements

REQ-DTLCM-XX: The MnS Producer should provide a mechanism for the MnS Consumer to distinguish between different configurations of the same NDTJob instance that may exist over time as a result of modify operations.

### 5.10.3	Potential solutions

An NDTJob MOI represents the job request and configuration provided by the MnS Consumer, including the network scenario, synchronization scope, and execution requirements.

To enable unambiguous correlation between NDTReports and the configuration used during NDTJob execution:

- A new read-only attribute (e.g. ndtJobSnapshotId) is added to the NDTJob IOC.

- The attribute identifies the NDTJob configuration information used when an NDTJob execution is started.

- The ndtJobSnapshotId added to the NDTJob is maintained solely by the MnS Producer.

The identifier may be implementation-specific (e.g. internal reference) but shall uniquely identify the configuration used for execution.

Association with NDTReport

- The same new read-only attribute (e.g. ndtJobSnapshotId) is added to the NDTReport IOC.

- The MnS Producer shall populate this attribute with the ndtJobSnapshotId identifier corresponding to the NDTJob execution that produced the NDTReport.

- The ndtJobSnapshotId added to the NDTReport is maintained solely by the MnS Producer.

This ensures that each NDTReport can be correlated both to:

- the NDTJob request (ndtJobRef), and

- the configuration information (ndtJobSnapshotId) used during execution.

### 5.10.4	Evaluation of potential solutions

Only one potential solution has been identified, which is feasible.

# 6	Conclusions and Recommendations

## 6.X	Use case #7: Defining the Lifecycle and Runtime Behaviour of NDT Jobs

There is only a single solution proposed that solves the use-case, the recommendation is to take the solution further into REL20 normative work.

Editor's note:	This clause provides conclusions and recommendations for the corresponding use case.

* * * End of Changes * * * *