3GPP TSG-SA5 Meeting #165	S5-260415

Goa, India, 09 - 13 February 2026

Source:	Nokia

Title:	pCR TR 32.801-01 Add key issue on data management – Representation of time series data

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution proposes to add a key issue on data management.

Proposed Changes

* * * First Change * * *

# 6	6G Management Scenarios

# 7	Key Issues

Editor's note 1: This clause will contain the potential Key Issues and the potential specification requirements corresponding to the 6G management scenarios in clause 6. The Mapping table in Annex A will reflect the relation between the management scenarios and Key Issues.

## 7.1 6G Network Management System Architecture

### 7.1.b Key Issue #b: Data management - Representation of time series data

Time-series data of different kinds should be represented by a common model. A common model will be the basis for common management.

Req: Data management shall be based on a common representation for management data.

The model should include the following information:

-	data identifier: the unambiguous identifier (name) for the data collected, such as temperature or handover failure rate.

-	data value: the value that is collected.

-	time stamp: the date and time when the data value is collected.

-	tags: key value pairs with meta data describing the collected data value, such as the location where the data was collected.

None of these information elements carries meaning by itself. Only the combination of all four information elements has meaning. These four elements form hence the smallest unit of undividable (atomic) information. It can be called data point.

The following example shows a data point for a simple temperature measurement where the data value is an integer (with the unit [°C]).

2026-01-26T18:30:00Z, 18, location=basement

A collection of multiple data points is referred to as a data set.

The following example shows a data set for the simple temperature measurement above.

2026-01-26T18:30:00Z, temperature, 18, location=basement

2026-01-26T18:30:00Z, temperature, 21, location=kitchen

2026-01-26T18:40:00Z, temperature, 18, location=basement

2026-01-26T18:40:00Z, temperature, 23, location=kitchen

Data sets can have tags that are common for all their data points. These tags do not need to b repeated in each data point but can be lifted up to the data set.

Therefore, the data name can be omitted in the data points from the previous example, if the name is meta data of the data set, hence

2026-01-26T18:30:00Z, 18, location=basement

2026-01-26T18:30:00Z, 21, location=kitchen

2026-01-26T18:40:00Z, 18, location=basement

2026-01-26T18:40:00Z, 23, location=kitchen

The data value can be more complex than a scalar, and be represented by key value pairs

For example, wind is measured with two distinct parameters, speed and direction. The unit of speed is for example [km/h], and the direction can be measured in [0° to 360°] relative to true north.

2026-01-26T18:30:00Z, windSpeed=18,windDirection=236, location=roof

Editor's note: This model should have a nice name. The name is ffs.

* * * End of Changes * * * *