3GPP TSG SA5 Meeting #165	S5-260419

# Goa, India, 9-13 February 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0260 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel20 CR TS 28.531 Procedure for NF instance creation on a generic orchestration and management system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NF_Deployment_LCM-OAM |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | 20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Currently clause 7.10 of TS 28.531 specifies the procedure for NF instance creation when the orchestration and management system is ETSI-NFV MANO. This CR adds another clause specifying the procedure for NF instance creation on a generic orchestration and management system not limited to only ETSI-NFV MANO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause adding a procedure for NF instance creation on a generic orchestration and management entity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current specification leads the industry to perceive the 3GPP management system as being able to interact exclusively with ETSI NFV-MANO for the NF instance creation, and not with other O&M systems commonly used in telco environments. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

## 7.10	Procedure of NF instance creation

The Figure 7.10-1 illustrates the procedure of creating a new network function instance to satisfy the required network function related requirements.

Figure 7.10-1: Network Function Instance Creation procedure

1)	Network Function Management Service Provider (NFMS_P) receives a createMOI, for ManagedFunction IOC, request from Network Function Management Service Consumer (NFMS_C) with network function related requirements.

Note:	The network function related requirements see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[6].

2)	If NF instance to be created contains virtualized part, NFMS_P derives the requirements for VNF instance based on the network function related requirements.

3)	If corresponding VNF Package needs to be on-boarded or changed, the NFMS_P invoke corresponding VNF Package management. For the interaction with ETSI NFV MANO, the procedure is described in clause 7.7 in ETSI GS NFV-IFA013 [3]. The VNF package is compliant with ETSI GS NFV-IFA011 [14].

4)	The NFMS_P invokes VNF lifecycle management. For the interaction with ETSI NFV MANO, the procedure is as described in clause 7.3.5 (Update NS operation) in ETSI GS NFV-IFA013 [3] or clause 7.2.2 (Create VNF Identifier operation) and clause 7.2.3 (Instantiate VNF operation) in ETSI GS NFV-IFA008 [15]..

5)	The NFMS_P creates the MOI for the ManagedFunction to be created. If the ManagedFunction contains virtualized part, the NFMS_P may send the request of creating the MOI to the NFMS_P in the ManagedFunction.

6)	The NFMS_P configures the new created MOI with corresponding configuration information (see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[6]).

7)	The NFMS_P sends the createMOI response to NFMS_C with DN of MOI for ManagedFunction.

## 7.10a	Procedure of NF instance creation on a generic orchestration and management system

The Figure 7.10a-1 illustrates the procedure of creating a new network function instance to satisfy the required network function related requirements on a generic orchestration and management system.

Figure 7.10a-1: Network Function Instance Creation procedure on a generic orchestration and management entity

1)	Network Function Management Service Provider (NFMS_P) receives a createMOI, for ManagedFunction IOC, request from Network Function Management Service Consumer (NFMS_C) with network function related requirements.

Note:	The network function related requirements see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[6].

2) If NF instance to be created contains virtualized part, NFMS_P derives the requirements for NF Deployment instance based on the network function related requirements.

3)	If corresponding NF Deployment package needs to be onboarded or updated, the NFMS_P invokes the corresponding operation. If the NF Deployment package needs to be updated, this involves interaction between the NFMS_P and the generic orchestration and management entity via the deployment management reference point.

4)	The NFMS_P interacts with the generic orchestration and management entity via the deployment management reference point to enable the NF Deployment instance creation.

5)	The NFMS_P creates the MOI for the ManagedFunction to be created. If the ManagedFunction contains virtualized part, the NFMS_P may send the request of creating the MOI to the NFMS_P in the ManagedFunction.

6)	The NFMS_P configures the new created MOI with corresponding configuration information (see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[6]).

7)	The NFMS_P sends the createMOI response to NFMS_C with DN of MOI for ManagedFunction.

==============End of change==============