3GPP TSG-SA5 Meeting #165	S5-260435

# Goa, India, 09 – 13 February 2026

Source:	Ericsson

Title:	Pseudo-CR TR 28.882 Add use case on energy-aware FL training process observability

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	0.2.0

Work Item:	FS_AIML_MGT_Ph3

Comments

This pCR proposes the following:

- Make a minor update to current description of sustainable AI/ML framework defined in clause 6.0, replacing “this information” (which is vague and can be misread) with “the configured energy information” (which clarifies that this is not the external consumers that are implied).

- Add a new use case to clause 6.1 “Sustainability for ML Training”. This use case, called energy-aware FL training process observability, is framed within the functional requirements defined in clause 6.0.2, and defines the ability of an FL server to be informed if there is any change of the configured energy information on FL clients participating in a FL training process. These changes, which can be triggered by the operator anytime (based on the information the operator gets e.g. on externa energy suppliers), can make that one or more participating FL clients no longer fulfil the FL client selection criteria. By being informed of these changes, the FL server can make informed decisions.

Proposed Changes

* * * First Change * * * *

# 6	AI/ML sustainability

## 6.0	Sustainable aspects of ML model training and inference

### 6.0.1	Description

ML model training and AI/ML inference can require substantial computing power and therefore significant energy.

For sustainability goals in AI/ML management, it is valuable for the 3GPP management system to reflect the type of energy supplied to the managed entities that are used for ML model training and AI/ML inference (for example, renewable and/or carbon related information). The operator handles this energy related information (e.g., see EnergySupplyInfo in TS 28.310 [9]) in the 3GPP management system, based on knowledge of the infrastructure it manages. In particular:

- the operator obtains energy related information typically based on data from external energy suppliers; the operator sets, maintains and updates the association of energy related information to managed entities that support ML model training and inference, by configuration of the Energy Information defined in TS 28.310 [9].

- the operator defines access control rules/policies to determine what managed entities (e.g. NFs, AI/ML functions) can gain access to the configured energy information.

In the context of ML model training and AI/ML inference, the MnS producer can read the operator-configured energy related information, subject to operator-defined access control. By having access to this information, the MnS producer can support receiving training/inference requests containing energy related information. These requests are triggered by the operator, who performs the role of MnS consumer.

In the context of ML model training, the MnS producer can also expose the operator-configured energy related information to other training functions, for the case these training functions need to know this information for selection criteria. This happens for example in Federated Learning (where FL client, acting as MnS producer, exposes energy related information to other training functions acting as FL servers, for them to manage FL lifecycle, e.g. choose FL clients based on energy criteria) and Distributed Training.

The MnS producer reads operator-configured information and applies it when selecting functions for model training or inference. The accuracy of this information depend on operator configuration and external agreements, and are outside the scope of MnS producer verification.

### 6.0.2	Potential requirements

REQ-ML_SUST-01. The 3GPP management system should enable the operator to configure and update the energy related information for the managed entities representing an ML model training function.

REQ-ML_SUST-02. The 3GPP management system should expose the energy information of the managed entities representing an ML model training function to authorised MnS consumers.

REQ-ML_SUST-03. The 3GPP management system should enable the operator to configure and update the energy information for the managed entities representing AI/ML inference.

REQ-ML_SUST-04. ML Training Function should enable an MnS consumer to request for ML model training with consideration to energy related information.

REQ-ML_SUST-05. ML Training Function should be able to adapt its training to the energy related information updates, according to the conditions provided by MnS consumer which requests the ML model training.

### 6.0.3	Possible solutions

It is proposed one solution consisting of two parts: one impacting TS 28.310 [9] (see clause 6.0.3.1), and other impacting TS 28.105 [4] (see clause 6.0.3.2).

#### 6.0.3.1	Update Energy Information NRM fragment

It is proposed to update the Energy Information NRM fragment represented in 8.2.1 of TS 28.310 [9], by adding in ManagedEntity the following:

- The MLTrainingFunction IOC (see clause 7.3a.1.2.1 in TS 28.105 [4])

- The list of IOCs represented by AIMLSupportedFunction (see clause 7.3a.4.1.1 in TS 28.105 [4])

These additions require importing IOCs from 3GPP TS 28.105 [4].

#### 6.0.3.2	Update MLTrainingFunction

It is proposed to add a new attribute to MLTrainingFunction IOC: “energyInfoGroupRef”. This attribute allows specifying the DN of the energyInfoGroup MOI representing the operator-configured energy information.

### 6.0.4	Possible solutions evaluation

The solution described in clause 6.0.3 is feasible.

The part of the solution described in clause 6.0.3.1 allows satisfying REQ-ML_SUST-01 and REQ-ML_SUST-03.

The part of the solution described in clause 6.0.3.2 allows satisfying REQ-ML-SUST-02.

## 6.1	Sustainability for ML training

Editor’s note: Similar clause structure as in subclause 5.1 will be adopted.

### 6.1.1	Energy-aware FL Training process observability

#### 6.1.1.1	Description

When the operator issues a FL training request with energy information based selection criteria on FL clients (see e.g. “Enhancement on LCM of Federated Learning” use case in clause 5.1.1.5 of the present document), the training MnS producer acting as FL server may use this information to select appropriate FL clients to participate in the FL process.

An appropriate FL client is a managed entity representing a ML model training function that fulfils the following four conditions:

- that supports Federated Learning as learning technology, i.e. supportedLearningTechnology <<dataType>> (see TS 28.105, clause 7.4.17) includes “supportedFLRole”.

- that can become FL client role, i.e. “supportedFLRole” attribute values include FL_CLIENT.

NOTE: A managed entity fulfilling the two conditions above is a FL client.

- that has operator-configured energy information, i.e. supports Energy Information NRM fragment (see TS 28.310 [9], clause 8).

NOTE: A managed entity fulfilling the condition above is an energy-aware FL client.

- whose energy information matches the energy information-based selection criteria included in the FL training request.

NOTE: A managed entity fulfilling the above conditions is an energy-aware FL client that fulfils the selection criteria, i.e. an appropriate FL client to participate in the FL process.

The first three conditions are design-time conditions and therefore are not expected to change. However, the information represented by the last condition can be updated by the operator anytime, asynchronously. Updates on this information can cause that, in the middle of FL process execution, participant FL client(s) are no longer appropriate (i.e. FL clients that did fulfil the fourth condition when the training process got started, do not now), and vice versa (i.e. FL clients that didn’t fulfil the fourth condition when the training process got started, do now). For example, a participant FL client can become non-appropriate when the energy feeding it is changed from renewable to non-renewable. Another example is if a battery-powered FL client runs out of battery. When there are changes on the energy information configured on FL clients, it is important that the FL server can be informed of these changes, so it  can decide what to do with these FL clients, e.g. by taking out the non-appropriate FL clients from the process and/or replacing with appropriate FL clients, or to stop/cancel training on non-appropriate FL clients, etc.

#### 6.1.1.2	Potential requirements

REQ-SUST-FL_OBSERVABILITY_ 01. The 3GPP management system should have a capability allowing a MLTrainingFunction acting as FL server to be informed of configured energy information changes on participant FL clients.

#### 6.1.1.3	Possible solutions

REQ-SUST-FL_OBSERVABILITY_01 can be fulfilled with the existing Energy Information NRM fragment specified in TS 28.310 [9]. The FL server is informed of the attribute values (and their changes) that the operator configures on FL clients, and compare them against the energy information based selection criteria received in the FL training request.

When the FL server identifies that a participant FL client is no longer appropriate (i.e. its configured energy information does not match the FL client selection criteria specified in the FL training request), the FL server can decide what to do with that FL client using existing mechanisms defined in TS 28.105 [4], e.g. to either remove that FL client from the running FL training process (see “participatingFLClientRefList” attribute in MLTrainingProcess IOC), or to stop/cancel its participation (i.e. see “suspendRequest” and “cancelRequest” attributes in MLTrainingRequest IOC).

#### 6.1.1.4	Possible solutions evaluation

The solution proposed for this use case is built on existing mechanisms in TS 28.310 [9] and TS 28.105 [4], and therefore are feasible.

## 6.2	Sustainability for AI/ML inference

Editor’s note: Similar clause structure as in subclause 5.1 will be adopted.