3GPP TSG SA5 Meeting #165	S5-260452

Goa, India, 9-13 February 2026

Source:	Ericsson

Title:	Pseudo-CR on Structuring of ASN.1

Document for:	Approval

Agenda item:	7.6.1

Spec:	3GPP TR 32.801-02

Version:	0.2.0

Work Item:	FS_6G_CH

Comments

Addition of solution for structuring ASN.1.

Proposed Changes

* * * First Change * * * *

#### 5.2.5.X	Solution #2.x: Have separate ASN.1-files for the domains, subsystem, and services level attributes.

Having one common ASN.1 part for the 6G CDR which then refences all the domains, subsystem, and services level ASN.1-files. Using the current TS 32.298 [9] as an example, it would mean that the current TS32.298_CHFChargingDataTypes.asn would importe SMSChargingInformation from SMSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) sMSChargingDataTypes (xy) asn1Module (0) version1 (0)} instead of defining the SMSChargingInformation in the CHFChargingDataTypes, for all domains, subsystem, and services level attributes. In this example the TS32.298_Nchf_SMSChargingDataTypes.asn would be its own file with its own version handling.

* * * End of Changes * * * *