TSG-SA5 Meeting #165	S5-260497

# Goa, IN, 09 – 13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0325 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: compr ehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.105 Correcting of MLContext and ClusteringCriteria datatypes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is an inconsistency in attribute naming for inferenceEntityRef. The main table specifies inferenceEntityRef, while the corresponding constraint table incorrectly refers to inferenceModelRef, resulting in a mismatch. There are errors in the capitalization and spelling of the attributes expectedRuntimeContext and runtimeContext. Some constraint wording is ambiguous and may lead to misinterpretation. The description of the ClusteringCriteria data type contains several issues, including font and language inconsistencies |  |  |  |  |  |  |  |  |
| Summary of change: |  | MLContext data type: - Correct the attribute name in the constraint table from inferenceModelRef to inferenceEntityRef. - Clarify constraint wording for unambiguous interpretation. - Align capitalization and spelling of context-related attribute names. ClusteringCriteria data type: - Correct language and formatting inconsistencies in the description, including font alignment with the IS template. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies and ambiguities may lead to incorrect interpretation and implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.3, 7.4.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 7.4.3	MLContext <<dataType>>

#### 7.4.3.1	Definition

The MLContext represents the status and conditions related to the MLModel. There are three types of context the expectedRunTimeContext, the trainingContext and the runTimeContext, see clause 7.5.1 for details of each type.

#### 7.4.3.2	Attributes

Table 7.4.3.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceEntityRef | CM | T | F | F | F |
| dataProviderRef | M | T | F | F | F |

#### 7.4.3.3	Attribute constraints

Table 7.4.3.3-1

| Name | Definition |
| --- | --- |
| inferenceEntitylRef | Condition: The attributed is supported for expectedRunTimeContext and runTimeContext. |

Next change

### 7.4.19	ClusteringCriteria <<dataType>>

#### 7.4.19.1	Definition

This dataType represents a set of requirements or criteria related to ML models used to decide which ML models can be grouped or clustered together for training.

performanceMetric indicates the criteria based on the performance metric according to which an ML model is mainly evaluated. For example, the models, that are intended to achieve same performance characteristic, can be clustered together for training. It indicates the performance metric used to evaluate the performance of an ML model.

taskType indicates grouping criteria based on the task the ML model is trained for. This can be aIMLInferenceName or capabilityName defined (see clause 7.4.8 and 7.5.1). Such identifiers can indicate the type of inference or the capability supported by the ML model, for example MDA types (3GPP TS 28.104), Analytics ID(s) of NWDAF (3GPP TS 23.288), inference types for NG-RAN (TS 38.300 and TS 38.401), as well as vendor's specific extensions

allowedClusterTrainingTime indicates the combined time limit within which the training of an ML model cluster shall be completed. A cluster of ML models typically takes more time to train together as compared to time taken for training an individual ML model. This criteria allows grouping only those ML models for training whose combined training time does not exceed the the configured time limit. preferredModelDiversity indicates the preferred model diversity types that can be considered when clustering ML models for training together.

NOTE:	It is left up to the producer to decide how these criteria are considered when clustering ML models in case of multiple criteria are specified.

#### 7.4.19.2	Attributes

The ClusteringCriteria includes the following attributes:

Table 7.4.19.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| performanceMetric | O | T | T | F | T |
| taskType | M | T | T | F | T |
| allowedClusterTrainingTime | M | T | T | F | T |
| preferredModelDiversity | O | T | T | F | T |

#### 7.4.19.3	Attribute constraints

End of changes