3GPP TSG SA5 Meeting #165	S5-260626

Goa, India, 9-13 February 2026

# Source:	Samsung

# Title:	DP on cell state handling

# Document for:	Endorsement

Agenda Item:	6.4

1	Decision/action requested

Endorsement

2	References

[1]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM)”.

[2]	3GPP TS 38.401: “NG-RAN; Architecture description”.

[3]	3GPP TS 38.473: “NG-RAN; F1 application protocol (F1AP)”.

3	Rationale

The reasons for the proposed actions are based on the following analysis:

- Misalignment of cell state definitions with RAN specifications: The definition of cellState attribute in NRCellDU IOC of TS 28.541 has not been updated since v15.0.0. This is misaligned with the updated definition of cell state in TS 38.401 (v15.4.0 onwards), which further introduced the concept of “Service State” to provide more granular air interface status reporting (In-Service or Out-Of-Service).

More specifically, the description of “Active” cell state is as follows in TS 28.541:

- Active: the cell is known by both the gNB-DU and the gNB-CU. The cell should be able to serve UEs.

However, the description of “Active” cell state is as follows in TS 38.401:

- Active: the cell is known by both the gNB-DU and the gNB-CU. The cell should try to provide services to the UEs.

The definition of “Service State” [2, 3] that can be newly added to the attributes of NRCellDU is as follows:

- In-Service: the cell is operational and able to serve UEs.

- Out-Of-Service: the cell is not operational, and it is not able to serve UEs. The gNB-DU is trying to make the cell operational.

Here, the term “operational” is different from the term used in SA5, and the “Service State” is reported by the gNB-DU for cells for which the cell state is “Active”.

- Errors and typos in Annex A (Cell state handling) [1]: Annex A contains several ambiguities and typos. For examples, although A.1 states that the relation between administrative state and the “Pre-operation state of the gNB-DU Cell” is defined in clause 8.5 of TS 38.401, it is not. Additionally, in Table A.2-1, the name of F1AP message, “gNB-DU Configuration Update”, is incorrectly cited as “gNB-DU Cell status Update”, which may lead to misunderstandings.

- Unnecessary/inefficient deletion of served cell in Annex A.2 [1]: In the current combined state diagram for gNB cell (Annex A.2), when a gNB cell’s administrative state is transitioned to LOCKED (Transition number 2a, 4, 4a, 10), the gNB-DU sends a message to gNB-CU to delete the served cell information, along with the neighbor cell relations information, from the gNB-CU. When the cell is subsequently UNLOCKED, significant signaling overhead and time delays may occur to reconfigure the deleted cell context.

Therefore, the state diagram must be modified to preserve the cell context in situations such as temporary OAM-led maintenance, where the administrative state is briefly transitioned to LOCKED.

Based on the analysis above, the following is proposed for TS 28.541:

- Proposal 1: Align TS 28.541 with RAN specifications [2, 3] by reflecting the updates from TS 38.401, and add the “Service State” attribute to the NRCellDU IOC, if needed.

- Proposal 2: Clean up Annex A to correct typos, misnamed messages, and inconsistent terminology.

- Proposal 3-1 (Option 1 in S5-260629): Modify the state diagram in Annex A.2 to incorporate the “Service State” and allow the gNB-DU to maintain cell context even when the administrative state transitions to LOCKED.

- Proposal 3-2 (Option 2 in S5-260634): Modify the state diagram in Annex A.2 so that it allows the gNB-DU to defer the deletion of cell context from the gNB-CU when the administrative state transitions to LOCKED.

4	Detailed proposal

Endorse the modifications of TS 28.541 according to proposals 1, 2 and 3-1 (Option 1 in S5-260629), or according to proposals 1, 2 and 3-2 (Option 2 in S5-260634) for releases 16 through 20.