3GPP TSG-SA5 Meeting #162	S5-253789

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0602 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.255 Correction of missing the Slice Info and the DNN Selection Mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In an interworking case there may be a network slice associated with the connection, and this should in this case be reported, also the selection mode for the APN/DNN is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of S-NSSAI, SCC mode, and DNN selection mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing information may lead to incorrect charging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-253574. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### C.3.2.1	Definition of 5G data connectivity charging information

The charging information defined in clause 6.2.1 is used for the SMF+PGW-C to support GERAN/UTRAN access.

The specific PDU session charging information when UE is connected to SMF+P-GW-C via GERAN/UTRAN is provided as defined in table 6.2.1.2.1, with the differences that PDU session is replaced by PDP context in fields description and other differences described under following table C.3.2.1-1:

Table C.3.2.1-1: Structure of PDU Session Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Charging Id | OM | Described in table 6.2.1.2.1. |
| User Information | OM | Described in table 6.2.1.2.1. |
| User Identifier | OC | Described in table 6.2.1.2.1. |
| User Equipment Info | OC | Described in table 6.2.1.2.1. |
| unauthenticatedFlag | OC | Described in table 6.2.1.2.1. |
| Roamer In Out | OC | Described in table 6.2.1.2.1. |
| User Location Info | OC | Described in table 6.2.1.2.1. The "User Location Information" (ULI) IE indicating CGI/SAI/RAI shall apply as described in TS 29.060 [205] clause 7.7.51. |
| User Location Time | OC | Described in table 6.2.1.2.1. |
| UE Time Zone | OC | Described in table 6.2.1.2.1. |
| PDU Session Information | OC | Described in table 6.2.1.2.1. |
| PDU Session ID | M | Described in table 6.2.1.2.1. |
| S-NSSAI | OC | Described in table 6.2.1.2.1. |
| PDU Type | OM | Described in table 6.2.1.2.1. |
| PDU Address | OC | Described in table 6.2.1.2.1. |
| PDU Ipv4 Address | OC | Described in table 6.2.1.2.1. |
| PDU IPv6 Address with Prefix | OC | Described in table 6.2.1.2.1. |
| PDU Address prefix length | OC | Described in table 6.2.1.2.1. |
| IPv4 Dynamic Address Flag | OC | Described in table 6.2.1.2.1. |
| IPv6 Dynamic Address Flag | OC | Described in table 6.2.1.2.1. |
| SUPI PLMN ID | OC- | Described in table 6.2.1.2.1. |
| Serving Network Function ID | OC | Described in table 6.2.1.2.1. |
| Serving Network Function Functionality | M | Described in table 6.2.1.2.1. This field holds “SGSN” when SMF+PGW-C serves GERAN/UTRAN access. |
| Serving Network Function Name | OC | Described in table 6.2.1.2.1. |
| Serving CN PLMN ID | OC | Described in table 6.2.1.2.1. |
| RAT Type | OC | Described in table 6.2.1.2.1. |
| Data Network Name Identifier | M | Described in table 6.2.1.2.1, with DNN replaced by APN. |
| DNN Selection Mode | OC | Described in table 6.2.1.2.1. |
| Authorized QoS Information | OC | Described in table 6.2.1.2.1 QoS information mapped according to interaction with PCC as specified in TS 23.502 [201] clause 4.11.0a.2. |
| Subscribed QoS Information | OC | Described in table 6.2.1.2.1 QoS information mapped according to interaction with PCC as specified in TS 23.502 [201] clause 4.11.0a.2. |
| Authorized Session-AMBR | OC | Described in table 6.2.1.2.1. |
| Subscribed Session-AMBR | OC | Described in table 6.2.1.2.1. |
| PDU session start Time | OC | Described in table 6.2.1.2.1. |
| PDU session stop Time | OC | Described in table 6.2.1.2.1. |
| Diagnostics | OC | Described in table 6.2.1.2.1. |
| Enhanced Diagnostics | OC | Described in table 6.2.1.2.1. |
| Charging Characteristics | OC | Described in table 6.2.1.2.1. |
| Charging Characteristics Selection Mode | OC | Described in table 6.2.1.2.1. |
| 3GPP PS Data Off Status | OC | Described in table 6.2.1.2.1. |
| Unit Count Inactivity Timer | OC | Described in table 6.2.1.2.1. |

The specific PDU Container Information when UE is connected to SMF+P-GW-C via GERAN/UTRAN is provided as defined in table 6.2.1.3.1, with the differences described under following table C.3.2.1-2:

Table C.3.2.1-2: Structure of PDU Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| Time of First Usage | OC | Described in table 6.2.1.3.1. |
| Time of Last Usage | OC | Described in table 6.2.1.3.1. |
| QoS Information | OC | Described in table 6.2.1.3.1. |
| QoS Characteristics | OC | Described in table 6.2.1.3.1. |
| AF Charging Identifier | OC | Described in table 6.2.1.3.1. |
| AF Charging Id String | OC | Described in table 6.2.1.3.1. |
| User Location Information | OC | Described in table 6.2.1.3.1. |
| UE Time Zone | OC | Described in table 6.2.1.3.1. |
| Presence Reporting Area Information | OC | Described in table 6.2.1.3.1. |
| Serving Network Function ID | OC | Described in table 6.2.1.3.1. |
| RAT Type | OC | Described in table 6.2.1.3.1. |
| Sponsor Identity | OC | Described in table 6.2.1.3.1. |
| Application Service Provider Identity | OC | Described in table 6.2.1.3.1. |
| Charging Rule Base Name | OC | Described in table 6.2.1.3.1. |
| 3GPP PS Data Off Status | OC | Described in table 6.2.1.3.1. |

The specific Roaming QBC information is described in table B.2.2.1.2-1 and QFI Container Information is described in table B.2.2.1.3-1 for when UE is connected to SMF+P-GW via GERAN/UTRAN, with the differences that PDU session is replaced by PDP context in fields description.

* * * End of Changes * * * *