3GPP TSG-SA5 Meeting #162 	S5-254070

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.423 | CR | 0213 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.423 Add additional TRSR on Trace Record Header for continuous MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR propose to enhancement on the Trace Record Header with associatedTraceRecordingSessionReference. This is used to ensure trace correlation and address security considerations for continuous MDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhancement on Trace Record Header. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing support to trace record correlation for continuous MDT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, G.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

### 5.2.2	Trace Record Header

The trace record header contains the common fields as specified in the Table 5.2.2-1, in addition it may also contain vendor specific extensions.

Table 5.2.2.1 : Common fields in the trace record header

| Trace Record Header field name | Description |
| --- | --- |
| timeStamp (M) | Time stamp (in milliseconds since Epoch) of when the streaming trace record is produced internally in the Producer encoded as (64 bit integer) |
| nfInstanceId (M) | Unique id of the Producer NF instance that produced this trace record represented by a (String) |
| nfType (M) | Type of the Producer NF that produced this trace record represented by a (String) |
| traceReference (M) | Trace Reference (see clause 5.6 of 3GPP TS 32.422 [3]) (represented by a 6 bytes octet string) See Note 6. |
| traceRecordingSessionReference (M) | Trace Recording Session Reference (see clause 5.7 of 3GPP TS 32.422 [3]) represented by a (2 byte octet string. See Note 1.) |
| traceRecordTypeId (M) | Identifier of the trace record type (see clause 5.2.4 for details) represented by an ENUM with the following values: NORMAL, TRACE_SESSION_START, TRACE_SESSION_STOP, TRACE_RECORDING_SESSION_START, TRACE_RECORDING_SESSION_STOP, TRACE_STREAM_HEARTBEAT, TRACE_RECORDING_SESSION_NOT_STARTED, TRACE_RECORDING_SESSION_DROPPED_EVENTS, TRACE_FILE_OPEN, TRACE_FILE_CLOSE, TRACE_FILE_ABNORMAL_CLOSED, TRACE_RECORDING_SESSION_THROTTLED_START, TRACE_RECORDING_SESSION_THROTTLED_STOP. (See Note 2). |
| ranUeId (O) | RAN defined id to represent a UE (8 byte octet string. See Note 3.) |
| payloadSchemaURI (O) | URI identifying the schema to be used in order to decode the payload represented by a (String. See Note 4.) |
| globalGnbId (CM) | Global gNB ID, as defined in subclause 9.3.1.6 of 3GPP TS 38.413 [23]. Applied for trace reported by gNB-CU-CP, gNB-CU-UP, gNB-DU. |
| vendorExtension (O) | Vendor-specific extension(s) (See Note 5.) |
| associatedTraceRecordingSessionReference (O) | Associated Trace Recording Session Reference used in continuous MDT represented by a 2 byte octet string. |
| CHOICE 1.1: traceRecordName (O) | A unique name string of a Trace Record. This attribute is not required for Administration events. |
| CHOICE 1.2: traceRecordId (O) | A unique integer ID of a Trace Record. |
| NOTE 1: The traceRecordingSessionReference must be present for the Streaming Trace Records with non-zero size payload where the payload carries data captured for a Trace Recording Session and in administrative messages related to a Trace Recording Session (e.g. "Trace Recording Session Start" or "Trace Recording Session Stop"). NOTE 2: The traceRecordTypeId with value "NORMAL" is used for Streaming Trace Records that do not carry an administrative message. NOTE 3: The ranUeId field is present in the trace record header when the identifier is supported by RAN. If RAN UE Id (see 3GPP TS 37.483 [46] and 3GPP TS 38.473 [26]) has been captured in the traced signaling messages that value is used. NOTE 4: The payloadSchemaURI is not required for Streaming Trace Records with payload of zero-size, or payload using common payload format (e.g. used to convey Streaming Trace administrative messages). NOTE 5: The vendorExtension is typically a generic list of key-value pairs. NOTE 6: The encoding of the Trace Reference is a 6-byte Octet String in BCD format. The first 3-byte Octet String is the PLMN ID which consists of MCC and MNC. The next 3-byte Octet String is the Trace ID. The PLMN ID is encoded as specified in clause 9.3.3.5 of 3GPP TS 38.413 [23]. |  |

*** NEXT CHANGE ***

# G.2	Trace Record Protocol Buffer (GPB) definitions

Normative GPB Trace Record schema, defined per clause 5.2:

syntax = “proto3”;

/* Trace Record per 3GPP 32.423 specification.

* v16

*/

enum TraceRecordType {

NORMAL = 0;

TRACE_SESSION_START = 1;

TRACE_SESSION_STOP = 2;

TRACE_RECORDING_SESSION_START = 3;

TRACE_RECORDING_SESSION_STOP = 4;

TRACE_STREAM_HEARTBEAT = 5;

TRACE_RECORDING_SESSION_DROPPED_EVENTS = 6;

TRACE_RECORDING_SESSION_NOT_STARTED = 7;

TRACE_FILE_OPEN = 8;

TRACE_FILE_CLOSE = 9;

TRACE_FILE_ABNORMAL_CLOSED = 10;

TRACE_RECORDING_SESSION_THROTTLED_START = 11;

TRACE_RECORDING_SESSION_THROTTLED_STOP = 12;

TRACE_SESSION_NOT_STARTED = 13;

}

message GlobalGnbId {

bytes plmn_identity = 1;

int64 gnb_id = 2;

}

message TraceRecordHeader {

int64  time_stamp = 1;

string nf_instance_id = 2;

string nf_type = 3;

bytes trace_reference = 4;

bytes trace_recording_session_ref = 5;

TraceRecordType trace_rec_type_id = 6;

optional bytes ran_ue_id = 7;

optional string payload_schema_uri = 8;

GlobalGnbId global_gnb_id = 9;

map<string, string> vendor_extension = 10;

optional bytes associated_trace_recording_session_ref = x;

}

message TraceSessionStart {

map<string, string> vendor_extension = 1;

}

message TraceSessionStop {

map<string, string> vendor_extension = 1;

}

message TraceRecordingSessionStart {

map<string, string> vendor_extension = 1;

}

message TraceRecordingSessionStop {

string reason = 2;

map<string, string> vendor_extension = 1;

}

message TraceStreamHeartbeat {

map<string, string> vendor_extension = 1;

}

message TraceRecordingSessionDroppedEvents {

int64 number_of_dropped_events = 1;

map<string, string> vendor_extension = 2;

}

message TraceRecordingSessionNotStarted {

string reason = 1;

map<string, string> vendor_extension = 2;

}

message TraceFileOpen {

map<string, string> vendor_extension = 1;

}

message TraceFileClose {

map<string, string> vendor_extension = 1;

}

message TraceFileAbnormalClosed {

string reason = 1;

map<string, string> vendor_extension = 2;

}

message TraceRecordingSessionThrottledStart {

string reason = 1;

map<string, string> vendor_extension = 2;

}

message TraceRecordingSessionThrottledStop {

map<string, string> vendor_extension = 1;

}

message TraceSessionNotStarted {

string reason = 1;

map<string, string> vendor_extension = 2;

}

message CommonTracePayload {

oneof record_payload {

TraceSessionStart trace_session_start = 1;

TraceSessionStop trace_session_stop = 2;

TraceRecordingSessionStart trace_recording_session_start = 3;

TraceRecordingSessionStop trace_recording_session_stop = 4;

TraceStreamHeartbeat trace_stream_heartbeat = 5;

TraceRecordingSessionDroppedEvents trace_recording_session_dropped_events = 6;

TraceRecordingSessionNotStarted trace_recording_session_not_started = 7;

TraceFileOpen trace_file_open = 8;

TraceFileClose trace_file_close = 9;

TraceFileAbnormalClosed trace_file_abnormal_closed = 10;

TraceRecordingSessionThrottledStart trace_recording_session_throttled_start = 11;

TraceRecordingSessionThrottledStop trace_recording_session_throttled_stop = 12;

TraceSessionNotStarted trace_session_not_started = 13;

}

}

message TraceRecordPayload {

optional int64 payload_size = 1;

bytes binary_payload = 2;

}

message TraceRecord {

TraceRecordHeader header = 1;

TraceRecordPayload payload = 2;

}

message StreamingTraceRecord {

TraceRecord record = 1;

optional CommonTracePayload administrative_message = 2;

}