3GPP TSG-SA5 Meeting #164	S5-255655

Dallas, USA, 17 - 21 November 2025											Revision of S5-255278

Source:	Ericsson

Title:	Pseudo-CR TR 28.888 Add use cases for authorization of external MnS consumer using CAPIF.

Document for:	Approval

Agenda item:	6.20.9

Spec:	3GPP TR 28.888

Version:	0.1.0

Work Item:	FS_EnExpo

Comments

The authorization of external MnS consumer using CAPIF (i.e. external MnS consumer Y2 as described in TS 28.533) is a feature consisting of two use cases:

- Service API authorization request (@CAPIF-1e interface), wherein the external MnS consumer requests CCF for authorization to access one or more service APIs, and the CCF (performs the role of OAuth2.0 server) issues this authorization to the consumer in the form of an access token.

- Service API invocation request (@CAPIF-2e interface), wherein the external MnS consumer issues a service API call together with the access token, and the AEF (performs the role of resource server) verifies the access token to authorize/deny the invocation.

This pCR proposes to add description and requirements for these two use cases. Solutions and evaluation are left out for now.

Proposed Changes

* * * First Change * * * *

# 5	Use cases and potential solutions

## 5.X	Use case #<X>: Service API authorization request of an external MnS consumer using CAPIF

### 5.X.1	Description

Service API authorization request using CAPIF is a procedure performed between an onboarded API invoker and the CCF over the CAPIF-1e interface. In this procedure, the API invoker requests CCF for authorization to access one or more service APIs. The CCF issues this authorization to the API invoker in the form of an access token. This token contains the permissions granted to the API invoker for requested service APIs, being these permissions represented in the token’s scope. For further details of this procedure, see steps 1-4 described in clause 4.X.1.1.

The token’s scope can have one of the formats described in clause 4.X.1.2. The format used needs to be supported by both the CCF and the serving AEF. If Format 0 is selected, then the access token contains service API level authorization, i.e. the list of service API names that the API invoker is authorized to invoke. If Format 1 is selected, then the access token contains resource and/or operation level authorization, i.e. the list of resources and/or operations of each service API name that the API invoker is authorized to invoke. If Format 2 is selected, then the level of authorization depends on the content agreed between the CCF and the serving AEF.

When the AEF is MSED’s AEF, then the API invoker is an authorized MnS consumer, and the service APIs contain published management service information.

### 5.X.2	Potential requirements

PREQ-FS_EnExpo-AuthZ-0X: The operator should select MSED’s AEF and CCF such that both support the same scope format of the access token.

### 5.X.3	Potential solutions

### 5.X.4	Evaluation of potential solutions

* * * Next Change * * * *

## 5.Y	Use case #<Y>: Service API invocation request of an external MnS consumer using CAPIF

### 5.X.1	Description

Service API invocation request using CAPIF is a procedure performed between an onboarded API invoker and an AEF over the CAPIF-2e interface. In this procedure, the API invoker issues a service API call along with invoker’s access token. The AEF verifies whether the service API call content (i.e. service API name, resource, operation) are within the granted permissions represented in the token’s scope or not, and authorizes the invocation accordingly. If authorization is successful, the service API invocation can be fulfilled. Otherwise, it returns an error. For further details of this procedure, see steps 5-8 described in clause 4.X.1.1.

When the AEF is MSED’s, then the API invoker is an authorized MnS consumer, and the service APIs contain published management service information.   The fulfilment of an authorized service API invocation requires it to reach the serving MnS producer, in the form of MnS API request.

### 5.Y.2	Potential requirements

PREQ-FS_EnExpo-AuthZ-0X: The MSED’s AEF should support authorizing service API invocations of one or multiple external MnS consumers.

PREQ-FS_EnExpo-AuthZ-0Y: The MSED’s AEF should support delivering authorized service API invocations as MnS API requests for their fulfilment.

### 5.Y.3	Potential solutions

### 5.Y.4	Evaluation of potential solutions

* * * End of changes * * * *