3GPP TSG SA5 Meeting #165	S5-260544

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR 32.872 Define the trigger conditions for V-CHF

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	TR V0.2.0

Work Item:	FS_RoamRE_CH

Comments

This pCR proposes to introduce a new solution on Local Breakout Inter CHFs scenario key issue3.

Proposed Changes

* * * First Change * * * *

#### 5.1.4.d	Solution #1.d: Define the trigger conditions for V-CHF

The solution addresses key issue #1.x.

The Charging Data Requests are issued by the V-CHF towards the H-CHF when certain trigger conditions are met. The applicable triggers in the V-CHF for inter-CHF communication are outlined as follows:

- Receive Charging Data Request [Initial] from original NF consumer;

- Receive Charging Data Request [Update] from original NF consumer;

- Receive Charging Data Request [Termination] from original NF consumer.

The triggers above will not be included in the message of Charging Data Requests sent to the H‑CHF.

* * * End of Changes * * * *