3GPP TSG SA5 Meeting #165	S5-260546

Goa, India, 9-13 February 2026

Source:	Ericsson

Title:	Pseudo-CR on Solution topic 1 issue 2 using SBA

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	0.2.0

Work Item:	FS_RoamRE_CH

Comments

Addition of using service-based architecture for H-CHF detected V-CHF failure and using service-based architecture for V-CHF detected H-CHF failure solutions.

Proposed Changes

* * * First Change * * * *

#### 5.1.4.x	Solution #1.x: Using service-based architecture for H-CHF detected V-CHF failure

A possible solution for key issue #2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When the H-CHF detects a failure, it may use the procedure outlined in TS 29.500 [x] clause 6.5.3.2, to restore a connection to the V-CHF. If the restoring of the connection fails it is operator specific if the H-CHF is to report this in the CDR with a specific reason for closing the CDR.

* * * Next Change * * * *

#### 5.1.4.y	Solution #1.y: Using service-based architecture for V-CHF detected H-CHF failure

A possible solution for key issue #2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When the V-CHF detects a failure, it may use the procedure outlined in TS 29.500 [x] clause 6.5.3.3, to restore a connection to the H-CHF. If the restoring of the connection fails it is operator specific if the connection between V-CHF and CTF is to be maintained and for how long.

* * * End of Changes * * * *