3GPP TSG SA5 Meeting #165	S5-260838

Goa, India, 9-13 February 2026

Source:	Ericsson GmbH, Eurolab, Orange, AT&T, Red Hat Limited, Verizon

Title:	Pseudo-CR on Correlation Context Structure

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

Reason for change: Add study of Correlation Context to Study on 6G Management and Orchestration.

Proposed Changes

* * * First Change * * * *

#### 6.1.y.1	Management scenario: Systematic Root Cause Analysis and End-to-End Observability

##### 6.1.y.1.1	Description

This management scenario describes how a causality context, established at the point of initiation of a management action and propagated across all participating domains, enables two complementary capabilities: systematic root cause analysis and end-to-end observability. The causality context is carried alongside management operations and attached to the telemetry (logs, metrics, traces, events) produced by each domain.

Telecommunication operators already face significant challenges when correlating the effects of management actions across their existing network domains. When a management operation, such as a configuration change or a scaling action, triggers a chain of downstream activities, operators have no standardised mechanism to trace the causal relationship between the initiating action and its effects. Instead, they rely on timestamp-based heuristics and vendor-specific correlation methods to reconstruct what happened. These approaches are inherently fragile: clock skew introduces ambiguity, parallel operations create false correlations, and boundaries between management domains break the chain of association entirely.

This problem becomes substantially worse as operators adopt cloud-based and disaggregated network deployments. In such environments, a single management action may traverse the 3GPP management domain, one or more network function domains, and external cloud infrastructure platforms, each producing its own telemetry independently. The number of domain boundaries increases, the diversity of telemetry sources grows, and the correlation challenge scales accordingly. Root cause analysis becomes labour-intensive and unreliable, and end-to-end observability across the full management stack remains impractical.

##### 6.1.y.1.2	Example 1: Systematic Root Cause Analysis of a management action

An OAM system initiates a configuration change targeting a network function. As the request passes through the 3GPP management domain, the network function processes the change and emits a completion event. The management system determines that additional resources are required and issues a scaling request to the cloud infrastructure platform. The cloud platform processes the scaling request, and if it encounters a failure, it emits a failure event.

When the operator investigates the original configuration failure, they need to understand the causal relationship between the management operation and the infrastructure failure.

##### 6.1.y.1.3	Example 2: End-to-End Observability of a management action

A management operation (e.g. performance metric collection) is initiated for a specific network slice instance. As the operation progresses, a network function involved in the slice emits performance data. Similarly, the underlying cloud infrastructure supporting that network function likewise emits infrastructure metrics.

There is no clear mechanism that enables the 3GPP management system to associate the data that was produced in the network function (related to the network slice) and the cloud infrastructure.

* * * End of Changes * * * *