3GPP TSG-SA WG6 Meeting #68	S6-253161

Goteborg, Sweden 25th – 29th August 2025	(revision of S6-252123)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0460 | rev | 2 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating existing IE to enable combining multiple ad hoc group calls (MCPTT) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph6-MC |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR updates the existing information elements to enable combining multiple ad hoc group calls. The changes address the ad hoc group call request and ad hoc group call release information elements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Note 6 in Table 10.19.2.1-1 is updated to include the possibility that an authorised MCPTT user may include the identity associated with an already combined ad hoc group call without including the list of participants and/or the criteria. A reason to release an on going ad hoc group call is added in Table 10.19.12.8-1 to inform the users of releasing a call to be combined with other ongoing one(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Authorised MCPTT user may not be able to initiate set up procedure for ad hoc group call based on combined multiple ad hoc group calls. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.2.1, 10.19.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 10.19.2.1	Ad hoc group call request (MCPTT client – MCPTT server)

Table 10.19.2.1-1 describes the information flow ad hoc group call request from the MCPTT client to the MCPTT server.

Table 10.19.2.1-1: Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID (see NOTE 1, NOTE 6) | O | The MCPTT group ID to be associated with the ad hoc group communication |
| Encryption supported (see NOTE 2) | O | Indicates whether this ad hoc group call supports end-to-end encryption |
| MCPTT ID list (see NOTE 3, NOTE 6, NOTE 7) | O | MCPTT IDs of the participants being invited for the ad hoc group call |
| SDP offer | M | Offered Media parameters |
| Implicit floor request (see NOTE 4) | O | When originating client requests the floor, this element shall be included |
| Broadcast indicator (see NOTE 5) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 5) | O | Indicates that the ad hoc group call request is an MCPTT imminent peril ad hoc group call |
| Emergency Indicator (see NOTE 5) | O | Indicates that the ad hoc group call request is an MCPTT emergency ad hoc group call |
| MCPTT ID list (see NOTE 3) | O | List of participants required to acknowledge the ad hoc group call before start of the audio transmission |
| Location information | O | Location of the calling party. |
| Criteria for determining the participants (see NOTE 6, NOTE 7) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location based criteria to invite participants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: If this information element is not included, the MCPTT server assigns an MCPTT ad hoc group ID to be used for the ad hoc group communication. This information element is returned to the calling party to use in the ad hoc group communication. If the request follows an ad hoc group for emergency alert, then this element must be present. NOTE 2: This information element is present and set to true only if this ad hoc group call is encrypted. When the ad hoc group call is initiated with participants provided by the initiator this acts as an indicator that subsequent requests follow targeting the individual participants and carrying the relevant key material. If this information element is set to false or not present, then this ad hoc group call is unencrypted. NOTE 3: This element is included only when the originating client sends the list of participants. NOTE 4: This element is included only when the originating client requests the floor. NOTE 5: If used, only one of these information elements is present. NOTE 6: Either the MCPTT ad hoc group ID is present (in case ad hoc group call request follows an ad hoc group combine procedure), or either the MCPTT ID list or criteria for determining the participants is present. NOTE 7: If the request follows an ad hoc group for emergency alert, then this element is not present. |  |  |

# * * * * Second change * * * *

#### 10.19.2.8	Ad hoc group call release request (MCPTT server – MCPTT client)

Table 10.19.2.8-1 describes the information flow ad hoc group call release request from the MCPTT server to the MCPTT client.

Table 10.19.2.8-1: Ad hoc group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the ad hoc group call participant |
| Functional alias | O | The functional alias of the ad hoc group call participant |
| MCPTT ad hoc group ID | M | The MCPTT group ID of the ad hoc group call on which call is released |
| Reason for release the ad hoc MCPTT group call | O | Indicates the reason for releasing the ongoing ad hoc MCPTT group call, e.g., it is released due to combining it with another ongoing ad hoc MCPTT group call(s) into a new ad hoc group call |

# * * * * End of changes * * * *