3GPP TSG-SA6 Meeting #68	S6-253172

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0048 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements to FL member grouping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The clause 8.17 on FL member grouping procedure contains request sent by VAL server to AIMLE server. The FL member grouping support create request contains an optional parameter “List of candidate/selected FL member IDs” and there is no provision of VAL server to provide a member selection criteria for grouping in this request. Hence, eventhough the “List of candidate/selected FL member IDs” is optional, its implicit that the VAL server needs to perform client discovery as member selection criteria is not a part of this request. So its required either to make the “List of candidate/selected FL member IDs” mandatory or define selection criterias for group members. This proposal is to mitigate this gap and add a “FL member selection criteria along with “number of clients” in this request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification of FL grouping support create request to incorporate client selection criteria and number of clients in group |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The VAL server will either be forced to perform AIMLE member selection prior to forming FL group or send a request with no selection criteria except model ID and ADAE analytics ID. - VAL server will not be able to put a limit on max or min number of AIMLE/FL clients in group. This can be an important aspect for charging regulation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 8.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

## 8.17	Support for FL member grouping

### 8.17.1	General

This clause provides the procedure for the grouping of the FL members using AIMLE. Such grouping for the given FL process, applicable to a specific VAL request or ML model ID or ADAE analytics ID. This grouping can be applicable also for a given service area in which one or more FL processes are expected to run. The grouping of FL members is performed for optimizing the process of selection and updating FL members which are entering or leaving the group, since due to dynamicity of FL member (e.g. AIMLE clients) changes this would be impose additional signalling / complexity.

### 8.17.2	Procedure

In this procedure, the AIMLE support capability is described for grouping the FL members, where the grouping is tailored to a specific ML task (VAL triggered task or analytics event/ID).

The grouping procedure covers the:

-	creation of the FL member group;

-	query for an individual FL Member whether it is part of the created group;

-	change of the FL member group, including

a)	modification of the group member based on a change on the availability or capability of the FL member (e.g. due to high load or energy consumption the FL member may have limited capability to act as FL client for a given area and time);

b)	update of the group due to a new member entering or an existing member leaving the FL group; and

-	deletion of the FL member group, and notification of the FL members regarding the deletion of the FL member group.

Figure 8.17.2-1 illustrates the procedure for supporting the FL member grouping.

Pre-conditions:

1.	VAL Server is connected to AIMLE Server.

2.	The candidate/selected FL member has registered to the FL member registry based on the capability in clause 8.4.

Figure 8.17.2-1: Procedure for FL member grouping

1.	The VAL server sends an FL member grouping support request to the AIMLE Server for supporting an FL process. The initial request is to create the FL member grouping support as described in Table 8.17.3.1-1, which may be followed by other requests for querying (as described in Table 8.17.3.1-2), change (as described in Table 8.17.3.1-3), or deletion (as described in Table 8.17.3.1-4) of the FL member grouping support.

NOTE 1:	The FL member grouping support request can be triggered by the AIMLE Server itself.

2.	The AIMLE Server based on the request, determines the need for creating and processing a group consisting of the needed FL members for a given ML task (i.e., an ML model training/inference job ID). The need for creating an FL member group may be based on an ML task for a given AIMLE service area or for a given AIMLE service area where one or more ML tasks are expected to run, based on step 1 request.

3.	To create, query, or change the FL member grouping support, the AIMLE Server fetches the available FL members for the given ML task (i.e., an ML model training/inference job ID) from the ML repository. Based on this information, AIMLE Server may select one or more FL members for the group for the ML task.

NOTE 2:	If the FL member is an AIMLE client, this step re-uses procedure 8.9.2 on AIMLE client selection.

4.	The AIMLE Server creates, configures, and processes the FL member group based on the available or selected FL members by the aggregator which may be the VAL Server or the AIMLE Server, generate group ID and store mapping to group member IDs. The criteria for determining the group members can be the capabilities of the FL participants, or whether the candidate participants are fixed or mobile nodes and their availabilities, the proximity of the participants among them.

The VAL server may additionally provide FL client selection criteria or dataset requirement on basis of which the AIMLE sever may further interact with ML repository and 5GC nodes like NEF (monitoring events, AFsessionwithQoS, memberUE etc.) to get a list of clients supporting criteria mentioned in request to form a group.

For the creation of the FL member group, AIMLE may utilize SEAL GMS capability for the FL member group ID generation.

5.	The AIMLE Server interacts with the each candidate FL member from the configured FL member group as shown in step 5a.

NOTE 3:	If the request is about the deletion of the FL member group, the FL member grouping indication includes notification for the deletion of the FL member group.

5a.	The AIMLE Server sends indication to the candidate FL member (the AIMLE client which is deployed on UE) about the group ID and the group member identities for the ML model ID / ADAE analytics ID (based on the request in step 1.

5b.	The candidate FL member sends to the AIMLE Server a FL grouping indication acknowledge.

6.	The AIMLE Server sends an FL member grouping support response to the VAL request indicating the group creation or providing the query response, change or deletion (based on step 1 request) and the group information.

### 8.17.3	Information flows

#### 8.17.3.1	FL member grouping support request

Table 8.17.3.1-1 shows the request sent by the VAL Server to AIMLE Server for the FL member grouping procedure to create the FL member grouping support.

Table 8.17.3.1-1: FL member grouping support create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the VAL Server performing the request. |
| VAL service identity | O (NOTE) | The identity of the VAL service for which the request applies. |
| ML model ID | O (NOTE) | The model ID for which the request applies |
| ADAE analytics ID | O (NOTE) | The ADAES analytics ID (as specified in TS 23.436) for which the FL grouping is to be used, in case when FL process is used for a given analytics task. |
| ML task information | O | Information related to the ML task / job for which the FL grouping is used. Such task can be an FL training task or FL inference task. This information may include an ML task ID which may be an FL process ID or correlation ID. |
| Number of required FL clients | O | Minimum and Maximum number of clients required in group. In absence of this, AIMLE server will rely on its local configuration. |
| ML model profile | O | The ML model profile for which the FL grouping is to be used. |
| FL Client selection criteria | O NOTE X | Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2 |
| List of candidate/selected FL member IDs | O NOTE X | The list of candidate or selected FL member identities (if known by the VAL Server) which are to be used in the grouping. If the FL member is a VAL UE, this is equivalent to the VAL UE ID. |
| > FL member type | O | The type of FL members (FL client, FL Server) |
| > FL member status | O | The status (selected, candidate) of the FL member. |
| NOTE: At least one of these information elements shall be present NOTE X: At least one of these information elements shall be present |  |  |

* * * End of Changes * * * *