3GPP TSG-SA WG6 Meeting #68	S6-253247

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.438 | CR | 0018 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures to support digital assets usage report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Digital assets usage report provides more insignts to the owners on how they are being utilized by other users and how popular they are. Currently the usage aspects are being stored as part of digital asset profile as history information which is incorect. This should be maintained by the digital assets server and should be made available to the authorized user when requested for effective management of the digital assets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper introduces the procedure for supporting digital assets usage report by the DA server |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Digital assets usage report is a salient feature and if it is not supported by the enabler layer it will lead to incomplete feature and specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.x	Digital asset usage report

### 8.x.1	General

The following clauses specify procedures, information flows, and APIs for digital asset usage report.

### 8.x.2	Procedures

#### 8.x.2.1	Digital asset usage report (Request-Response)

Figure 8.x.2.1-1: Digital asset usage report (Rquest-Response)

1.	The requestor (e.g. DA client or VAL server) sends a digital assets usage report request to the DA server to get the digital assets usage information report and the request contains the information elements listed in Table 8.x.3.1-1.

2.	The DA server authenticates and authorizes the request. If authorized, the DA server prepares the digital assets usage report based on the requested information received in step 1.

3.	The DA server provides the digital assets usage report response to the requestor. The response message includes the information as specified in Table 8.x.3.2-1.

#### 8.x.2.2	Digital asset usage report (Subscribe-Notify)

Figure 8.x.2.2-1: Digital asset usage report (Subscribe-Notify)

1.	The requestor (e.g. DA client or VAL server) sends a Digital assets usage report subscription request to the DA server. The request includes the information as specified in Table 8.x.3.3-1.

2.	Upon receiving the request, the DA server checks whether the requestor is authorized to subscribe for the digital assets usage report.

3.	If authorized, DA server sends the Digital assets usage information subscription response as specified in Table 8.x.3.4-1 with status as success otherwise failure if not authorized.

4.	The DA server creates the report related to digital assets usage based on the filters provided in the step 1 and provides notification to the requestor as per the reporting interval. The notification message includes the information as specified in Table 8.x.3.5-1.

* * * Next Change * * * *

### 8.x.3	Information flows

#### 8.x.3.1	Digital asset usage report request

Table 8.x.3.1-1 describes the information elements for digital asset usage report request sent from a requestor (e.g. DA client) to a DA server.

Table 8.x.3.1-1: Digital asset usage report request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Digital assets filters | M | Indicates the digital assets filter for the report as specified in Table 8.x.3.1-2 |

Table 8.x.3.1-2: Digital asset usage report filters

| Information element | Status | Description |
| --- | --- | --- |
| Digital assets identifiers | O (NOTE) | Indicates the digital assets identities |
| Location information | O(NOTE) | Indicates the location information of a particular geographical area for identifying the digital assets usage in that area |
| Application service information | O(NOTE) | Indicates the digital assets usage of a particular application (e.g., application identifier) |
| Identities of the consumers | O(NOTE) | Identifiers of the users (e.g. VAL user ID, VAL Server identity) of whose digital assets usage information/report required. |
| Unutilized digital assets | O(NOTE) | Indication to include the list of digital assets identities which are discovered but not accessed/downloaded |
| Undiscovered digital assets | O(NOTE) | Indication to include the list of digital assets identities which are not discovered by the consumers |
| NOTE: Atleast one of these information elements shall be present. |  |  |

#### 8.x.3.2	Digital asset usage report response

Table 8.x.3.2-1 describes the information elements for digital asset usage report response sent from DA server to requestor (e.g. DA client).

Table 8.x.3.2-1: Digital asset usage report response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| Digital assets usage report | M | Details of the digital assets usage report |
| > List of digital asset (NOTE) | O | Identifies the list of digital assets |
| >> Digital asset identifier | M | Identity of the digital asset |
| >>> List of consumers | M | Indicates the list of consumer identifiers (VAL user Id, VAL server ID) accessed the digital assets |
| >>> Location information | O | Indicates the geographical area where the digital asset is accessed frequently |
| > Location information (NOTE) | O | Indicates the geographical area for which digital assets usage report is generated |
| >> List of Digital asset identifiers | M | Identifies the digital assets being used/accessed in the given geographical area |
| >>> Number of access | M | Indicates the no of times a particular digital asset is accessed/used in the given geographical area. This helps to identify the frequently or less frequently used digital assets in a given geographical area |
| > List of Identifiers of the consumers (NOTE) | O | Indicates the list of consumer identifiers (VAL user Id, VAL server ID) accessed the digital assets |
| >> List of Digital asset identifiers | M | Identifies the digital assets being used/accessed by the consumer |
| > Unutilized digital assets | O | Indicates the list of digital assets identities which are discovered but not accessed/downloaded |
| > Undiscovered digital assets | O | Indicates the list of digital assets identities which are not discovered by the consumers |
| Failure response | O | Indicates that the request was failure |
| > Reason code | M | Reason for the failure |
| NOTE: Success response shall include at least one of these information elements.. |  |  |

#### 8.x.3.3	Digital asset usage subscription request

Table 8.x.3.3-1 describes the information elements for digital asset usage subscription request sent from requestor (e.g. DA client) to DA server.

Table 8.x.3.3-1: Digital asset usage subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Digital assets filters | M | Indicates the digital assets filter for the report as specified in Table 8.x.3.1-2. |
| Reporting interval | O | Indicates the periodic reporting interval of information |
| Notification endpoint | O | Indicates the endpoint(e.g., IP address, FQDN, URI) to send the information notification |

#### 8.x.3.4	Digital asset usage subscription response

Table 8.x.3.4-1 describes the information elements for digital asset usage subscription response sent from DA server to requestor (e.g. DA client).

Table 8.x.3.4-1: Digital asset usage subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Digital asset usage information subscription ID | M | Identity for the digital asset usage information subscription |
| Failure response | O | Indicates that the subscription request has failed. |
| > Cause | O | The cause for the request failure. |

#### 8.x.3.5	Digital asset usage report notification

Table 8.x.3.5-1 describes the information elements for digital asset usage report notification sent from DA server to requestor (e.g. DA client).

Table 8.x.3.5-1: Digital asset usage report notification

| Information element | Status | Description |
| --- | --- | --- |
| > Digital asset usage information subscription ID | M | Identity for the digital asset usage information subscription |
| Digital assets usage report | M | Details of the digital assets usage report |
| > List of digital asset | O (NOTE) | Identifies the list of digital assets |
| >> Digital asset identifier | M | Identity of the digital asset |
| >>> List of consumers | M | Indicates the list of consumer identifiers (VAL user Id, VAL server ID) accessed the digital assets |
| >>> Location information | O | Indicates the geographical area where the digital asset is accessed frequently |
| > Location information | O (NOTE) | Indicates the geographical area for which digital assets usage report is generated |
| >> List of Digital asset identifiers | M | Identifies the digital assets being used/accessed in the given geographical area |
| >>> Number of access | M | Indicates the no of times a particular digital asset is accessed/used in the given geographical area. This helps to identify the frequently or less frequently used digital assets in a given geographical area |
| > List of Identifiers of the consumers | O(NOTE) | Indicates the list of consumer identifiers (VAL user Id, VAL server ID) accessed the digital assets |
| >> List of Digital asset identifiers | M | Identifies the digital assets being used/accessed by the consumer |
| > Unutilized digital assets | O | Indicates the list of digital assets identities which are discovered but not accessed/downloaded |
| > Undiscovered digital assets | O | Indicates the list of digital assets identities which are not discovered by the consumers |
| NOTE: Digital assets usage report must include at least one optional IE. |  |  |

* * * End of Changes * * * *