3GPP TSG-SA WG6 Meeting #68	S6-253348

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Samsung

Title:	Pseudo-CR on updates to key issue #3

Spec:	3GPP TR 23.700-83 v0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Narendranath (Naren) Durga Tangudu (n.tangudu@samsung.com)

1. Introduction

This pCR proposes updates to Key issue #3 (Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios). New open issues are added.

2. Reason for Change

With respect to the key issue #3 (Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios), it is not clear as to how the AIMLE server or ML repository is aware of the serving PLMN information of the AIMLE client that it is serving. For example, a UE that is associated with a AIMLE client, moves from PLMN A to PLMN B. And, in such a scenario, the AIMLE server, ML repository may not be aware of this movement of the UE associated with the AIMLE client. Further, the AIMLE server, ML repository is not aware of the serving PLMN information of the UE associated with a AIMLE client in order to provide the required support in cross-PLMN roaming scenarios.

Additionally, the current AIML specification and other aspects of the Key issue #3, do not focus on various conditions (for e.g., availability of AIMLE servers, load conditions of serving AIMLE servers, change in ML tasks requirement from AIMLE client , etc.) that can trigger the mobility of AIMLE context. Only VAL UE’s mobility is considered.

The above open issues need to be addressed in this study, and hence this pCR proposes new open issues to the existing key issue #3.

3. Conclusions

None.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 5.3	Key Issue #3: Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios

### 5.3.1	Description

As part of Rel-19, AIMLE was introduced as an enabler for application-layer AI/ML services, supporting model training, distribution primarily within a single operator’s network domain. However, as AI/ML services increasingly involve VAL UEs moving across network domains (e.g., cross-PLMN mobility, UE roaming, federation across operators), and AI/ML services becoming increasingly distributed and dynamic, the ability to ensure service continuity and mobility of AIMLE functions becomes a critical requirement.

This key issue aims to study how AIMLE services can support mobility, including roaming scenarios where UEs access AI/ML services in visited networks, and cross-PLMN environments where AI/ML operations involve entities from different operators. This will address both the continuity of AI/ML services and the federation across MNOs and CSPs/ECSPs.

Additionally, the roaming aspects of VAL UEs do not address how the AIMLE entities (AIMLE server, ML repository) are aware of the serving PLMN / network information of the UE associated with the AIMLE client. For example, when an UE that is associated with a AIMLE client, moves from PLMN A to PLMN B. Then, in such case, the AIMLE server and/or ML repository may not be aware of this movement of the UE associated with the VAL AIMLE client. Further, the AIMLE server, ML repository are not aware of the serving PLMN information of the UE associated with a AIMLE client in order to provide the required support in cross-PLMN roaming scenarios.

In particular, as ML model inference increasingly moves toward the network edge to meet ultra-low latency and bandwidth efficiency demands (e.g., in real-time video analytic, industrial automation), maintaining inference continuity across mobility events becomes critical. When VAL UEs move across network boundaries or service areas, edge AIMLE instances may need to be reselected, dynamically discovered, or seamlessly replaced to ensure uninterrupted AI/ML inference execution and delivery.

The current AIMLE specifications and ongoing studies focus on static deployments and assume the VAL UE remains within its home network. However, real-world deployments often require:

1.	VAL UEs to maintain access to AIMLE services when roaming into visited PLMNs or NPNs,

2.	Cross-border AI/ML service collaboration (e.g. in eV2X or multiplayer gaming scenarios),

3.	Edge inference or AI model access to continue seamlessly even as the UE moves between coverage areas.

Also, the current AIML specification and other aspects of the ongoing studies do not focus on various conditions (for e.g., availability of AIMLE servers, load conditions of serving AIMLE servers, change in ML tasks requirement from AIMLE client , etc.) that can trigger the mobility of AIMLE context. Only VAL UE’s mobility is considered.

* * * Next Change * * * *

### 5.3.2	Open Issues

Therefore, the following aspects need to be studied:

1.	Identify the use cases for supporting multiple PLMN/NPN, and across MNOs and CSPs/ECSPs scenarios

2.	Whether and how AIMLE services can support VAL UEs that roam into visited PLMNs or NPNs, e.g. registration, discovery, and access of AIMLE in the visited domain, as well as go back to home domain.

3.	Whether and how to ensure service continuity and switching for inference tasks or AI service sessions during UE mobility across MNOs and CSPs/ECSPs.

4.	Whether and how UEs or VAL functions can discover the most suitable AIMLE instance in edge-specific scenarios while UEs move across network boundaries or service areas, considering latency, capability, and service policy.

5.	Whether and how the AIMLE server, ML repository are aware of the serving PLMN/network information of the UE associated with the AIML client.

6.	Whether and how the AIMLE servers are aware of conditions for movement of AIMLE context transfer.

NOTE: Open issues 2, 3, 4, 5 and 6 will be based on the identified use cases in open issue 1.

* * * End of Changes * * * *