3GPP TSG-SA WG6 Meeting #69	S6-254079

Wuhan, China 13th – 17th October 2025	(revision of S6-253xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0488 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call forwarding for Ad hoc Group calls (information flows) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph6-MC |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Stage1 has agreed CR0083 against TS 22.179 on call forwarding for ad hoc group call (clause 6.20.1): [R-6.20.1-001] The MCPTT Service shall provide a mechanism for an authorized MCPTT User, receiving an Ad hoc Group call, based on its MCPTT user Identity or its associated functional alias to configure the forwarding of an incoming Ad hoc Group call, to another MCPTT user in the following situations: - Always - If the receiving MCPTT User is not reachable (e.g., out of coverage) - If the incoming Ad hoc Group call is a call with manual commencement mode and the receiving MCPTT User does not answer within a configured period of time - If the receiving MCPTT User rejects an incoming call (in manual commencement mode) [R-6.20.1-002] The MCPTT Service shall provide a mechanism to inform the initiator of the Ad hoc Group call that the call has been successfully forwarded to another MCPTT user by providing the MCPTT user Identity and/or associated functional alias of the forwarding target. This CR addresses the requirements from above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the information flows to cover call forwarding for ad hoc group calls |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirement to provide call forwarding for ad hoc group call is not fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 10.19.2.22, (new) 10.19.2.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 10.19.2.22	Ad hoc group call forwarding request (MCPTT client - MCPTT server and MCPTT server – MCPTT server)

Table 10.19.2.22-1 describes the information flow of the ad hoc group call forwarding request from the MCPTT client to the MCPTT server and MCPTT server to MCPTT server.

Table 10.19.2.22-1: Ad hoc group call forwarding request (MCPTT client - MCPTT server and MCPTT server – MCPTT server) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID requesting the call forwarding |
| Functional alias | O | The functional alias of the requesting participant |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call of the requesting participant |
| MCPTT ID (see NOTE) | O | The target MCPTT ID of the call forwarding |
| Functional alias (see NOTE) | O | The target functional alias of the call forwarding |
| NOTE: One identity shall be present. |  |  |

#### 10.19.2.23	Ad hoc group call forwarding response (MCPTT server - MCPTT client and MCPTT server – MCPTT server)

Table 10.19.2.23-1 describes the information flow of the ad hoc group call forwarding response from the MCPTT server to the MCPTT client and MCPTT server to MCPTT server.

Table 10.19.2.23-1: Ad hoc group call forwarding response (MCPTT server - MCPTT client and MCPTT server – MCPTT server) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID requesting the call forwarding |
| Result | M | Result of the call forwarding request – success or fail |

* * * End Of Change * * *