3GPP TSG-SA6 Meeting #69	S6-254097

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025                             (revision of S6-xxxxxx)

Source:	Airbus

Title:	Key Issue: Recording a simultaneous MCPTT/MCVideo session

Spec:	3GPP TR 23.700-39 V0.3.0

Agenda item:	9.2

Document for:	Approval

Contact:	Jukka Vialen (jukka.vialen@airbus.com)

1. Introduction

This contribution proposes a new key issue to study the issues related to recording of a simultaneous session.

2. Reason for Change

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 V0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.283: " Mission Critical Communication Interworking with Land Mobile Radio Systems; Stage 2".

[7]	3GPP TS 23.289: "Mission Critical services over 5G System; Stage 2".

[8]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[9]	3GPP TS 33.180: "Security of the Mission Critical (MC) service".

[z]	3GPP TS 24.379: " Mission Critical Push To Talk (MCPTT) call control; Protocol specification".

* * * Second Change * * * *

### 5.x	Key Issue y: Recording a simultaneous MCPTT/MCVideo session

This KI is related to scenario 1 and more specifically to clause 10.4 of 3GPP TS 23.280 [2], clause 10.8 of 3GPP TS 23.379 [3] and clause 7.11 of 3GPP TS 23.281 [4].

According to 3GPP TS 23.280 [2],

“A simultaneous session is functionality whereby the MC service client can receive the media from multiple MC service calls/sessions over the same SIP session and media bearer(s) between the MC service client and the MC service server”.

The following text is created by Google Gemini.

Key points from TS 24.379 regarding simultaneous sessions include:

- Establishment: A simultaneous session can be established during an on-demand call setup or, more commonly, when a pre-established session is being set up or modified. The SIP signaling specifies that this single session can handle multiple call identities.

- SIP Signaling: The specification details the SIP INVITE and re-INVITE messages that are used to manage a simultaneous session. The messages include specific feature tags and XML bodies (e.g., <mcptt-info>) to signal the presence of multiple calls. The SIP dialog remains open while media for multiple calls is being handled.

- Multiple call-ids: Within a single SIP session (identified by a single SIP Call-ID), the protocol allows for multiple MC calls, each with its own unique identifier. The client uses these identifiers to distinguish between the different calls it is participating in.

- Media Handling: The specification clarifies that a simultaneous session can use a single media bearer for all calls, or it can be modified (e.g., via a SIP re-INVITE) to establish multiple media bearers, potentially to handle changes in priority or to separate different media types (e.g., video and audio). The UE is responsible for mixing or presenting the different audio streams to the user in a manageable way.

- Priority Management: TS 24.379 explains how the priority of different calls within a simultaneous session is handled. For instance, a higher-priority emergency call can be initiated, and the SIP signaling will instruct the client to handle it appropriately, even if other, lower-priority calls are active on the same session.

According to 3GPP TS24.379 [z]:

It needs to be studied whether SIPREC can separate multiple calls in one SIP session / media bearer or if some MC specific functionality needs to be added to the MCS servers. It also needs to be decided how much of these details, if any, are necessary in 3GPP TSs and which issues shall be left out-of-3gpp-scope.

The solution(s) should identify impacted clauses and propose changes/amendments, preferably in format that is easy to copy to the CRs during the Rel-20 normative work for this feature.

* * * End of changes * * * *