3GPP TSG-SA WG6 Meeting #69	S6-254174

Wuhan, China, 13th – 17th Oct 2025

Source:	ZTE Corporation

Title:	Solution for KI#1 on support for sensing service registration and subscription

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Wei Luo <luo.wei11@zte.com.cn>

1. Introduction

This paper proposes a solution to KI#1 on support for sensing service registration and subscription procedure.

2. Reason for Change

This paper proposes a solution to KI# 1 on support for sensing service registration and subscription procedure. This solution addresses the key issue on how to perform Sensing Service registration for a Sensing Service Supplier and  how to perform Sensing Service subscription for a sensing consumer.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

6.x	Solution #x: Support for sensing service registration and subscription

6.x.1	Solution description

This solution aims to address the issues identified in Key Issue 1, specifically on  Sensing Service Supplier registration and Sensing Service subscription.

In this solution it is proposed to introduce a SEAL Sensing Management Server (SMS), which determines and coordinates a sensing procedure between sensing service supplier and consumer. The SEAL SMS receives and stores one or more Sensing Service Suppliers' capability information as sensing service supplier profile(s). Then after the SEAL SMS receives sensing subscription request from a sensing consumer, it will check whether the sensing requirements carried in the sensing subscription request can be met or not according to the stored sensing service supplier profile(s) information.

In this solution, a functional entity (e.g., 5GS NF, vertical application server/client) which can provide sensing results may act as the Sensing Service Supplier. It interacts with the SMS and provide sensing capability information. A VAL server or client may act as the Sensing Service Consumer and initiate the sensing service subscription procedure if it’s interested in sensing service.

6.x.2	Procedures

6.x.2.1	Procedure for Sensing Service Supplier registration

Figure 6.x.2.1-1 illustrates the procedure of Sensing Service Supplier-triggered sensing service registration.

Figure 6.X.2.1-1: sensing service registration procedure

- The Sensing Service Supplier (e.g.,5GS NF, Sensing application server/client) sends sensing service registration request to the SMS, including the identifier of the  Sensing Service Supplier and capabilities information. The capabilities information may include supported sensing types (e.g., object detection, tracking, environmental monitoring), target object type (e.g., humans, vehicles, infrastructure), sensing accuracy metrics (e.g.,range, positioning precision), supported sensing service areas and geographical limitations, supported sensing parameters (update frequency, latency) and so on.

- The SMS checks authorization for the Sensing Service Supplier 's registration request.

- After successful authorization, SMS sends sensing service registration response to the Sensing Service Supplier and stores the Sensing Service Supplier's information received in step 1 as sensing service supplier profile information.

6.x.2.2	Procedure for Sensing Service subscription

Pre-conditions:

- The SMS has received and stored one or more Sensing Service Supplier profile(s) information.

Figure 6.x.2.2-1: Sensing service subscription procedure

- The VAL server/client acting as sensing consumer sends a sensing service subscription request to the SMS including some sensing subscription parameters such as Sensing service area of interest, Sensing target object type (human/vehicle), location range, accuracy requirements, QoS requirements(e.g., update frequency, latency constraints for real-time use cases) and subscription duration(i.e.,validity period of the sensing result subscription).

- The SMS shall check if the VAL server/client is authorized to initiate the sensing service subscription request.

- The SMS may further check if it meets the sensing requirements as received in sensing subscription parameters or not according to the stored one or more Sensing Service Supplier profile(s) information.

- The SMS replies with a sensing service subscription response indicating the subscription result.

6.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *