3GPP TSG-SA WG6 Meeting #69	S6-254248

Wuhan, China 13th – 17th October 2025	(revision of S6-254xyz)

Source:	Ericsson

Title:	Pseudo-CR on new KI on Roaming Considerations for Service API Invocation

Spec:	3GPP TR 23.700-43 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This pCR provides a new key issue for TR 23.700-43 on enhancements required to CAPIF to support gaps identified from use cases and scenarios from SDOs/industry forums, e.g., how to avoid that a service API invocation is served when the UE is roaming and no exposure related roaming agreements with the visiting network exists in the home network.

2. Reason for Change

One existing gap in current deployments, detected by field experiences, is how to avoid in a controlled way that a service API invocation is served when, e.g., the UE is roaming and no exposure related roaming agreements with the visiting network exists in the home network or indicates that the service should not be delivered over a visiting network.

Another problem exists when, e.g., the application invoker is not willing to pay for any extra charges due to the UE being roaming.

To ensure correct handling of Service API invocations, knowledge of whether the UE is roaming or not is necessary for evaluating roaming policies and application constraints. To reduce barriers in the usage of service APIs by applications, the API invoker should not be aware and should not be forced to become aware of the serving PLMN of the UE.

A solution in this area would enhance CAPIF to enable the control of service API invocations when the UE is roaming.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.1.0)

* * * First Change * * * *

# 4	Key issues

## 4.x	Key issue #Y: Roaming Considerations for Service API Invocation

### 4.x.1	Description

One existing gap in current deployments, detected by field experiences, is how to avoid in a controlled way that a service API invocation is served when, e.g., the UE is roaming and no exposure related roaming agreements with the visiting network exists in the home network or indicates that the service should not be delivered over a visiting network.

Another problem exists when, e.g., the application invoker is not willing to pay for any extra charges due to the UE being roaming.

To ensure the correct handling of a Service API invocation, knowledge of whether the UE is roaming or not is necessary for evaluating roaming policies and application constraints. To reduce barriers in the usage of service APIs by applications, the API invoker should not be aware and should not be forced to be aware of the serving PLMN of the UE.

A solution in this area would enhance CAPIF to enable the control of service API invocations when the UE is roaming.

### 4.x.2	Open issues

To support the control of service API invocations when the UE is raoming, the following aspects need to be studied:

-	How to determine and ensure correct handling on Service API invocations, based on:

-	roaming agreements and policies between CSPs needed for exposure; and

-	application preferences related to roaming of the UE.

-	How to minimize the number of interactions with the 5GC for roaming determination of UEs.

* * * End of Change * * * *

<Proposed change in revision marks>