3GPP TSG-SA WG6 Meeting #68	S6-253412

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253026)

Source:	Netherlands Police

Title:	Pseudo-CR on Scenario for Discreet monitoring of MCData SDS (using media plane)

Spec:	3GPP TR 23700-37-020

Agenda item:	9.1

Document for:	Approval

Contact:	keesdotverweijatpolitiedotnl

1. Introduction

A scenario detailing MCData SDS (using media plane) is added to clarify the usecase and identify technical impact. The text originates from TR 23.784, but has been enhanced as required.

2. Reason for Change

Scenario for MCData SDS (using media plane) is missing.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-37-020 v 0.2.0.

* * * First Change * * * *

## 4.7	Scenario 6:	Discreet monitoring of MCData SDS (using media plane)

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC user B is within the authority of authorized MC user A, and where MC user A and target MC user B have the same primary MC system. The scenario is illustrated in figure 4.7-1 below.

Figure 4.7-1: Discreet monitoring of one to one MCData SDS (using media plane)

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	Target MC user B sends or receives an MCData SDS data session request to or from MC user C.

-	MC system A provides MC service client A with the MCData SDS data session request, including meta data information.

-	If the recipient sends one or more disposition notifications, these are also sent to MC user A.

NOTE 1:	The scenario also applies to enhanced status sent over SDS.

NOTE 2:	If target MC user B is sending or receiving SDS within a group, the scenario is similar to that described in subclause 4.3 for MCPTT and MCVideo group call.

* * * End of Changes * * * *