3GPP TSG-SA WG6 Meeting #69	S6-254385

Wuhan, P.R. China, 13th – 17th October 2025	(revision of S6-254218)

Source:	Ericsson

Title:	Updates to Solution #1 on Enhancement of AIMLE to support model inference

Spec:	3GPP TR 23.700-83 v1.0.0

Agenda item:	9.4

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

The Solution #1 on Enhancement of AIMLE to support model inference, as defined in TR 23.700-83 v1.0.0, aims to provide support for ML model inference computed by the AIMLE/ADAE server. This service is utilized by the VAL server, which sends a request containing information about the ML model and the data to be used in the inference process. Currently, this solution specifies the VAL server as the sole consumer of ML model inference. However, it can be extended to generalize the consumer of the model inference.

Additionally, the ML model inference response information flow can be updated to improve its consistency. Specifically, the Information Element (IE) “Cause” is currently set as mandatory, yet it is followed by a note stating that “Only one of these information elements shall be present.” Therefore, the IE's status should be changed to optional to align with the note description. The second update is on the inference response information flow is to update the inference response from optional to mandatory.

Therefore, this pCR proposes two contributions: first, to enhance Solution #1 on AIMLE model inference by generalize the consumer of the inference service; and second, to update the information flow 7.1.3.2-2.

2. Reason for Change

To enhance the solution #1 of TR.23.700-83 v1.0.0 to include the AIMLE client as the consumer and to update the information flow 7.1.3.2-2.

3. Conclusions

This paper proposes to enhance the solution #1 of TR 23.700-83 v1.0.0 on Enhancement of AIMLE to support model inference

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.0.0.

* * * First Change * * * *

## 7.1	Solution #1: Enhancement of AIMLE to support model inference

### 7.1.1	Solution description

This clause describes the procedure for supporting the ML model inference by the AIMLE server. The  consumer (e.g. VAL server, AIMLE client) requests the AIMLE server for ML model inference by specifying the details about the ML model or requirements to select a ML model for inference.

Pre-condition:

-	The ML model needed to be used for this ML model inference is available by the AIMLE server.

Figure 7.1.1-1 illustrates the procedure for AIMLE server to support ML model inference based on the request from the consumer .

Figure 7.1.1-1: ML model inference

1.	The consumer (e.g. VAL server, AIMLE client) sends an ML model inference request to AIMLE server, requesting the inference result from the AIMLE server. This request consists of ML model information or ML model inference requirement information, etc.

2.	The AIMLE server checks whether the consumer is authorized to perform the ML model inference request. If no model information is provided but only the model inference requirement information is provided in step 1, the AIMLE server identifies and selects the appropriate ML model for inference based on the ML model inference requirement information using the procedure as specified in clause 8.11.3. The AIMLE server handles the request and execute the inference task.

3.	If the consumer is authorized, AIMLE server returns the inference response, otherwise a failure response indication the reason for failure.

### 7.1.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.1.3	Corresponding APIs

Table 7.1.3.1-1 describes the information flow from the consumer to the AIMLE server for requesting the ML model inference.

Table 7.1.3.1-1: ML model inference request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the consumer (e.g. identifier of the VAL server, AIMLE client or the VAL UE) performing the request. |
| Inference data (NOTE 1) | O | Original data for inference that may include text, images, audio, numerical data, etc. This IE may be included in the request in the scenario, e.g. the data is small. |
| Inference data index (NOTE 1) | O | An index of the data for inference. This IE may be included in the request in the scenario, e.g. the data is big, or stored in different servers. |
| Inference Parameters | O | Additional inference parameters that may be needed, such as maximum output length, temperature, batch size, etc., depending on the specific requirements of the model. |
| ML model information (NOTE 2) | O | Identifies the ML model that the consumer specified to perform inference. This information consists of the model identifier, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides |
| ML model inference requirement information (NOTE 2) | O | Identity the type or goal of the inference task, such as 'text classification,' 'image recognition,' 'sentiment analysis,' etc |
| NOTE 1: Only one of these IE shall be present. NOTE 2: At least one of these IE shall be present. |  |  |

Table 7.1.3.2-1 describes the information flow of ML model inference response from the AIMLE server to the consumer.

Table 7.1.3.2-2: ML model inference response

| Information element | Status | Description |
| --- | --- | --- |
| Inference response | M | Indicates that the ML model inference request was successful or not. |
| >Inference result (NOTE) | O | If the inference ML model inference request is successful, the Inference result is included in the response. |
| > ML model identifier | O | Identifies the ML model selected by AIMLE server for inference. |
| > Cause (NOTE) | O | If the inference ML model inference request is failed, the reason for the failure is included in the response. |
| NOTE: Only one of these information elements shall be present |  |  |

* * * End of Change * * * *