3GPP TSG-SA WG6 Meeting #69	S6-254520

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-254229

Source:	Ericsson

Title:	New Solution for KI#4 on Energy Saving in AI/ML Task Transfer

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

This pCR proposes a solution for the key issue #4.

2. Reason for Change

As described in TR 23.700-44, clause 5.4, there is an open issue on support for AIMLE enhancements for AIML services for energy saving, which includes the study aspects:

- Assistance in AI/ML task transfer

- How to collect and utilize energy consumption data (and which data) from the UE side to support an application layer ML task (or ML model lifecycle / workflow operation).

- How to enhance functionality and architecture of AIMLE to support energy saving.

- Whether and how to use energy consumption information to fulfill FL member selection, AIMLE client selection, split AI/ML operation, etc

As mentioned in the above open issues, there is a lack of solutions to support the AIMLE enhancements for AIML services for energy saving. A solution to address the key issue #4 on support for the AIMLE enhancements in AIML services for energy saving in AI/ML task transfer is proposed in this pCR. The application enablement service on ESE (Energy Saving Enablement) introduced in S6-254224 is considered for the sulution.

3. Conclusions

A solution for energy saving in AI/ML task transfer is proposed to address the key issue #4.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[x]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

* * * Next Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |
| Sol #y |  |  |  | X |  |  |

* * * Next Change * * * *

## 6.y	Solution #y: Energy saving in AI/ML task transfer

This solution corresponds to the key issue #4 described in TR 23.700-44, on energy saving in AI/ML task transfer.

### 6.y.1	Solution Description

According to the key issue #4, there is a need for AIMLE enhancements for AIML services such as AI/ML task transfer. This solution enables the AIMLE Server to select the target AI/ML member(s) for the AI/ML task transfer, based on energy consumption related information of the target AI/ML member(s) and the energy saving assistance information from the ESE server.

#### 6.y.1.1	Energy saving in AI/ML task transfer procedure

Figure 6.y.1.1-1: Energy saving support for requesting AIMLE server assist AI/ML task transfer

Figure 6.y.1.1-1 illustrates the procedure for requesting AIMLE Server to assist the energy saving for AI/ML task transfer. The detailed corresponding procedure is detailed as follows:

1.	The source AI/ML member (e.g., an AIMLE Client) sends an AI/ML task transfer assist request to AIMLE Server for assisting with the AI/ML task transfer based on energy saving. To select a green target AI/ML member(s), the source AI/ML member set energy consumption criteria for the target AI/ML member(s) such as energy flag, energy consumption level, AIML target traffic volume, battery level, and maximum energy cost.

2.	The AIMLE Server discovers other AI/ML members (e.g., other AIMLE Clients) directly or via ML repository, and selects one or more target AI/ML member(s) based on the energy criteria and other requirements from the source AI/ML member.

3.	The AIMLE Server interacts with the ESE server for the energy saving assistance information for the list of target AI/ML member(s) that is selected in step 2. The ESE server generates energy saving assistance information for the list of target AI/ML member(s) and provides it to the AIMLE Server.

4.	The AIMLE Server collects the energy saving related information for the requested target AI/ML member(s) and selects the most energy efficient AI/ML member (green AI/ML member) as the target AI/ML member based on the collected information from the ESE server.

The AIMLE Server sends request to the selected green target AI/ML member(s) for the AI/ML task transfer. Then, the AIMLE Server determines the transfer mode, i.e., transfer with server-controlled or transfer directly from the source AI/ML member to the target AI/ML member. For example, if the task is model training, the AIMLE Server determines the target AI/ML member(s) based on AI/ML member(s)’s capability in the AIMLE client profile, task's AI/ML model training requirement, and the energy status. The task’s AI/ML model training requirement indicates the AI/ML Information.

If target AI/ML member(s) is selected, the AIMLE Server calculate the total energy cost for the AI/ML task transfer, and generates assistance information for the AI/ML task transfer from the source AI/ML member to the target AI/ML member(s).

5.	The AIMLE Server sends an AI/ML task transfer assist response to the source AI/ML member with the information on the green target AI/ML member(s), the assistance information, and the energy cost generated in step 4.

The information flow for AI/ML task transfer as specified in clause 8.6.3 of 3GPP TS 23.482[x] is enhanced as follows (new text in bold italics):

#### 8.6.3.2 AI/ML task transfer assist request Table 8.6.3.2-1 describes information elements for the AI/ML task transfer assist request from the source AI/ML member to the AIMLE Server. Table 8.6.3.2-1: AI/ML task transfer assist request Information element; Status; Description. Requestor identity; M; The identifier of the source AI/ML member (e.g. identifier of the source AIMLE client or the VAL UE).. VAL service ID; O; The identifier of the VAL service for which the assistance information is requested.. AI/ML Task Type; M; The type of the AI/ML operation (e.g. ML model training).. AI/ML Information Type; M; The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results).. > AI/ML model remaining training requirement (NOTE); O; Indicate the requirement for AI/ML model training including, required remaining training resource, required remaining training number of iterations,. > AI/ML intermediate information (NOTE); O; Indicate the AI/ML intermediate information for intermediate AI/ML operation and result (e.g. ML model training), including AI/ML intermediate model, AI/ML intermediate model used training time, used training resource, used training number of iterations.. Target AI/ML member energy criterion; M; Specifies the energy consumption criteria (e.g., energy flag, energy consumption level, AIML target traffic volume, battery level, max energy cost) of the target AI/ML member.. Time validity; O; The time validity of the request.. NOTE: The IE only present when AI/ML Task Type is ML model training.. #### 8.6.3.3 AI/ML task transfer assist response Table 8.6.3.3-1 describes information elements for the AI/ML task transfer assist response from the AIMLE Server to the source AI/ML member. Table 8.6.3.3-1: AI/ML task transfer assist response Information element; Status; Description. Transfer Mode; O; Indication the transfer mode (e.g. direct transfer).. Target AI/ML member(s); M; The identifier of the target AI/ML member (e.g. identifier of the target AIMLE client or the VAL UE).. Target AI/ML member energy report; M; Specifies the energy result (e.g., energy flag, energy consuming result, AIML target traffic volume, battery level, energy cost) of the target AI/ML member.. >Energy Log; O; Indicates the energy consumption logging per task or step.. Assistance Information; M; Assistance information for the AI/ML task transfer (e.g. time window for the transfer)..

### 6.y.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.y.3	Solution evaluation

Editor’s Note:	The solution evaluation of the solution is FFS.

* * * End of Changes * * * *