3GPP TSG-SA WG6 Meeting #69	S6-254607

Wuhan, China 13th – 17th October 2025	(revision of S6-254323, S6-253371)

Source:	Samsung

Title:	Pseudo-CR Solution for location service via satellite access

Spec:	3GPP TR 23.700-02 V0.1.0

Agenda item:	9.12

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

This pCR proposes solution for location management service via satellite access Key issue#5.

2. Reason for Change

Solution proposal to key issue#5.

## 4.5 Key issue #5: Location management service via satellite access ### 4.5.1 Description 3GPP TS 23.434 [4] describes Location Management service that offers the location management related capabilities to one or more vertical applications. The location management server is a functional entity that receives and stores user location information and provides user location information to the vertical application server. The location management server, with the fused location function, may combine/aggregate location information from multiple sources to provide a more accurate UE location. The location management server also supports to report the value-added location information (e.g. monitoring location deviation events, history location data, periodic verify UE location, prediction related to UE location, etc.) to the VAL server. In a network with satellite access, the Location management service works as expected when both the service link and feeder link are available, and on par with the location management service with terrestrial access. However, when either of them (service link or feeder link) is not available, the Location management service is expected to have interruption that may have direct impact on the behaviour of the application. Particularly, features such as monitoring location deviation events, history location data, periodic verify UE location does not work when such interruption happens. ### 4.5.2 Open issues To support the location management service with satellite service, the following aspects need to be studied: - How to support history location data, monitoring location deviation events, periodic verify UE location when the UE is connected via satellite access, during the loss of service link or feeder link?

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-02 V0.1.0.

* * * First Change * * * *

## X.Y	Solution #Y: Location management service via satellite access

### X.Y.0	General description

Figure X.Y.0-1, illustrates the deployment of location enabler on-board satellite for assisting location management service.

Figure X.Y.0-1: Deployment of location enabler on-board satellite for assisting location management service

Basically, in this architecture option, SEAL location management server is deployed on-board satellite (GEO, MEO, or LEO) to complement and for continuity of the location management service, especially when the feeder link is not available and S&F is supported.

In this deployment option, location management server is deployed both on ground and on-board satellite. The UE can be on the ground/sea or in the air (e.g. drone). The UPF to access on-board location management server is deployed on one or more satellites. RAN (e.g. gNB) can either be deployed on ground/sea (e.g. in a ship) and connected to satellite UPF or be deployed on regenerative satellite. The 5GS control plane functions (e.g. AMF, SMF) are deployed on the ground and/or on satellite, which is not depicted in the figure for simplicity. UPF is deployed on ground to access the location management server on the ground, and the UE can reach the location management server on ground or via space. The on-board location management server can be mobile depending on the satellite they are deployed on: GEO, MEO, or LEO.

The location management server, with the fused location function, may combine/aggregate location information from multiple sources including the location determined on-board satellite to provide a more accurate UE location. The location management server on-board satellite also assists to report the value-added location information (e.g. monitoring location deviation events, history location data, periodic verify UE location, prediction related to UE location, etc.) to the VAL server via satellite access.

### X.Y.1	Solution description

Figure X.Y.1-1 illustrates the procedure for on-board satellite location management server assisting SEAL location history procedure specified in 3GPP TS 23.434.

Pre-conditions:

1.	The location management server on-board satellite is deployed to assist the value-added location information.

Figure X.Y.1-1: Procedure for on-board satellite LMS assisted SEAL location history service

1.	Location tracing configuration from VAL server to the location management server (on-ground) as described in 3GPP TS 23.434 clause 9.3.21.1.

2.	The location management server (on-ground) sends a location tracing configuration request to the location management server (on-board satellite) to configure the location tracing service while the feeder link is available. The location tracing configuration request may contain the geographical area for history location tracing, events/conditions for triggering the location history report, location QoS, the identities for the UE and service, the time till when the SEAL LM needs to store the location history data and the required positioning method(s) , etc.

Editor's note:	How the target satellite to configure the location tracing service is determined is FFS.

3.	The location management server (on-board satellite) checks whether the location tracing configuration request is authorized and available to serve. If the request is available, the location management server (on-board satellite) stores the configuration received in step 2. If the request includes multiple UEs, the location management server creates and stores the contexts for each UE.

4.	The location management server (on-board satellite) sends the location tracing configuration responses to the location management server (on-ground).

5.	When the feeder link is not available, the location management server (on-board satellite) has obtained and stored the UE location data from multiple sources (e.g. UE, gNB/GMLC onboard, 3rd party), periodically based on the event triggers or conditions received in the request of step 2.

6.	Once the feeder link is available, the location management server (on-board satellite) forwards the stored UE location data towards the location management server (on-ground).

NOTE:	The interactions between the location management server (on-board satellite) and location management server (on-ground) are supported by LM-E reference point as described in 3GPP TS 23.434 clause 9.2.5.6.

### X.Y.2	Architecture Impacts

SEAL location management server is expected to be deployed on-board satellite. And the SEAL-E interface LM-E is required to support interaction between LMS on-ground to LMS on-board satellite.

### X.Y.3	Corresponding APIs

This solution impacts the SEAL-E interface LM-E with new interactions between LMS on-ground to LMS on-board satellite to support SEAL location history service via satellite access.

### X.Y.4	Solution evaluation

This solution provides a proposal to address the open issues listed in key issue#5 by supporting location management service via satellite access. Architecture impacts are identified where location management server is required to be deployed on-board satellite. SEAL-E interface LM-E is required with new interactions between LMS on-ground to LMS on-board satellite to support SEAL location history service via satellite access.

Editor's note:	It is FFS to evaluate further on the impact made by this solution to the use case.

* * * End of Changes * * * *