3GPP TSG-SA WG6 Meeting #69	S6-254722

Wuhan, China 13th – 17th October 2025	(revision of S6-254296)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0079 | rev | 1 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial Anchor Group Retrieve operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Procedures related to Spatial anchors group retrieval and retrieval of spatial anchors associated with the groups are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the procedures and information flows for the spatial anchor group retrieval and retrieval of spatial anchors associated with group. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spatial anchor group related procedures are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (All new) 8.3.7.2.x, 8.3.7.2.y, 8.3.7.3.x, 8.3.7.3.y, 8.3.7.3.a, 8.3.7.3.b, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 8.3.7.2.x	Spatial anchor group retrieve

Figure 8.3.7.2.x-1 illustrates the spatial anchor group retrieve procedure through which the requestor SAn client or the VAL server can retrieve the spatial anchor group from SAn server.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.7.2.x-1: Spatial anchor group retrieve

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor group retrieve request to the SAn server. The request as includes a requestor identifier, security credentials, identifier of the spatial anchor group.

2.	Upon receiving the request, the SAn server validates if the spatial anchor group exists and if the requestor is authorized to retrieve the spatial anchor group. If the requestor is authorized, the SAn server retrieves the requested spatial anchor group information.

3.	The SAn server sends a spatial anchor group retrieve response including the retrieved spatial anchor group information to the requestor. The SAn server includes the spatial anchor profiles if the spatial anchor group retrieve request indicates to include the profiles of the spatial anchors belonging to the group.

# * * * * Next change * * * *

##### 8.3.7.2.y	Retreive spatial anchors of group

Figure 8.3.7.2.y-1 illustrates the retrieve spatial anchors of group procedure through which the requestor SAn client or the VAL server can retrieve the spatial anchor associated with a particular spatial anchor group.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.7.2.y-1: Retrieve spatial anchors of group

1.	The requestor (e.g., SAn client or VAL server) sends a Retrieve spatial anchors of group request to the SAn server. The request includes a requestor identifier, security credentials and identifier of the spatial anchor group.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the retrieving the spatial anchors associated with the spatial anchor group. If the requestor is authorized and if the spatial anchor group exists, the SAn server retrieves the list of spatial anchors associated with the spatial anchor group received in the request and its information stored in its repository.

3.	The SAn server sends a Retrieve spatial anchors of group response to the requestor. If its success response, it includes the list of spatial anchor IDs and its information and if its a failure response it includes the reason for the failure.

# * * * * Next change * * * *

##### 8.3.7.3.x	Spatial anchor group retrieve request

Table 8.3.7.3.x-1 describes information elements for the spatial anchor group retrieve request from the SAn client or VAL server to the SAn server.

Table 8.3.7.3.x-1: Spatial anchor group retrieve request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| Spatial anchor group ID | M | Identify of the spatial anchor group |
| Include spatial anchor profile(s) | O | Indicates whether the response shall include the spatial anchor profiles. |

##### 8.3.7.3.y	Spatial anchor group retrieve response

Table 8.3.7.3.y-1 describes information elements for the spatial anchor group retrieve response sent by the SAn server to the SAn client or VAL server.

Table 8.3.7.3.y-1: Spatial anchor group retrieve response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the spatial anchor group retrieve request was successful. |
| > Spatial anchor group information | M | Identifes the contents of requested spatial anchor group information |
| >> Spatial anchor group ID | M | Identity of the spatial anchor group |
| >>> Group description | O | Human readable description about the spatial anchor group (e.g. List of restaurants) |
| >>> Service area | O | Service area where the spatial anchor group is visible or accessible. |
| >>> Service ID | M | The identifier of the application service associated with the spatial anchor group |
| >>> Access control rules | O | Access control rules defining which entities are permitted to discover the spatial anchor group. |
| >>> List of spatial anchors IDs | M | List of spatial anchors IDs to be associated with the group. |
| >>> List of spatial anchor profiles | O | Contains the list spatial anchors profile information as specified in clause 7.3.2.1-1. This is present only if the request indicates to include the spatial anchor profiles. |
| Failure response | O (NOTE 1) | Indicates that the spatial anchor group retrieve request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE : One of the IEs shall be present. |  |  |

##### 8.3.7.3.a	Retrieve spatial anchors of group request

Table 8.3.7.3.a-1 describes information elements for the retrieve spatial anchors of group request from the SAn client or VAL server to the SAn server.

Table 8.3.7.3.a-1: Retrieve Spatial anchors of group request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| Spatial anchor group ID | M | Identify of the spatial anchor group |

##### 8.3.7.3.b	Retrieve Spatial anchors of group response

Table 8.3.7.3.b-1 describes information elements for the retrieve Spatial anchors of group response sent by the SAn server to the SAn client or VAL server.

Table 8.3.7.3.b-1: Retrieve Spatial anchors of group response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the retrieve spatial anchors of group request was successful. |
| > List of spatial anchor profiles | M | Contains the list spatial anchors profile information as specified in clause 7.3.2.1-1 |
| Failure response | O (NOTE) | Indicates that the retrieve spatial anchors of group request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: One of the IEs shall be present. |  |  |

# * * * * End change * * * *