3GPP TSG-SA WG6 Meeting #69	S6-254753

Wuhan, China 13th – 17th October 2025	(revision of S6-254070,S6-254518)

Source:	InterDigital

Title:	Pseudo-CR on New solution for AIMLE client selection based on energy consumption

Spec:	3GPP TR 23.700-44

Agenda item:	9.9

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This pCR proposes a new solution for KI#4 on AIMLE enhancements for AIML services energy saving.

2. Reason for Change

This contribution proposes a solution to Key Issue #4 – open issue #3 on how to use energy consumption information for AIMLE client selection.

The solution proposes enhancements to the ML model information storage procedure and to procedures involving AIMLE client selection to enable the storage and use of energy consumption information for AIMLE client selection.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[r23482]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

* * * Next Change * * * *

## 6.x	Solution #x: Support of AIMLE client selection based on energy consumption

### 6.x.1	Solution Description

This solution addresses Key Issue #4 – open issue #3 on how to use energy consumption information for AIMLE client selection. The solution proposes enhancements to the ML model information management procedure, and to procedures involving AIMLE client selection, to enable the storage and use of energy consumption information for AIMLE client selection.

The enhancements enable an AIMLE client to provide energy consumption capabilities during registration. Energy consumption capabilities include the maximum energy budget for performing AIML operations, and the VAL UE’s power profile. The VAL UE power profile indicates the degree of energy consumption of a VAL UE for performing AI/ML operations.

The enhancements also enable an ML model provider to include energy consumption information during ML model storage. The energy consumption information includes expected energy consumption values for performing AIML operations using an ML model. The expected energy consumption values are provided per power profile.

Both the AIMLE energy budget and the ML model energy consumption information are used for AIMLE client selection (e.g., request or subscription). During AIMLE client selection, the AIMLE server receives energy consumption requirements from a requestor. The energy consumption requirements include an indication to consider energy consumption during AIMLE client selection, and the maximum total energy consumption that is acceptable to the requestor. The AIMLE server uses the energy budget and energy consumption information to identify and select the ML models and AIMLE clients that meet the energy consumption requirements of the requestor.

#### 6.x.1.1	Impact to ML model information management

The ML model information management procedure and information flows in 3GPP TS 23.482 [r23482] are enhanced (highlighted in bold italics) as follows.

#### 8.11.4.1 ML model information storage request Table 8.11.4.1-1 describes the information flow from the AIMLE server to the ML repository or from the AIMLE client/VAL server to the AIMLE server as a request for the ML model information storage. Table 8.11.4.1-1: ML model information storage request Information element; Status; Description. Requestor Identity; M; The identity of the requestor performing the request.. Security Credentials; O (NOTE 1); Security credentials of the requestor performing the request.. ML model; O (NOTE 2); The ML model to be stored in the ML repository.. ML model address; O (NOTE 2); The address (e.g., a URL or an FQDN) of the ML model file.. ML model information; M; Provides information of the ML model, as described in Table 8.11.4.1-2.. NOTE 1: This information is needed if the requestor is at the VAL service provider domain. NOTE 2: At least one of these information elements shall be provided.. Table 8.11.4.1-2: ML model information Information element; Status; Description. ML model identifier; M; An identifier for the ML model. ADAE Analytics ID; O; Represents ADAE analytics ID for which the model can be used.. ML model size; O; Indicates the size of the ML model.. ML model source identifier; O; The identifier of ML model source (e.g., VAL server ID, VAL client ID) that stored the model in the ML repository.. VAL service ID(s); O; Identify the VAL service ID(s).. Domain; O; Specifies domain for which the model can be used (e.g., for speech recognition, image recognition, video processing, location prediction, etc.).. List of allowed vendors; O (NOTE 1); Indicates which vendors that are allowed to use the ML model and thereby also are interoperable to the model.. ML model interoperability information; O (NOTE 1); Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, input/output parameters of the ML model, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes.. ML Model phase; O (NOTE 1); Represents the ML model phase, e.g., in training, trained, re-training, deployed.. > Observed performance; O (NOTE 2); Provides information on the performance of the model e.g. accuracy, or application-specific performance metrics (if ML model is in trained or deployment phase).. > Training information; O (NOTE 2); If the ML model is in trained or deployed phase: Information on the data that has been used to train the model (e.g. data sources, volume, freshness), and the base model ID in case of Transfer Learning.. > Indication of continuous model training; O; Indicates whether the model can be continuously trained or not.. > Continuous model training parameter; O; Parameters required for continuous model training.. ML model storage and discovery requirements; O (NOTE 1); Represents the requirements for the ML repository for the ML model storage and discovery.. > Storage duration; O; Represents the ML model storage duration time. When the storage duration time is expired, the stored ML model and the related information shall be deleted.. > Security and access requirements; O; Represents the information on security requirements for storing the ML model information and the ML model access requirements (e.g., publicly available, private use only, or available for the list of VAL server IDs or VAL client IDs, time period and location access). If the access requirement is private use only, then the model is not discoverable by other consumers.. ML model usage requirements; O; Represents the requirements for using the ML model (e.g. for inference or for training). The requirements are used by the AIMLE server to determine whether an AIMLE client is capable of using the model based on comparing the requirements with information in the AIMLE client profile in Table 8.7.3.1-2.. Energy consumption information; O; Energy consumption information associated with the ML model.. > List of AI/ML operations; M; The AI/ML operations (e.g., ML model training, model transfer, model inference, model offload and model split).. >> Expected energy budget; M; The expected energy budget (e.g., watt-hour, joules) associated with performing an AI/ML operation. The expected energy budget relates energy consumption values to a VAL UE power profile as described in Table 8.7.3.2-2.. NOTE 1: At least one of these information elements shall be provided. NOTE 2: This IE is included only if trained ML model is available..

#### 6.x.1.2	Impact to AIMLE client registration

The AIMLE client registration procedure and information flows in 3GPP TS 23.482 [r23482] are enhanced (highlighted in bold italics) as follows.

#### 8.7.2.2 AIMLE client registration Pre-conditions: 1. The AIMLE client has been pre-configured or has discovered the address (e.g., URI) of the AIMLE server. 2. The AIMLE client has been pre-configured with an AIMLE client profile. Figure 8.7.2.2-1: AIMLE client registration 1. The AIMLE client sends an AIMLE client registration request to the AIMLE server, the registration request includes information as described in Table 8.7.3.2-1. The AIMLE client indicates in the registration request its AI/ML capabilities such as supported ML model types and supported AI/ML operations, supported AIMLE client task capabilities with compute and task performance capabilities, and energy consumption capabilities to assist with performing AIMLE client discovery and AIMLE client selection. 2. The AIMLE server validates the registration request and performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. Upon successful authorization, the AIMLE server saves the context of the AIMLE client registration in the ML repository. 3. The AIMLE server returns an AIMLE client registration response to the AIMLE client with the status of the request. #### 8.7.3.2 AIMLE client registration request Table 8.7.3.2-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration request. Table 8.7.3.2-1: AIMLE client registration request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. List of supported profiles; M; Supported AIML client profile(s). For each client profile provided in the list, the supported service information is provided.. > AIMLE client profile; M; Information about the capability of the AIMLE client to support AI/ML operations for the VAL service ID as detailed in Table 8.7.3.2-2.. > List of supported services; M; List of VAL service IDs with corresponding permissions.. > VAL service ID; M; The identifier of the VAL service.. > Service permission level; O; Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage).. Table 8.7.3.2-2: AIMLE client profile Information element; Status; Description. Supported AI/ML model types; O; AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks).. Supported AI/ML operations; M; AI/ML operations supported by the AIMLE client such as ML model training, model transfer, model inference, model offload, model split, continue perform intermediate AI/ML operation/task).. AIMLE client time schedule configurations; O; Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations (e.g. ML model training) in the given time slot(s) and/or day(s) of the week.. AIMLE client location configurations; O; Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID.. AIMLE client capabilities; M; AIMLE client capability information (e.g. ML application type, allowed resource usage level).. Dataset availability; O; Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers.. Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions.. AIMLE client task capability; O (NOTE 1); Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs). Energy consumption capabilities; O; The energy consumption capabilities of the AIMLE client.. > Maximum energy budget; O (NOTE 2); The maximum energy budget (e.g., watt-hour, joules) permitted for performing AI/ML operations.. > Power profile; O (NOTE 2); The power rating of the VAL UE that represents the VAL UE’s processing capabilities (e.g., entry-level, mid-tier, premium, high performance, high efficiency). The power rating is used to determine the usage rate of energy per unit time that factors into the determination of energy consumption and is static.. NOTE 1: The Green and Energy-efficient task performance may not be applicable to a UE. NOTE 2: At least one of these information elements shall be present..

#### 6.x.1.3	Impact to AIMLE client discovery

The AIMLE client selection procedure and information flows in 3GPP TS 23.482 [r23482] are enhanced (highlighted in bold italics) as follows.

#### 8.8.3.1 AIMLE client discovery request Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure. Table 8.8.3.1-1: AIMLE client discovery request Information element; Status; Description. Requestor identity; M; The identifier of the requestor (e.g., VAL server).. AIMLE client discovery criteria; M; Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2.. Number of required AIMLE clients; O; Indicates the requested number of AIMLE clients to be discovered based on the discovery criteria.. Table 8.8.3.1-2: AIMLE client discovery criteria Information element; Status; Description. Service requirement; M; Information about the required service. > VAL Service ID; M; VAL Service ID that the client is required to support. This identifies the service associated with the requester.. > Service permission level; O; Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage).. Requested ML model types; O; Requested ML model types (decision trees, linear regression, neural networks).. Requested AIML operations; M; Requested role for AI/ML operations such as model training, model transfer, model inference, model offload, model split.. Application layer AIMLE client capabilities; M; Application layer AIMLE client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate).. Dataset requirements; O; Information about dataset.. > Dataset availability; O; Dataset availability, including dataset identifiers, dataset size, age, list of dataset features.. > Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions.. AIML client task capability requirements; O; Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g. Green task, Energy-efficient, low costs). AIMLE client velocity; O; Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static.. Location information; O; Indicates the location information (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses, VAL service area ID) to discover the AIMLE clients.. AIMLE client QoS requirements; O; Indicates the AIMLE client QoS information (like PLR, bandwidth, latency jitter) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above or below) to discover the AIMLE clients.. Energy consumption requirements; O (NOTE 1); The energy consumption requirements of the requestor.. > Maximum energy budget; M; The maximum energy budget (e.g., watt-hour, joules) that is acceptable to the requestor.. NOTE 1: This information element applies to AIMLE client selection..

#### 6.x.1.4	Impact to AIMLE client selection

The AIMLE client selection procedure and information flows in 3GPP TS 23.482 [r23482] are enhanced (highlighted in bold italics) as follows.

#### 8.9.2.1 AIMLE client selection Pre-conditions: 1. AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services. 2. The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with the AIMLE clients. Figure 8.9.2.1-1: AIMLE client selection 1. A VAL server sends an AIMLE client selection request to an AIMLE server to select AIMLE clients available for participation in AI/ML operations (e.g., is available and have required data to train an ML model). The AIMLE client selection request includes information as described in Table 8.9.3.1-1. 2. The AIMLE server performs authentication and authorization checks to determine if the requestor is able to select AIMLE clients. 3. If the requestor is authorized, the AIMLE server performs AIMLE client selection based on the provided inputs as described below. For VAL server selection, the AIMLE server receives a list of AIMLE client IDs in the request and selects the AIMLE clients in the list as candidate AIMLE clients. For AIMLE server selection, the AIMLE server retrieves a list of clients with AI/ML capabilities from the ML repository. From the list of clients, the AIMLE server selects a list of candidate AIMLE clients, whose client profiles fulfil the AIMLE client selection criteria. The AIMLE server may use SEAL (LM service), NEF (e.g. MonitoringEvent API), and NWDAF (e.g. UE mobility analytics as defined in clause 6.7.2 of 3GPP TS 23.288 [2]) capabilities to assist the AIMLE client selection. The AIMLE server determines UEs by using their identifiers to determine their location and then selects only those that fulfil the location requirements specified in the AIMLE client selection criteria. The AIMLE server may use SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.9, or 3GPP TS 23.434 [5] clause 9.3.10) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to determine the UEs which fulfil the location requirement. The AIMLE server may then determine the QoS parameters for the AIML traffic session between the requestor and the candidate AIMLE client(s) and configure the AI/ML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API). The AIMLE server determines the application QoS parameters based on the VAL Service ID. NOTE 1: To determine the application QoS parameters based on the VAL service ID, the AIMLE server can use the VAL service ID associated AIMLE client QoS requirements in the AIMLE client selection criteria provided in step 1. If the AIMLE client selection criteria include energy consumption requirements, the AIMLE server retrieves a list of ML model information from the ML repository using the ML model information discovery procedure as described in clause 8.11.3. The AIMLE server uses information from the AIMLE client selection criteria as filtering criteria for ML model information discovery. If energy consumption requirements are included, the AIMLE server selects the candidate AIMLE clients that fulfil the expected energy consumption needs for the retrieved ML models. The expected energy consumption needs are based on the requested AI/ML operations and on the AIMLE client’s power profile. If the energy consumption requirements include a maximum energy budget, the AIMLE server identifies and selects the ML models and candidate AIMLE clients that can perform the requested AI/ML operations without exceeding the maximum energy budget. NOTE 2: If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server can discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the AIMLE client selection response message. NOTE 3: The AIMLE server can reuse SEAL group management for any necessary group management. 4. The AIMLE server performs AIMLE client participation procedure with each candidate AIMLE client as described in clause 8.10. For all candidate AIMLE clients that agreed to participate in AI/ML operations, the AIMLE server selects the AIMLE clients and assigns an AIMLE client set identifier for the selected clients. The AIMLE client set may then be used for training a ML model. NOTE 4: If a required minimum number of AIMLE clients for AI/ML operation is provided in the AIMLE client selection request and the number of AIMLE clients that agreed to participate in AI/ML operations is less than this number, an AIMLE client set identifier will not be assigned and the status will be set to fail in the AIMLE client selection response in step 5. 5. The AIMLE server sends an AIMLE client selection response that includes information in Table 8.9.3.2-1.

#### 6.x.1.5	Impact to AIMLE client selection subscription

The AIMLE client selection subscription procedure and information flows in 3GPP TS 23.482 [r23482] are enhanced (highlighted in bold italics) as follows.

#### 8.13.2.2 AIMLE client selection subscription and notification Figure 8.13.2.2-1: AIMLE client selection subscription and notification 1. A VAL server sends an AIMLE client selection subscription request to the AIMLE Server. The AIMLE client selection subscription request includes information as described in Table 8.13.3-1 which includes selection criteria. 2. The AIMLE server validates the AIMLE client selection subscription request. The AIMLE server further performs authentication and authorization checks to determine if the requestor is able to subscribe to the selected AIMLE client selection subscription request. 3. The AIMLE server sends the AIMLE client selection subscription response to the VAL server. 4. The AIMLE server monitors AIMLE clients whether they fulfil the selection criteria as provided in step 1. The AIMLE server interacts with the NEF and/or SEAL services (including SEALDD) to establish monitoring. The AIMLE server utilizes SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.11, or 3GPP TS 23.434 [5] clause 9.3.12) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to establish monitoring of UEs entering or present in the target location provided in the location information in the selection criteria. 5. The AIMLE Server obtains the identifiers of the AIMLE clients from the monitoring and selects the clients that fulfil the selection criteria and remove the AIMLE clients which do not fulfil the selection criteria. The AIMLE server uses the location monitoring for selecting UEs that fulfil the location criteria and removing UEs which cease to fulfil the location criteria as provided in the location information in the AIMLE client selection criteria. The AIMLE Server may determine the application QoS parameters (e.g. bandwidth, latency, jitter) for the AIML traffic session between the VAL server and the selected AIMLE client and configure the AIML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API). When the AIMLE clients no more meet the criteria, the QoS adjustment is reversed. The AIMLE server may determine the application QoS parameters based on the VAL Service ID. If the AIMLE client selection criteria include energy consumption requirements, the AIMLE server retrieves a list of ML model information from the ML repository using the ML model information discovery procedure as described in clause 8.11.3. The AIMLE server uses information from the AIMLE client selection criteria as filtering criteria for ML model information discovery. If energy consumption requirements are included, the AIMLE server selects the candidate AIMLE clients that can fulfil the expected energy consumption needs of the ML model. The expected energy consumption needs are based on the requested AI/ML operations and on the AIMLE client’s power profile. If the energy consumption requirements include a maximum energy budget, the AIMLE server identifies and selects the ML models and candidate AIMLE clients that can perform the requested AI/ML operations without exceeding the maximum energy budget. If a desired service in the selection criteria is ceased to be provided by the client or its profile change so that it no longer meets the selection criteria, the AIMLE server removes the AIMLE clients which ceases to fulfil the criteria and selects other clients that fulfil selection criteria. NOTE 1: To determine the application QoS parameters based on the VAL service ID, the AIMLE server can use the VAL service ID associated AIMLE client QoS requirements in the AIMLE client selection criteria provided in step 1. NOTE 2: In case of AIMLE server determining the application QoS parameters based on the VAL service ID, how it does this is up to implementation. 6. The AIMLE Server notifies the VAL server about the selected and re-selected AIMLE clients e.g., the AIMLE Client A is re-selected and replaced by AIMLE Client B.

#### 6.x.1.6	Impact to AIMLE client participation

The AIMLE client participation procedure and information flows in 3GPP TS 23.482 [r23482] are enhanced (highlighted in bold italics) as follows.

#### 8.10.3.1 AIMLE client participation request Table 8.10.3.1-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation procedure. Table 8.10.3.1-1: AIMLE client participation request Information element; Status; Description. Requestor identity; M; The identifier of the requestor.. AIMLE client set identifier; M; An identifier for the AIMLE client set.. Add/remove indicator; M; Indicator for adding/removing the AIMLE client to/from the AIMLE client set.. ML model ID; M; The ML model ID to use for the AI/ML operation.. AI/ML operations; M; A list of AI/ML operations (e.g., training) required to be performed.. Expected energy budget; O; The expected energy budget (e.g., watt-hour, joules) associated with performing the AI/ML operations.. Operational schedule; O; Schedule for the AI/ML operations.. Dataset requirement; M; Dataset requirements for the AI/ML operations. Requirements includes dataset identifier, dataset size and age, and/or dataset features.. Service requirement; M; Information about the required Service including its VAL service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage).

### 6.x.2	Architecture impacts

This clause will provide the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 6.x.3	Solution evaluation

This clause will provide an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Changes * * * *