3GPP TSG-SA WG6 Meeting #69	S6-254779

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-254735

Source:	Ericsson, ETRI, CATT, InterDigital, TNO, KPN.N.V, China Mobile, Nokia, ZTE

Title:	New Solution for KI#1 on Functional Architecture

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #1 (TR 23.700-15 clause 4.1), the study aspects on application enablement architecture enhancements to support utilization and exposure of sensing results include:

- Whether and how to enhance the application enablement architecture and related functions to support the mapping of sensing service requirements of vertical applications.

- Whether and how the existing exposure mechanisms (e.g., subscription, notification, etc) defined in the SEAL architecture and functions are sufficient to meet vertical application requirements.

- Whether and how to process the collected sensing results (e.g., sensing exposure from the core network), to generate application enabler layer enhanced sensing results, and to expose them to vertical applications.

- Whether and how to resolve conflicts in sensing service requirements among different vertical applications or under resource limitations.

- Whether new application enabler architecture is needed to support the above functions.

There are some common requirements from difference sensing applications, for example, sensing data/result collection, sensing result aggregation, sensing result exposure. Enhancements to the application enablement layer architecture are needed to introduce common functionalities to fulfil these comment requirements.

Thus, this pCR proposes to enhance the application enablement layer architecture by introducing new enabler service to support sensing application relevant services.

2. Reason for Change

This pCR proposes a new solution for Key Issue #1, to enhance the application enablement layer architecture by introducing new enabler service to support sensing application relevant services.

3. Conclusions

This paper proposes solution on functional architecture to support sensing application relevant services.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #X | X |  |  |  |

* * * Next Change * * * *

## 6.X	Solution #X: Functional Architecture to Support Sensing Application Relevant Services

### 6.X.1	Solution Description

#### 6.X.1.0	General

There are some common requirements from difference sensing application relevant services, for example, sensing data/result collection, sensing result aggregation, sensing result exposure. Enhancements to the application enablement layer architecture are needed to introduce common functionalities to fulfil these comment requirements.

The following clauses specify generic functional model of SEAL Sensing Enabler for support application relevant sensing services.

#### 6.X.1.1	Functional Model

##### 6.X.1.1.1	General

The functional architecture enhancement for the SEAL Sensing Enabler is based on the generic functional model specified in clause 6.2 of 3GPP TS 23.434 [2]. It is organized into functional entities to describe an architecture enhancement which addresses the support for sensing aspects for vertical applications.

##### 6.X.1.1.2	Network Functional Model

Figure 6.X.1.1.2-1: Network functional model

Figure 6.X.1.1.2-1 illustrates the network functional model with SEAL Sensing Enabler. In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. The SEAL functional entity with sensing enabler function on the server is grouped into SEAL Sensing Enabler server. The SEAL Sensing Enabler server consists of a common set of services and reference points. The SEAL Sensing Enabler server offers its services to the vertical application layer (VAL), VAE/UAE, and other SEAL servers.

NOTE: In this release, the SEAL sensing client is not used for sensing data collection.

Editor’s Note:	Whether the SEAL sensing client is needed and the usage of it are FFS.

#### 6.X.1.2	Functional Entities Description

##### 6.X.1.2.1	General

The SEAL Sensing Enabler functional entities with sensing application relevant service functions are described in the following subclauses.

##### 6.X.1.2.2	SEAL Sensing Enabler client

The SEAL Sensing Enabler client interacts with the SEAL Sensing Enabler server.

NOTE: In this release, the SEAL sensing client is not used for sensing data collection.

##### 6.X.1.2.3	SEAL Sensing Enabler server

The SEAL Sensing Enabler server functional entity provides for sensing application relevant services supported within the vertical application layer.

#### 6.X.1.3	Reference Points Description

##### 6.X.1.3.1	General

The reference points for the functional model for sensing application relevant service are described in the following subclauses.

Editor’s Note:	The description of reference points is FFS.

### 6.X.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***