3GPP TSG-SA WG6 Meeting #69	S6-254790

Wuhan, China 13th – 17th October 2025	(revision of S6-254738, S6-254683, S6-254304)

Source:	China Mobile

Title:	Pseudo-CR on Solution on high level architecture and procedures for use of sensing results for spatial maps

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR provides the solution of KI#3 on use of sensing results for spatial maps.

This pCR proposes this new solution for TS23.700-15.

2. Reason for Change

This pCR provides the solution of KI#3 on use of sensing results for spatial maps.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

6.x	Solution #x: high level architecture and procedures for use of sensing results for spatial map

6.x.1	Description

This solution resolves solution of KI#3 on use of sensing results for spatial maps.

This solution  is based on following principles:

1.	No significant changes are made to the spatial map management procedures defined in 3GPP TS 23.437 [437];

2.	A separated SEAL Sensing service server is provided by Enabler layer and interacts with Spatial Map server via a new SEAL-X interference.

The functional model for the SEAL Sensing server is based on the generic functional model specified in clause 6 of 3GPP TS 23.434 [4] and the generic architecture of sensing service concluded in this study. The functional model enhanced to Spatial map for this solution is illustrated in Figure 6.x.1-1.

Figure 6.x.1-1: On-network spatial map functional model for use of sensing

NOTE:	In this solution, SEAL SM client/server in the architecture and procedures refer to SEAL Spatial Map client/server specified in 3GPP TS 23.437 [437].

This solution contains two basic procedures:

1.	Create spatial map with sensing capabilities.

2.	Spatial map subscription and notification with sensing capabilities.

6.x.2	Procedures

6.x.2.1	Create spatial map with sensing capabilities

Figure 6.x.2.1-1 below illustrates the procedure for creating a spatial map when sensing capabilities are introduced. For the request from VAL server or SEAL SM client, as a spatial map consumer. SEAL SM server creates a spatial map by utilizing sensing capabilities provided by SEAL Sensing server.

Pre-conditions:

1.	The VAL Server or SEAL SM client is authorized to use APIs provided by the SEAL SM server.

2.	The SEAL SM server can interact with SEAL Sensing server via SEAL-X interface.

3.	The SEAL Sensing server is authorized to use Sense ing related capability APIs provided by sensing related function in the 3GPP core network or authorized to use the NEF exposed Sense ing related capability.

NOTE:	In this solution, it is assumed that neither the VAL Server nor the SEAL SM Server has sufficient knowledge to understand the detailed use of sensing-related capabilities/parameters provided by the 3GPP core network, whether directly or via the NEF. Alternatively, If the VAL Server/SEAL SM Server knows that sensing service can be provided by SEAL sensing server, the VAL Server/SEAL SM Server may include a sensing indicator in the request. If so, the SEAL SM server will use the sensing indicator to decide whether to utilize sensing capabilities.

.

Figure 6.x.2.1-1: Create spatial map procedure

1.	The requester (e.g., VAL server or SEAL SM client) sends a Create Spatial Map Request message to the SEAL SM server to create a spatial map. The request includes the same IEs as specified in clause 9.3.1.3.1 of 3GPP TS 23.437 [437].

2.	The SEAL SM server decides to utilize sensing capabilities to assist the creation of the spatial map based on local policy. If SEAL SM server decides to utilize sensing capabilities, it sends a Get Sensing Results Request to SEAL Sensing server. The request includes requester ID, security credentials and three-dimensional area of interest.

Editor’s note:	the detailed IEs included in the Get Sensing Result Request is FFS.

3.	The SEAL Sensing server sends a Sensing Service Request either directly to a sensing-related function in the 3GPP core network or to this function via the NEF (e.g., using the Nnef_Sensing_Subscribe service operation). The request includes the Target Sensing Area information. This information is derived from a three-dimensional area of interest and can be handled by 3GPP core network.

Editor’s note:	the Sensing Service Request may be used to trigger the sensing service provided by 3GPP core network, or used to request the sensing result. How the SEAL sensing server utilizes the sensing service will be based on SA2 progress and is FFS.

4.	The sensing-related function in the 3GPP core network sends the Sensing Service Response either directly to the SEAL Sensing server or via the NEF. The response includes the sensing result (with or without the sensing contextual information).

Editor’s note:	the detailed IEs included in the step 3 and step 4 are based on the output of SA2 and are FFS.

5.	The SEAL Sensing server sends the Get Sensing Result Response to the SEAL SM server. The response includes the List of objects information in the Area of interest obtained from the sensing service.

6.	The SEAL SM server creates the spatial map as per step 2 in clause 9.3.1.2 of 3GPP TS 23.437 [437]. In this step, the processed sensor data may be sourced from step 5 instead of the VAL layer.

7.	The SEAL SM server sends response message to the requester as per step 3 in clause 9.3.1.2 of 3GPP TS 23.437 [437].

8.	If the response in step 7 indicates "in-progress", the SEAL SM server notifies the requester with the spatial map ready notification message as per step 4 in clause 9.3.1.2 of 3GPP TS 23.437 [437].

6.x.2.2	Spatial map subscription and notification with sensing capabilities

Figure 6.x.2.2-1 below illustrates the procedure for Spatial map subscription and notification with sensing capabilities. It is based on the Subscribe spatial map event specified in clause 9.3.6.2.1 of 3GPP TS 23.437 [437] and Notify spatial map event specified in clause 9.3.6.2.2 of 3GPP TS 23.437 [437].

Pre-conditions:

1.	The VAL Server or SEAL SM client is authorized to use APIs provided by the SEAL SM server.

2.	The SEAL SM server can interact with SEAL Sensing server via SEAL-Xsensing interface.

3.	The SEAL Sensing server is authorized to use Sense ing related capability APIs provided by sensing related function in the 3GPP core network or authorized to use the NEF exposed Sense ing related capability.

Figure 6.x.2.2-1: Create spatial map with periodic update notification procedure

The following step 1 to step 8 are Spatial map subscription procedure.

1.	The VAL server (or SEAL SM client) sends a Subscribe Spatial Map Event Request to the SEAL SM server to subscribe spatial map event.. The request includes the same IEs as specified in clause 9.3.6.2.1 of 3GPP TS 23.437 [437].

2.	The SEAL SM server processes the Subscribe Spatial Map Event Request as per step 2 in clause 9.3.6.2.1 of 3GPP TS 23.437 [437]. If the "Change of objects event" is subscribed by requester (VAL server or SEAL SM client), the SEAL SM server may subscribe the Sensing Results from SEAL Sensing server.

3.	The SEAL SM server sends a Get Sensing Results Subscription Request to SEAL Sensing server. The request includes requester ID, security credentials, and may include one or more IEs in the Target spatial map(s) and/or List of events.

Editor’s note 1:	the detailed IEs included in the Get Sensing Result Request is FFS.

4.	The SEAL Sensing server processes the Sensing subscription for this Get Sensing Results Subscription Request.

5.	The SEAL Sensing server sends a Sensing Service Request either directly to a sensing-related function in the 3GPP core network or to this function via the NEF (e.g. Nnef_Sensing_Subscribe service). The request may includes:

a.	the Target Sensing Area information. This information is derived from the Target spatial map(s) (e.g. Area of interest) and can be handled by 3GPP core network.

b.	If periodic or event triggered sensing result notification is supported by the 3GPP core network, the SEAL Sensing server may also utilize them based on the Notification intervals included in the List of events.

c.	If Detection or Tracking specific object is supported by the 3GPP core network, the SEAL Sensing server may also translates the information included in the specified objects IE in the List of events to a list of IDs that can be handled by  the 3GPP core network and includes them in this Sensing Service Request.

Editor’s note 2:	the Sensing Service Request may be used to trigger the sensing service provided by 3GPP core network, or used to request the sensing result. How the SEAL sensing server utilizes the sensing service will be based on SA2 progress and is FFS.

6.	The sensing-related function in the 3GPP core network sends the Sensing Service Response either directly to the SEAL Sensing server or via the NEF.The response indicates the result of Sensing Service subscription.

Editor’s note 3:	the detailed IEs included in the step 5 and step 6 are based on the output of SA2 and are FFS.

7.	The SEAL Sensing server sends a Get Sensing Results Subscription response to the SEAL SM server to indicate the result of Sensing Results Subscription.

8.	The SEAL SM server sends a Subscribe Spatial Map Event Response to the VAL server (or SEAL SM client) as per step 3 in clause 9.3.6.2.1 of 3GPP TS 23.437 [437].

The following step 9 to step 12 are Spatial map notification procedure.

9.	The SEAL Sensing server detects a sensing event that requires the use of sensing capabilities, e.g. a specific time, then the SEAL Sensing server interacts with 3GPP core network to get the sensing result. The interaction may be also based on the information included in the Get Sensing Results Subscription Request, e.g. if periodic notification is required, the 3GPP core network may notify the sensing result to the SEAL Sensing server periodically.

10.	The SEAL Sensing server sends a Sensing Event Notification to the SEAL SM server. This notification includes the information obtained from step 9.

11.	The SEAL SM server sends a Spatial Map Event Notification to the requester as per step 2 in clause 9.3.6.2.2 of 3GPP TS 23.437 [437].

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 22.137, "Service requirements for Integrated Sensing and Communication; Stage 1".

[4] 		3GPP TS 23.255, "Application layer support for Uncrewed Aerial System (UAS); Functional architecture and information flows;".

[5]	3GPP TS 23.286, "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows; "

[6]	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication (Release 19)".

[437]	3GPP TR 23.437: "Service Enabler Architecture Layer for Verticals (SEAL);Spatial map and Spatial anchors".