3GPP SA WG6 Meeting #69	S6-254528

# Oct. 13 – 17th, 2025; Wuhan, China. 					(revision of S6-254113)

Source: 	China Mobile

Title: 	KI#2 Solution: Exposing the value-added information of AIoT devices to the consumer

Document for: 	Approval

Agenda Item: 	9.8

Work Item / Spec.:	FS_AmbientIoT_Ph2_APP / TR 23.700-26 v0.1.0

Abstract of the contribution: This document proposes a solution to address the KI#2 (TR 23.700-026, i.e., expose the value-added information of AIoT devices to consumer.

## 1	Introduction

This pCR proposes a solution to address the KI#2, i.e., expose the value-added information of AIoT devices to consumer. The solution is based on both the enhanced architecture of the application enabler layer for the support of the AIoT App service as proposed in the pCR S6-254112 and the current SA2 Ambient IoT architecture as in 3GPP TS 23.369 [3]. Thanks to the exclusive support of the application enablement layer at server side for the Ambient IoT App service, any additional processing for value-add services will be handled by the application enabler AIoTApp that is newly defined in the pCR S6-254112. The VAL server communicates with the enabler AIoTApp, which then interacts with the 5GS NF AIoTF as the trusted AF (or via NEF as the untrusted AF) via the network function services that are exposed through their service-based interfaces for AIoT services.

## 2	Reason for Change

For AIoT service operation requests, the collected AIoT Devices ID(s) could be huge and may not fulfill the service requirements from the 3rd parties (e.g. AS/AF). This might result in the gaps between the service requirements and the current exposure capabilities.

For example, a 3rd-party retail App-server (i.e., VAL server) may issue periodic inventory service requests to check the volume of some specific types of AIoT devices in a certain area. When the number of the available AIoT devices of a type drops below a certain threshold, the retail store might run into service interruption of not having sufficient goods for customers. In this scenario, the corresponding retailer is better off replenishing the stock in advance by adding more devices of the type. The current exposure scheme from the 5G core network as specified in 3GPP TS 23.369[3] will only provide the AIoT devices ID without categorizing the AIoT devices in service granularity. The application enablement layer can be enhanced to address the deficiency.

In another scenario, if a VAL server (AF) sends an 'inventory service’ request covering a large area or send continuously a batch of 'inventory service’ requests covering different areas (with different area-IDs), though belonging to a same (large) targeted area whose identity could be a geographical civic address. Then, the AIoT application enabler may aggregate these requests and send only one combined 'inventory service’ request or send multiple requests to multiple AIOTF in 5GC to optimize the efficiency.

Besides, some application servers (e.g. a logistics company) may need to regularly track the location of the AIoT devices (e.g. the goods in the truck) and identify the trajectory or the direction for the AIoT devices depending on their locations, and then send warnings to the AIoT devices in case they deviate from the right trajectory/direction. How to leverage the 5GC exposure capability of mapping the RAN ID to the location information (e.g. civic address) and accordingly tracking the devices’ location continually as requested should be studied further in application enablement layer.

In a word, it would be of great advantage to AIoT services consumers, i.e., the VAL server, if they can receive enriched results from the Ambient IoT App enabler.

## 3	Proposal

It is proposed to agree and capture the solution for the inclusion in the TR 23.700-26 v0.1.0.

# * * * * First change * * * *

## 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| …… |  |  |  |  |
| Sol #X |  | X |  |  |

# * * * * next change * * * *

## 7.x	Solution #x: Exposing the value-added information of AIoT devices to the consumer

### 7.x.1	Solution Architecture

This solution is based on an independent architecture that is newly defined in the pCR S6-254524. In the architecture, an AIoTApp application enabler is defined and acts as a SEAL layer function entity. This function entity can be deployed and implemented by integrating with other related service enabler functions (e.g., SEALDD server, NSCE server) for the support of the Ambient IoT application services.

### 7.x.2	Solution description

For AIoT service operation requests, the collected AIoT Devices ID(s) could be huge, less categorized and may not fulfill the service requirements from the 3rd parties (e.g. AS/AF).

For example, a 3rd-party retail App-server (i.e., VAL server) may issue periodic inventory service requests to check the volume of some specific types of AIoT devices in a (large) area. When the number of the available AIoT devices of a type drops below a certain threshold, the retail store might run into service interruption of not having sufficient goods for customers. In this scenario, the corresponding retailer is better off replenishing the stock in advance by adding more devices of the type. The current exposure information from the 5G core network as specified in 3GPP TS 23.369[3] lacks the categorization for the AIoT devices.

In another scenario, if a VAL server (AF) sends an 'inventory service’ request covering a large area, or send continuously a batch of 'inventory service’ requests covering different (small) areas, though belonging to the same (large) targeted area whose identity could be a geographical civic address, then, the AIoT application enabler may aggregate these requests to optimize the efficiency by sending only one combined 'inventory service’ request, or sending multiple requests to multiple AIOTF in 5GC.

Besides, some VAL servers may need to regularly track the location of the AIoT devices (e.g. the goods in the truck) and identify the trajectory or the direction for the AIoT devices depending on their locations. Alert messages may be sent to certain AIoT devices in case they deviate from the right trajectory/direction. How to leverage the 5GC exposure capability of mapping the RAN ID to the location information (e.g. civic address) and accordingly tracking the devices’ location continually as requested should be studied further in application enablement layer.

In a word, it would be of great advantage to AIoT services consumers, i.e., the VAL server, if they can receive enriched results from the Ambient IoT App enabler.

Based on the functional model for AIoTApp service as in the Fig. 7.x.1-1, the application enablement architecture layer is proposed to enhance at the server side. The VAL server communicates with the AIoTApp enabler via the AIoTApp-S reference point, which further communicates with the underlying 3GPP network systems. For any additional processing of value-add services, e.g., more effective batched processing of service requests, aggregation of captured AIoT device information, exposure of location info. of AIoT devices, etc.,   the application enabler AIoTApp can provide the enriched services.

### 7.x.3	Procedure

### 7.x.3.1	Exposure of value-added information via AIoT application enabler

Figure 7.x.3.1-1 illustrates the procedure for the AIoT application enabler to interact with the 5GS for the support of the Ambient IoT App service that leverages the corresponding 5GC NF services and APIs as specified in the TS 23.369 [3]. Upon receiving the requested AIoT device information, the AIoTApp enabler conducts the value-added information processing before exposing the information to the VAL server (i.e., AIoT service consumer).

Pre-condition:

-	The VAL server discovers and selects the AIoTApp enabler by CAPIF functions.

-	Multiple AIOTFs may be discovered and selected in the 5GS.

Figure 7.x.3.1-1: Exposure of value-added info via AIoT App enabler

### The VAL server decides to use the AIoT application enabler (AIoTApp) to collect the AIoT device information and allocates address/port as AIoTApp-S Data transmission connection information for receiving the AIoT data packets from the AIoTApp enabler. The VAL server sends the AIoT_app_service request to the AIoTApp enabler. The type of AIoT service is Inventory request. The service request includes VAL server ID, VAL service ID, AIoTApp-S Data transmission connection information of the VAL server side, and the AIoT service related parameters (e.g., AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], etc.). The details of these service parameters are described in TS 23.369 [3].

As specified in the clause 5.8 of TS 23.369 [3], Information about the target AIoT Device(s) may include Filtering Information that might be constructed by one or multiple components (i.e. ID Type, PLMN Identifier, NID, third party identifier and Identification Information). These components can be used by the AIoTApp enabler to enrich the processing.

### 2.	Upon receiving the request, the AIoTApp enabler performs an authorization check. If authorization is successful, the AIoTApp enabler allocates the AIoTApp-S data transmission connection information (e.g., address/port) of AIoTApp enabler side to receive the AIoT data packets from the VAL server to be delivered to the 5GC. The AIoTApp enabler responds with the AIoT_app_service response.

### NOTE 1:	The AIoTApp-S data transmission connection information of the AIoTApp enabler side is optional, if the AIoTApp enabler uses the downlink pull mode to fetch the AIoT data from the address provided by the VAL server in step 1, and uses the uplink push mode to send the AIoT data to the address provided by VAL server.

### After the AIoTApp enabler receives from the VAL server the AIoT service related parameters, depending on the local configuration, the AIoTApp enabler may apply enriched and optimized processing, e.g.:

### If the received request covers a large area, the AIoTApp may send multiple requests to multiple discovered and selected AIOTF NFs in 5GC. This is applicable to the trusted AF case.

### If the AIoTApp receives batched requests continuously with each one spanning a small area, the AIoTApp may either aggregate the batched requests and send only one combined service request or send multiple requests concurrently to multiple AIOTF NFs in 5GC.

### The AIoTApp sends service operation requests towards the (selected) AIOTF(s), as in TS 23.369 clause 6.2. There could be multiple AIOTF NFs selected from the step-3.

### NOTE 2:	All the invoked service operations, together with the service parameters, conform to what have been specified in TS 23.369. E.g., depending on the trustiness of the AF (i.e., AIoTApp), AIoTApp sends either External Target Area information (to NEF as an untrusted AF) or Target Area information (to AIOTF as a trusted AF).

### The 5GS processes the AIoT service request(s) and collects the AIoT device information based on the provided service parameters, or local configuration. Details are in TS 23.369 [3].

### (Multiple) AIoTF(s) send collected AIoT device information to AIoTApp (or via NEF).

### 5.	The AIoTApp enabler apply value-added processing to the received AIoT device information, e.g.

### If AIoTApp receives AIoT device information for a large target area from multiple AIOTF NFs, then it can consolidate the information before send to the VAL server.

### Based on the components in the Filtering Information received in the step-1, e.g., ID Type, the AIoTApp enabler may consolidate and categorize AIoT devices and derive the number of the available AIoT devices of a type in a target area. The AIoTApp then exposes the value-added information to the VAL server for further processing.

### The AIoTApp enabler sends the processed AIoT device information via the AIoT_app_service notification to the VAL server.

### 7.x.3.2	Information flows

Editor’s Note: The details of Information Flows are FFS.

### 7.x.3.2.1	AIoT App Service request

Table 7.x.3.2.1-1 describes the information flow from the VAL server to the AIoTApp enabler for the support of the AIoT App service.

Table 7.x.3.2.1-1: AIoT App service request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| AIoTApp-S endpoint information | M | Address/port and/or URL of the VAL server to receive the AIoT data packets from the AIoTApp. |
| AIoT service parameters | M (Note 1) | AIoT service parameters (e.g., AF ID, (external) target area info, etc.). |
| DL AIoT data delivery status subscription indication | O | Indicates the VAL server expected to receive the DL AIoT delivery status notification |
| NOTE 1: The AIoT service parameters are specified in the 3GPP TS 23.369 [3]. |  |  |

### 7.x.3.2.2	AIoT App service response

Table 5.x.3.2.2-1 describes the information flow from the AIoTApp enabler to the VAL server to respond to the AIoT App service request.

Table 7.x.3.2.2-1: AIoT App service response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| AIoTApp-S enpoint information | O | Address/port and/or URL of the AIoTApp enabler to receive the AIoT data packets from the VAL server to be delivered to 5GC. |
| Cause | O (see NOTE) | Indicates the reason for the failure, e.g., AF ID not supported, Target Area not allowed. |
| NOTE: The IE is only present if the Result is failure. |  |  |

### 7.x.3.2.3	AIoT App service notification

Table 5.x.3.2.3-1 describes the information flow from the AIoTApp enabler to the VAL server to notify the events related to the AIoT App service.

Table 7.x.3.2.3-1: AIoT App service notification

| Information element | Status | Description |
| --- | --- | --- |
| Event ID | O (see NOTE) | Identifies event of the AIoTApp enabler interaction status with 5GC for AIoT App services, e.g., enriched processing results, no collected AIoT device info., etc. |
| VAL service ID | O | Identity of the VAL service. |
| DL AIoT data delivery instructions | O (see NOTE) | Indicates the instructions to the VAL server regarding the DL AIoT App service |
| > Collected AIoT device info. | O | Indicates the collected raw AIoT device information. |
| > Enrich-processed AIoT info. | O | Indicates enriched processing results of AIoT info (e.g., categorization, consolidation, filtering, etc.) |
| NOTE: Either Event ID IE or the DL AIoT data delivery instruction ID is present. |  |  |

### 7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# * * * * End of changes * * * *